Functional Specification: centroid_lut_select

Signal Name

centroid_lut_select

Module Context

Used in: rgb_kmeans_cluster_engine

Purpose

centroid_lut_select selects which centroid entry in the centroid lookup table (LUT) is being addressed for update or access.

It acts as the centroid index selector for the clustering engine.

Functional Role

The clustering engine stores multiple centroid RGB values, one for each cluster. centroid_lut_select identifies one centroid slot among those stored entries.

Typical use:

- choose which centroid to write

- choose which centroid to configure

- choose which centroid is currently targeted by control logic

Direction

Input

centroid_lut_select : in natural

In the portable version it is more safely constrained as:

centroid_lut_select : in natural range 0 to cluster_count-1

Functional Behavior

1. Centroid Write Selection

When new centroid RGB data is presented on centroid_lut_in, centroid_lut_select identifies the destination LUT entry.

Example:

- centroid_lut_select = 0 → update centroid 0

- centroid_lut_select = 1 → update centroid 1

- centroid_lut_select = 4 → update centroid 4

2. Configuration Indexing

During centroid initialization or runtime centroid updates, this signal allows external control logic to configure one centroid at a time.

This is useful for:

- loading initial cluster centers

- adaptive centroid update

- software-controlled centroid programming

3. Controlled Interaction with k_ind_w

In your current portable architecture, centroid write occurs only when:

if k_ind_w = centroid_lut_select then

So centroid_lut_select works together with k_ind_w as a write-address qualification mechanism.

This means:

- centroid_lut_select chooses the target centroid

- k_ind_w confirms the active write index

Related Signals

centroid_lut_in

Carries the RGB value to be written into the selected centroid entry.

Typical format:

centroid_lut_in(23 downto 16) -> Red

centroid_lut_in(15 downto 8)  -> Green

centroid_lut_in(7 downto 0)   -> Blue

k_ind_w

Write-side centroid index control.

k_ind_r

Read-side centroid index control.

centroid_lut_out

Returns the centroid value from the selected read path, typically using k_ind_r.

Valid Range

If cluster count is N, valid values are:

0 to N-1

For example, with 5 clusters:

0 to 4

Any value outside this range should be prevented by type constraint or control logic.

Timing Behavior

Clock Domain

centroid_lut_select is sampled on the rising edge of clk when write logic is active.

Reset Behavior

During rst_n = '0', centroid memory is cleared, and centroid_lut_select has no functional effect until normal operation resumes.

Operational Use Cases

Use Case 1: Initial Centroid Loading

A controller writes centroid values one by one:

- set centroid_lut_select = 0, load centroid 0

- set centroid_lut_select = 1, load centroid 1

- continue until all centroids are loaded

Use Case 2: Runtime Centroid Update

A training or adaptation block updates a specific centroid based on new image statistics.

Use Case 3: Cluster Reconfiguration

System software changes centroid definitions for a new color segmentation profile.

Design Intent

centroid_lut_select provides a simple and deterministic way to access a specific centroid in a multi-centroid memory structure without broadcasting updates to all entries.

This improves:

- configurability

- modularity

- scalability for larger K

Recommended Stronger Definition

Instead of:

centroid_lut_select : in natural;

use:

centroid_lut_select : in natural range 0 to cluster_count-1;

This gives:

- safer synthesis behavior

- clearer interface intent

- prevention of invalid centroid index values

Example Functional Description for Document

You can place this directly in your design document:

centroid_lut_select is the centroid address selection input of the K-means clustering engine. It identifies which centroid LUT entry is targeted during centroid configuration or update operations. The signal is used in conjunction with centroid write data (centroid_lut_in) and write index control (k_ind_w) to load RGB centroid values into the selected cluster slot. Its valid range is from 0 to cluster_count-1.

Short Engineering Definition

centroid_lut_select = centroid LUT address select

Recommended Rename

For even better clarity, you may rename it to:

- centroid_index_sel

- centroid_addr

- centroid_write_select

Best choice:

centroid_index_sel

because it clearly conveys index-based centroid selection.