Interface (parallel, 8-bit each)

- Inputs

  - clk, reset

  - H : std_logic_vector(7 downto 0) -- 0..255 (maps to 0..360°)

  - S : std_logic_vector(7 downto 0) -- 0..255

  - V : std_logic_vector(7 downto 0) -- 0..255

  - in_valid (optional sidebands: sof, eol, eof)

- Outputs

  - R, G, B : std_logic_vector(7 downto 0) -- 0..255

  - out_valid (plus sidebands, latency-aligned)

Fixed-point math (integer, 8-bit domain)

Let h = unsigned(H), s = unsigned(S), v = unsigned(V).

- m = (v * (255 - s)) >> 8 -- floor term

- C = v - m -- chroma

- h_sector = h / 43 -- 0..5 (≈ 256/6)

- h_phase = h - (h_sector * 43) -- 0..42

- X = (C * h_phase) / 43 -- interpolation term

  - timing-friendly form: X ≈ (C * 43 + 2^10) >> 11 after scaling (or a small divider)

Sector mapping (combinational MUX)

case h_sector is

when 0 => (r',g',b') := (C, X, 0);

when 1 => (r',g',b') := (X, C, 0);

when 2 => (r',g',b') := (0, C, X);

when 3 => (r',g',b') := (0, X, C);

when 4 => (r',g',b') := (X, 0, C);

when others => (r',g',b') := (C, 0, X); -- 5

end case;

R = sat(r' + m);  G = sat(g' + m);  B = sat(b' + m);  -- clamp to 0..255

Pipeline & timing

- Throughput: 1 pixel/clk.

- Latency: 4–7 cycles (unpack → terms → sector MUX → add/clip → pack).

- Delay valid/sof/eol/eof by the same number of cycles.

Edge cases

- S=0 ⇒ grayscale: R=G=B=V.

- Hue wrap handled by sector logic (0↔255).

- Clip all outputs to 0..255.

Minimal entity (parallel HSV → RGB)

entity hsv2rgb_parallel is

port (

clk, reset  : in  std_logic;

in_valid    : in  std_logic;

H, S, V     : in  std_logic_vector(7 downto 0);

out_valid   : out std_logic;

R, G, B     : out std_logic_vector(7 downto 0)

);

end;

-- architecture: implement m, C, h_sector/phase, X, sector MUX, add+clip, and pipeline regs.

If you want, I can turn this into synthesizable VHDL with a divider-free X (reciprocal multiply + shift) and a small testbench.