The document provides a detailed technical overview of the iHSL to RGB VHDL module, which converts 24-bit HSV or HSL pixel streams into 24-bit RGB pixel streams while preserving video synchronization sidebands. The module is designed for video applications, maintaining data integrity and synchronization throughout the conversion pipeline at a throughput of one pixel per clock cycle and typical latency of 4 to 7 cycles   .

## Module Overview and Interface

The module accepts input pixel data in either HSV or HSL format, distinguished by the interpretation of the lowest byte (Value for HSV or Lightness for HSL). Inputs include clock and reset signals, the iHSL data channel carrying pixel data with associated sidebands such as valid, start-of-frame (sof), end-of-frame (eof), and end-of-line (eol) signals. Outputs are provided on the oRGB channel, delivering RGB pixel data with synchronized sidebands, ensuring all timing and control signals are delay-matched to maintain synchronization   .

## Input Format Selection

The module automatically detects whether the input is HSV or HSL based on the interpretation of the lowest byte and selects the corresponding processing path. This enables flexible handling of either format without manual intervention  .

## Datapath and Processing Pipeline

The conversion process is fully pipelined and integer fixed-point based, consisting of four main stages:

### Stage 0 — Unpack and Normalize

Input pixel data is unpacked into Hue (H), Saturation (S), and a third component X, representing Value (V) in HSV or Lightness (L) in HSL. The hue value is divided into six sectors by computing Hsector = H / 43 and the intra-sector phase Hphase = H % 43, mapping the 0–255 hue range into six equal segments for color wheel positioning  .

### Stage 1 — Core Terms Calculation

For HSV, the module calculates:

- Minimum value ( m = (V \times (255 - S)) >> 8 )

- Chroma ( C = V - m )

- Intermediate value ( X{int} = (C \times H{phase}) / 43 )

For HSL, the calculations are:

- ( L = X )

- ( C = ((255 - |2L - 255|) \times S) >> 8 )

- ( X{int} = (C \times H{phase}) / 43 )

- ( m = L - (C >> 1) )

These computations use integer fixed-point arithmetic optimized for hardware efficiency   .

### Stage 2 — Sector Multiplexer

Based on the hue sector ( H_{sector} ) (0 to 5), the module assigns the chroma (C), intermediate value (Xint), and zero to preliminary RGB components ( r', g', b' ) according to the following mapping:

| Sector | r' | g' | b' |
| --- | --- | --- | --- |
| 0 | C | Xint | 0 |
| 1 | Xint | C | 0 |
| 2 | 0 | C | Xint |
| 3 | 0 | Xint | C |
| 4 | Xint | 0 | C |
| 5 | C | 0 | Xint |

This mapping ensures accurate color representation corresponding to the hue sector on the color wheel   .

### Stage 3 — Offset, Clip, and Pack

The preliminary RGB values are offset by adding ( m ), clipped to the 0–255 range to prevent overflow or underflow, and packed into the 24-bit output data channel:

- ( R = \text{sat}(r' + m) )

- ( G = \text{sat}(g' + m) )

- ( B = \text{sat}(b' + m) )

All video control signals are forwarded with matching latency to maintain synchronization   .

## Control, Timing, and Edge Cases

The converter processes pixels at one per clock cycle with a latency of 4 to 7 cycles. Reset clears the pipeline and deasserts outputs until refilled. Special cases handled include:

- Grayscale output when saturation is zero, where RGB components equal Value (HSV) or Lightness (HSL).

- Hue wraparound between 0 and 255 to maintain continuous color transitions.

- Saturation of output values to the 0–255 range to avoid invalid values   .

## Implementation Details

The design typically uses 3 to 5 DSP-mapped multipliers along with adders, subtractors, and LUT multiplexers, with no block RAM required. Divisions by constants are optimized through reciprocal multiplication and bit shifts to maximize frequency. Pipeline stages are registered and retimed for performance  .

## Summary of Conversion Approach

The module employs a fully pipelined integer fixed-point architecture tailored for video streams, ensuring sideband signals are delay-aligned with pixel data. The conversion involves unpacking, normalization, calculation of chroma and intermediate terms, sector-based multiplexing, offset addition, clipping, and final RGB assembly, all at a high throughput suitable for real-time video applications .

## HSV to RGB Datapath Details

The HSV input consists of 8-bit Hue (mapped to 0–360°), Saturation, and Value components. The hue is normalized into sectors using modulus 60 for interpolation. The minimum value ( m ) is calculated as ( (V \times (255 - S)) >> 8 ), chroma ( C = V - m ), and an interpolation term ( RGB_ADJUST = (C \times (HUE \bmod 60)) / 60 ) is computed. These values are combined via a sector multiplexer to produce the final RGB output, exemplified by the red sector where ( R = V ), ( G = m + RGB_ADJUST ), and ( B = m )   .