This diagram you shared is the datapath for HSV→RGB adjustment logic, showing how Hue, Saturation, and Value are processed into intermediate terms before assembling the final RGB values. Here’s a clear description you can use alongside your VHDL module documentation:

HSV→RGB Datapath (Figure Explanation)

Inputs

- HUE (8-bit, 0–255 mapped to 0–360°)

- SATURATE[7:0] (8-bit saturation)

- VALUE[7:0] (8-bit brightness, V)

Processing Flow

- Hue Normalization

  - Hue is divided into sectors using mod 60.

  - Produces HUE MOD 60, which determines intra-sector interpolation.

- Chroma & Minimum Value

  - Compute 255 − Saturation.

  - Multiply with VALUE → (VALUE × (255 − Saturation)) >> 8.

  - Result gives MIN_VALUE, representing the floor term m = V(1 − S).

- Chroma (Delta)

  - Subtract: MAX_VALUE − MIN_VALUE.

  - This yields the chroma C, the color intensity component.

- Interpolation (X term)

  - Multiply chroma by (HUE MOD 60).

  - Divide by 60 to scale → RGB_ADJUST.

  - This is the interpolation offset used for non-primary channels.

- Outputs

  - From V, m, C, and RGB_ADJUST, the sector multiplexer (not shown here) assembles the actual (R, G, B) triplet:

    - Example: for Hue sector 0 (red region):
R = V, G = m + RGB_ADJUST, B = m.

Key Equation in Diagram

RGB_ADJUST = ((VALUE − (VALUE × (255 − Saturation) >> 8)) × (HUE MOD 60)) / 60

This is the integer-friendly fixed-point expression for the interpolation term.