This document presents a detailed technical overview of the iHSL to RGB VHDL module, which is designed to convert 24-bit HSV or HSL pixel streams into 24-bit RGB pixel streams. The module maintains video synchronization sidebands and is optimized for real-time video applications, achieving a throughput of one pixel per clock cycle with a latency between 4 and 7 cycles.

## Module Overview and Interface

The module accepts input in either HSV (Hue, Saturation, Value) or HSL (Hue, Saturation, Lightness) format. It automatically detects the input format based on the lowest byte, which represents either Value or Lightness. The inputs include clock and reset signals along with the iHSL data channel that carries sidebands such as valid, start of frame (sof), end of frame (eof), and end of line (eol). The output is an oRGB data channel with synchronized sidebands and 8-bit R, G, and B channels .

## Processing Pipeline

The conversion process is implemented through a fully pipelined architecture divided into four main stages:

### Stage 0 — Unpack & Normalize

- Extracts the H, S, and V/L components.

- Computes the hue sector as the integer division of H by 43.

- Computes the phase within the hue sector as H modulo 43 .

### Stage 1 — Core Term Calculations

- For HSV input:

- Calculates m as (V × (255 − S)) shifted right by 8 bits.

- Computes C as V minus m.

- Determines Xint as (C × H_phase) divided by 43.

- For HSL input:

- Assigns L from the input.

- Calculates C as ((255 − |2L − 255|) × S) shifted right by 8 bits.

- Computes Xint as (C × H_phase) divided by 43.

- Calculates m as L minus half of C .

### Stage 2 — Sector Multiplexer

Assigns temporary color values (r′, g′, b′) based on the hue sector according to the following table:

| Sector | r′ | g′ | b′ |
| --- | --- | --- | --- |
| 0 | C | Xint | 0 |
| 1 | Xint | C | 0 |
| 2 | 0 | C | Xint |
| 3 | 0 | Xint | C |
| 4 | Xint | 0 | C |
| 5 | C | 0 | Xint |

### Stage 3 — Offset, Clip & Pack

- Final RGB outputs are computed as R = saturate(r′ + m), G = saturate(g′ + m), and B = saturate(b′ + m).

- Values are clipped to the 0–255 range before packing into the output stream .

## Control, Timing & Edge Cases

- The module sustains a throughput of one pixel per clock cycle with a latency of 4 to 7 pipeline stages.

- The reset signal clears the pipeline and deasserts outputs until the pipeline is refilled.

- Special cases handled include:

- Grayscale output when saturation S is zero, resulting in R = G = B = V for HSV or R = G = B = L for HSL.

- Hue wraparound at 0 and 255 to ensure smooth color transitions.

- All output values are clipped to the 0–255 range to maintain valid color values .

## Implementation Details

- The design requires approximately 3 to 5 DSP multipliers along with adders and subtractors, and does not use BRAM.

- Divisions are replaced by reciprocal multiplication and bit-shifts to achieve a high maximum frequency (Fmax).

- The pipeline stages are fully registered with retiming enabled to maximize performance .

## Summary of Conversion Approach

The iHSL to RGB VHDL module provides a robust and hardware-efficient solution for color space conversion. Through pipelined processing, fixed-point arithmetic, and optimized datapath design, it delivers real-time performance suitable for video applications while preserving sideband synchronization and data integrity .