# iHSL to RGB VHDL Module

The iHSL to RGB VHDL module is designed to convert incoming HSV or HSL pixel streams, each 24 bits per pixel, into RGB streams of the same bit width, while maintaining video-style channel sidebands for synchronization and data integrity.

iHSL→RGB VHDL Module — Data Flow

Function

Converts 24-bit HSV/HSL pixels to 24-bit RGB with video-style sidebands preserved and latency-aligned.

Interface

Inputs

- clk, reset

- iHSL channel

  - tdata[23:16] = H (0–255 → mapped to 0–360°)

  - tdata[15:8] = S (0–255)

  - tdata[7:0] = V (HSV) or L (HSL), 0–255

  - sidebands: valid, sof, eof, eol (strobes/ready-valid style)

Outputs

- oRGB channel

  - tdata[23:16] = R, tdata[15:8] = G, tdata[7:0] = B (all 0–255)

  - sidebands forwarded and delay-matched

If upstream is HSV, the low byte is V; if HSL, it is L and the HSL path is taken.

Datapath (per pixel)

Stage 0 — Unpack & Normalize (1 clk)

- Extract H, S, X where X=V (HSV) or X=L (HSL).

- Compute hue sector and phase:
H_sector = H / 43 (≈ 256/6), H_phase = H % 43 (0..42).

Stage 1 — Core Terms (1–2 clk, integer fixed-point)

HSV

- m = (V * (255 − S)) >> 8

- C = V − m

- Xint = (C * H_phase) / 43 (use reciprocal multiply + shift for timing)

HSL

- L = X

- C = ((255 − |2*L − 255|) * S) >> 8

- Xint = (C * H_phase) / 43

- m = L − (C >> 1)

Stage 2 — Sector MUX (1 clk)

Map (C, Xint, m) to (r', g', b') by H_sector ∈ {0..5}:

| Sector | r' | g' | b' |
| --- | --- | --- | --- |
| 0 | C | Xint | 0 |
| 1 | Xint | C | 0 |
| 2 | 0 | C | Xint |
| 3 | 0 | Xint | C |
| 4 | Xint | 0 | C |
| 5 | C | 0 | Xint |

Stage 3 — Offset, Clip, Pack (1 clk)

- R = sat(r' + m), G = sat(g' + m), B = sat(b' + m) (clip to 0..255)

- Pack to oRGB.tdata[23:0]; forward valid/sof/eof/eol with identical latency.

Control & Timing

- Throughput: 1 pixel/clock.

- Latency: typically 4–7 cycles (sum of pipeline stages).

- Reset: clears pipeline; outputs deasserted until filled.

Edge Cases

- S = 0 → grayscale: R=G=B=(V for HSV) or (L for HSL).

- Hue wrap: H=255 and H=0 fall in adjacent sectors; logic wraps correctly.

- All outputs saturate to 0..255.

Implementation Notes (8-bit typical)

- Resources: ~3–5 mults (DSP-mapped), add/sub, LUT MUX; no BRAM.

- Fmax tips: replace /43 or /60 with reciprocal multiply + shift (e.g., *6 >> 8, *43 >> 11 with rounding); register all stage boundaries; enable retiming.

A fully pipelined, integer fixed-point converter that accepts 24-bit HSL/HSV with video sidebands and emits 24-bit RGB at one pixel per clock. The design computes chroma/offset/intermediate terms, selects by hue sector, adds the offset, clips to 8 bits, and delay-aligns sidebands to the data.

datapath for HSV→RGB adjustment logic, showing how Hue, Saturation, and Value are processed into intermediate terms before assembling the final RGB values. Here’s a clear description you can use alongside your VHDL module documentation:

HSV→RGB Datapath (Figure Explanation)

Inputs

- HUE (8-bit, 0–255 mapped to 0–360°)

- SATURATE[7:0] (8-bit saturation)

- VALUE[7:0] (8-bit brightness, V)

Processing Flow

- Hue Normalization

  - Hue is divided into sectors using mod 60.

  - Produces HUE MOD 60, which determines intra-sector interpolation.

- Chroma & Minimum Value

  - Compute 255 − Saturation.

  - Multiply with VALUE → (VALUE × (255 − Saturation)) >> 8.

  - Result gives MIN_VALUE, representing the floor term m = V(1 − S).

- Chroma (Delta)

  - Subtract: MAX_VALUE − MIN_VALUE.

  - This yields the chroma C, the color intensity component.

- Interpolation (X term)

  - Multiply chroma by (HUE MOD 60).

  - Divide by 60 to scale → RGB_ADJUST.

  - This is the interpolation offset used for non-primary channels.

- Outputs

  - From V, m, C, and RGB_ADJUST, the sector multiplexer (not shown here) assembles the actual (R, G, B) triplet:

    - Example: for Hue sector 0 (red region):
R = V, G = m + RGB_ADJUST, B = m.

Key Equation in Diagram

RGB_ADJUST = ((VALUE − (VALUE × (255 − Saturation) >> 8)) × (HUE MOD 60)) / 60

This is the integer-friendly fixed-point expression for the interpolation term.