# Technical Overview: iHSL to RGB VHDL Module

# Executive Summary

This section presents a comprehensive technical overview of the iHSL to RGB VHDL module. The module is engineered to convert 24-bit HSV (Hue, Saturation, Value) or HSL (Hue, Saturation, Lightness) pixel streams into 24-bit RGB (Red, Green, Blue) pixel streams. Throughout the conversion process, the module preserves video synchronization sidebands, ensuring seamless integration with real-time video applications. The architecture features a fully pipelined, integer fixed-point design, supporting a high-throughput operation of one pixel per clock cycle and maintaining a typical latency ranging from four to seven cycles.

# Design and Operation

The iHSL to RGB VHDL module is specifically optimized for real-time video processing. It accepts pixel stream inputs in either HSV or HSL format, automatically detecting the format based on the lowest byte provided. The interface consists of synchronized input and output data channels that include sidebands for conveying video timing signals, such as valid, start-of-frame (sof), end-of-frame (eof), and end-of-line (eol). The module guarantees efficient video throughput, processing one pixel per clock cycle, while maintaining a latency of four to seven cycles.

## Module Architecture and Interface

- Input formats: HSV (Hue, Saturation, Value) and HSL (Hue, Saturation, Lightness)

- Automatic detection: The format is determined by the lowest byte, with 'V' indicating HSV and 'L' indicating HSL.

- Inputs:

- clk (clock), reset (reset signal)

- iHSL data channel with sidebands: valid, sof, eof, eol

Outputs:

oRGB data channel with synchronized sidebands

R, G, B channels, each 8 bits wide

## Processing Pipeline

- Stage 0 — Unpack & Normalize

- Extract H, S, and V/L from the input stream.

- Calculate the hue sector as H_sector = H / 43.

- Determine the phase with H_phase = H % 43.

- Stage 1 — Core Term Calculations

- For HSV:

- m = (V × (255 − S)) >> 8

- C = V − m

- Xint = (C × H_phase) / 43

- For HSL:

- L = X

- C = ((255 − |2L − 255|) × S) >> 8

- Xint = (C × H_phase) / 43

- m = L − (C >> 1)

- Stage 2 — Sector Multiplexer

- Assign intermediate RGB values (r′, g′, b′) based on hue sector:

- Based on the hue sector ( H_{sector} ) (0 to 5), the module assigns the chroma (C), intermediate value (Xint), and zero to preliminary RGB components ( r', g', b' ) according to the following mapping:

| Sector | r' | g' | b' |
| --- | --- | --- | --- |
| 0 | C | Xint | 0 |
| 1 | Xint | C | 0 |
| 2 | 0 | C | Xint |
| 3 | 0 | Xint | C |
| 4 | Xint | 0 | C |
| 5 | C | 0 | Xint |

- Stage 3 — Offset, Clip & Pack

- Calculate final output values:

- R = sat(r′ + m)

- G = sat(g′ + m)

- B = sat(b′ + m)

- All values are clipped to the 0–255 range before being packed into the output stream.

## Control, Timing, and Edge Cases

- Throughput: The module processes one pixel per clock cycle.

- Latency: The pipeline introduces a latency of four to seven stages.

- Reset behavior: On reset, the pipeline is cleared and outputs are deasserted until refilled.

- Special cases:

- Grayscale output is produced when S = 0, resulting in R = G = B = V (for HSV) or R = G = B = L (for HSL).

- Hue wraparound at 0 and 255 is managed to ensure smooth color transitions.

- All outputs are clipped within the 0–255 range to maintain valid RGB values.

## Implementation Details

- Utilizes approximately three to five DSP multipliers in addition to adders and subtractors; no BRAM is required.

- Division operations are replaced by reciprocal multiplication and bit-shifting to achieve high maximum clock frequency (Fmax).

- Pipeline stages are fully registered, with retiming enabled for optimal performance.

## Summary of Conversion Approach

The iHSL to RGB VHDL module offers a robust and hardware-efficient solution for color space conversion. By integrating pipelined processing, fixed-point arithmetic, and a carefully optimized datapath, it delivers real-time performance tailored for video applications. The design ensures preservation of sideband synchronization and maintains data integrity throughout the conversion process.