FPGA Implementation of Image K-Means Clustering Algorithm

The implemented image segment uses K-Mean clustering algorithm for FPGA Devices, and it has been designed with a standard Xilinx AXI4 stream

Things used in this project

| Hardware components |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | AMD Kria™ KV260 Vision AI Starter Kit. | × | 1 |  |  |
|  | Seeed Studio IMX219-130 8MP Camera. | × | 1 |  |  |
|  | Adafruit 16MP IMX519. | × | 1 |  |  |
|  | Adafruit 12.5MP IMX477. | × | 1 |  |  |
|  | Raspberry Pi Camera Module. | × | 1 |  |  |
| Software apps and online services |  |  |  |  |  |
|  | Baremetal. |  |  |  |  |
|  | AMD Vivado Design Suite. |  |  |  |  |
|  | AMD Vitis Unified Software Platform. |  |  |  |  |

Story

The implemented image segment in video color processing module uses K-Mean clustering algorithm for FPGA Devices, and it has been designed with a standard Xilinx AXI4 streaming interface, so that it can be inserted as module within any image processing pipeline.

This design takes 10-bit RGB format video stream and apply various filters and color space conversions. At high level, the design comprises of three pipelines: Capture, Process and Output pipelines. During process pipeline each rgb pixel either filtered or converted into color space which is implemented video color processing (VCP) module.

VCP module contains a collection of control registers and maintain a local small buffers to store video frame lines. A VCP module takes input video pixel stream, performs computation, or adds new contents to the stream through filters, and then outputs the processed pixel stream. VCP can be either a pixel transformation or a pixel generation.

VCP have following filters and color space conversion which can enabled before build:

Filters

- SHARP

- BLUR

- EMBOSS

- SOBEL EDGE DETECTION

- CONTRAST

Color Space Conversions

- RGB TO HSL

- HSL TO RGB

- RGB TO YCBCR

- YCBCR TO RGB

- RGB TO CMYK

- RGB TO YDBDR

- RGB TO CIEXYZ

- RGB TO CIEYUV

- RGB TO YIQ

- RGB TO YPBPR

- RGB TO LMS

- RGB TO ICTCP

- RGB TO HED

- RGB TO YC1C2

- RGB TO YCBCR

Color Correction Matrix

K-Mean Color Clustering

Test-Pattern

The Sony IMX477 12.5MP sensor is a 1/2.3-inch CMOS digital image sensor with an active imaging pixel array of 4056H x 3040V 60fps@max resolution. This sensor is configured by I2C interface and has MIPI output interface that is connected to the MIPI CSI-2 RX subsystem inside.

The AR 1335 13.0MP sensor is a 1/3.2-inch CMOS digital image sensor with an active imaging pixel array of 4208H x 3120V 30fps@max resolution. This sensor is configured by I2C interface and has MIPI output interface that is connected to the MIPI CSI-2 RX subsystem inside.

The MIPI CSI-2 receiver subsystem includes a MIPI D-PHY core and that connects 2 data lanes for imx477 camera device and 4 lanes for AR1335 camera device. The subsystem captures video stream frames from the AR1335 camera and IMX477 camera in RAW10 format.

The Demosaic module converts bayer pattern input frame to RGB color frame.

The VCP custom module apply various filters, color space conversion and K-Mean color clusters. It also applies image enhancement control to improve image quality which include contrast, brightness, saturation, white/black balance and rgb gain through axi4 lite configuration registers.

VDMA (Video Direct Memory Access) transfer video streaming to and from external memory and operate under software control.

Image frame resolution is set to 2048x1080 at 60 frames per second and maximum full resolution of 4056x3040 is also supported but limited to 15 frames per second.

K-MEAN COLOR CLUSTER

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs data (iRgb.red, iRgb.green, iRgb.blue) and control signals (iRgb.valid, iRgb.eol, iRgb.eof, iRgb.sof). The result of this module steam K-Mean rgb cluster color space in output oRgb channel.

This module is synthesized and implemented using Vivado 2022.1 for KRIA KV260 board and verified using ModelSim 2020 edition simulator.

The Functional block diagram of the implemented rgb data path to n clustered conversion is shown in Figure below.

In this module, K mean-based color quantization algorithm is applied to rgb input stream pixels.

- Select rgb k color references for k points to decide number of clusters.

- Find the distance of each input pixels with selected rgb k points.

- Assign each new pixel value to closest selected rgb n values using Euclidean distance.

- Pixels which are nearest to the selected rgb values allocated to a cluster.

Euclidean distance is calculated between original image pixel Red1, Green1 and Blue1 and reference pixel color schemes.

Rgb Image of video frame consist of 0 to 255 values per rgb channel which gives 256*256*256 colors, and the goal is to use color k mean cluster to set number of clusters. Minimum distance is the final candidate for being closest to the source rgb color. If an RGB image color depth of 24 bits which is 16 millions of colors, after K-mean clustering with value n, then image is converted to a version of n colors.

The codebook created for K-Means is called the color palette or reference color scheme.

K-mean cluster module 24bit pixels convert 16 millions of rgb colors into n color version. The module has clock and reset ports. Port iRGB and oRGB consist of red, green, and blue rgb channels with valid signal.

K EQUALTO 6 REFERENCE COLOR SCHEMES

Generated image below is the result of k-mean clustering using 6 colors references of palette schemes.

K EQUALTO 51 REFERENCE COLOR SCHEMES

In this reference, k parameter set to k = 51, where k is the number of clusters. K-mean color quantization quantizes input image to number of colors into 51 clusters from 51 refences of color schemes.

IMX477 CAMERA: K EQUALTO 90 REFERENCE COLOR SCHEMES

In this reference, k parameter set to k = 90, where k is the number of clusters. K-mean color quantization quantizes input image to number of colors into 90 clusters from 90 refences of color schemes.

90 REFERENCE PROGRAMABLE COLOR SCHEMES

VIDEO CONTROLLER INTERFACE

SONY 12.5MP IMX477 SENSOR

IMX477-MIPI-CS is a high-resolution digital camera. It incorporates a Sony 1/2.3" CMOS digital image sensor with an active imaging pixel array of 4056H x3040V.

SPECIFICATIONS

DRIVE MODE

FEATURES

- Back-illuminated and stacked CMOS image sensor Exmor RS.

- Digital Overlap High Dynamic Range (DOL-HDR) mode with raw data output.

- High signal to noise ratio (SNR).

- Full resolution @60 frame/s (Normal), 4K2K @60 frame/s (Normal), 1080p @240 frame/s.

- Full resolution @40 frame/s (12 bit Normal), Full resolution @30 frame/s (DOL-HDR, 2 frame).

- Output video format of RAW12/10/8, COMP8.

- Power Save Mode.

- Pixel binning readout and V sub-sampling function.

- Independent flipping and mirroring.

- Input clock frequency 6 to 27 MHz

- CSI-2 serial data output (MIPI 2lane/4lane)

- Two PLLs for independent clock generation for pixel control and data output interface.

- Ambient Light Sensor (ALS)

- Dual sensor synchronization operation (Multi camera compatible)

- 7 K bit of OTP ROM for users

- Built-in temperature sensor

- 10-bit/12-bit A/D conversion on chip

INTERFACE

IMX477 Camera module and the KRIA KV260 development board are connected through the FPC flexible flat cable. Camera pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to development board via 15-pin flat flexible cable.

The interface is defined as follows:

The Camera Serial Interface (CSI), a division from the MIPI Alliance which have formulated set of interface standards to standardize the interface of mobile devices such as camera and display. MIPI's full name is "Mobile Industry Processor Interface", MIPI DSI correspond to video display and MIPI CSI correspond to video input standards respectively. CSI interface is divided into physical layer (D-PHY) and protocol layer (CSI-2).

CONFIGURATION

For this design application, the IMX477 camera device is configured to output 1920x1080p video frames with RGB 30 bits pixel format, at 60 frames rate. Both the camera device and MIPI CSI-2 subsystem module is configured in advance.

The MIPI CSI-2 subsystem module is used to captures images from IMX477 camera sensor and outputs to axi4 stream video stream. Demosaic module is used to convert raw axi4 stream video data to rgb format video stream.

ETHERNET UDP VIDEO STREAMING

To successfully run the real-time network udp video streaming, this reference design uses the KRIA KV260 development board developed by Xilinx.

The design is composed of two platforms: the transmitter platform and the receiver platform. The receiver platform which receives input video stream from imx477 camera implements PL logic design of device. The transmitter platform which is PS Side of device implements a UDP/IP hardware protocol stack that enables high speed communication over a LAN or a point-to-point connection.

UDP/IP video streaming is implemented by using LWIP (LightWeight Internet Protocol) which is open-source networking stacked design for embedded system. LWIP supports both TCP and UDP at transport layer and application-level protocols.

KRIA KV260 board connect directly to the host PC and not running a DHCP server and host PC need to configure to an IP address of 192.168.0.42 and subnet mask of 255.255.255.0. Default IP address for FPGA development board is configured to “192.168.0.10”.

At startup LWIP fetch an IP address from DHCP server. If there no DHCP server than DHCP server request will timeout and the board will default to preset default to an IP address of 192.168.0.10.

The FFMPEG application on the host PC will connect with the video transmitter application on board and video data transmission will commence. When the video images are received, the FFMPEG invoke ffplay.exe which decode first 54 bytes source video type. Below figure shows 54 bytes of video header format.

The BSP required to run the lwip application and it includes many settings that impact ethernet performance. Below figure shows lwip BSP settings.

FPGA Firmware

- MIPI Camera interfacing

- Modular and scalable

- IP-Cores for Vivado Design Suite

- AXI4 / AXI-Stream compliancy

- Support for Xilinx 7Series, Ultrascale, Ultrascale+, SoC and MPSoC

Software

- Controlling and application

- Modular and scalable

- Written in C

PL Design include following IPs

PS Design include following Modules

Configuration and Intialization of VTC, VDMA, Camera Sensors And Lwip.

On PC run ffplay to watch realtime UDP streaming

- ffplay.exe -i -vf vflip -framerate 60 "udp://127.0.0.0:8080" -loglevel quiet

# FPGA Implementation of Image K-Means Clustering Algorithm

# Introduction

The FPGA implementation of the image segmentation module utilizes the K-Means clustering algorithm, specifically designed for FPGA devices. This solution incorporates a standard Xilinx AXI4 stream interface, allowing seamless integration as a module within any image processing pipeline.

## Project Components

### Hardware Components

| Component | Quantity |
| --- | --- |
| <a href="https://www.hackster.io/AMD/products/amd-kria-kv260-vision-ai-starter-kit?ref=project-8155cd">AMD Kria™ KV260 Vision AI Starter Kit</a> | 1 |
| <a href="https://www.hackster.io/seeed/products/imx219-130-8mp-camera?ref=project-8155cd">Seeed Studio IMX219-130 8MP Camera</a> | 1 |
| Adafruit 16MP IMX519 | 1 |
| Adafruit 12.5MP IMX477 | 1 |
| <a href="https://www.hackster.io/raspberry-pi/products/camera-module?ref=project-8155cd">Raspberry Pi Camera Module</a> | 1 |

### Software Applications and Online Services

Application

Baremetal

<a href="https://www.hackster.io/AMD/products/vivado-design-suite?ref=project-8155cd">AMD Vivado Design Suite</a>

<a href="https://www.hackster.io/AMD/products/vitis-unified-software-platform?ref=project-8155cd">AMD Vitis Unified Software Platform</a>

# System Architecture and Functionality

## Overview

The implemented image segmentation module, designed for video color processing, uses the K-Means clustering algorithm on FPGA devices. The design features a standard Xilinx AXI4 streaming interface for easy integration into image processing workflows.

The system processes video streams in a 10-bit RGB format and applies various filters and color space conversions. The architecture consists of three main pipelines: Capture, Process, and Output. In the Process pipeline, each RGB pixel undergoes filtering or color space conversion using the Video Color Processing (VCP) module.

## Video Color Processing (VCP) Module

The VCP module is equipped with control registers and small local buffers for video frame storage. It receives a stream of video pixels as input, performs computations or modifies the stream via filters, and outputs the processed pixel stream. VCP modules can operate as either pixel transformation or pixel generation units.

The VCP module offers a range of filters and color space conversions that can be enabled at build time.

### Available Filters

- Sharp

- Blur

- Emboss

- Sobel Edge Detection

- Contrast

### Supported Color Space Conversions

- RGB to HSL

- HSL to RGB

- RGB to YCbCr

- YCbCr to RGB

- RGB to CMYK

- RGB to YDbDr

- RGB to CIEXYZ

- RGB to CIEYUV

- RGB to YIQ

- RGB to YPbPr

- RGB to LMS

- RGB to ICtCp

- RGB to HED

- RGB to YC1C2

- RGB to YCbCr

### Additional Features

- Color Correction Matrix

- K-Means Color Clustering

- Test-Pattern Generation

## Camera Sensors and Capture Interface

The Sony IMX477 12.5MP sensor is a 1/2.3-inch CMOS digital image sensor with an active imaging pixel array of 4056H x 3040V, capable of 60 fps at maximum resolution. It is configured via an I2C interface and outputs video through a MIPI interface connected to the MIPI CSI-2 RX subsystem.

The AR1335 13.0MP sensor is a 1/3.2-inch CMOS digital image sensor with a 4208H x 3120V array and supports up to 30 fps. Like the IMX477, it is configured by I2C and outputs via MIPI CSI-2.

The MIPI CSI-2 receiver subsystem includes a MIPI D-PHY core, connecting two data lanes for the IMX477 and four lanes for the AR1335. It captures RAW10 video frames from both cameras, which are then processed by the Demosaic module to convert the Bayer pattern to RGB format.

## Image Processing and Enhancement

The custom VCP module applies various filters, color space conversions, and K-Means color clustering. It also provides image enhancement controls such as contrast, brightness, saturation, white/black balance, and RGB gain, all configurable through AXI4 Lite registers.

Video Direct Memory Access (VDMA) transfers video streams to and from external memory, operating under software control. The image frame resolution is set to 2048x1080 at 60 fps, with a maximum supported resolution of 4056x3040 at 15 fps.

## K-Means Color Clustering Module

This module processes a stream of camera pixel data in pipeline format, presented on input signals (iRgb.red, iRgb.green, iRgb.blue) along with control signals (iRgb.valid, iRgb.eol, iRgb.eof, iRgb.sof). The output is a stream of RGB clusters on the oRgb channel.

The module is synthesized and implemented using Vivado 2022.1 for the KRIA KV260 board and verified with ModelSim 2020.

The functional block diagram illustrates the conversion of RGB data to n clustered values. The K-Means algorithm is applied to input stream pixels:

- Select k RGB reference colors to define clusters.

- Calculate the distance between each input pixel and the k reference points.

- Assign each pixel to the nearest reference value using Euclidean distance.

- Allocate pixels to clusters based on proximity.

Euclidean distance is computed between the original pixel and the reference color schemes. Images use 8 bits per RGB channel, allowing for 16 million colors. K-Means clustering reduces this to n clusters, generating a color palette (codebook) for quantization.

The module converts 24-bit RGB pixels into n-color versions. It includes clock and reset ports, with iRGB and oRGB channels carrying red, green, and blue values with a valid signal.

### K-Means Reference Color Schemes

#### K = 6 Reference Colors

Images generated with K-Means clustering using 6 palette reference colors.

#### K = 51 Reference Colors

Images quantized into 51 clusters using 51 reference colors.

#### IMX477 Camera: K = 90 Reference Colors

Images quantized into 90 clusters using 90 color references.

#### 90 Programmable Reference Color Schemes

The module supports up to 90 programmable color reference schemes for color clustering.

## Video Controller and Camera Interface

### IMX477 Camera: Overview

The IMX477-MIPI-CS is a high-resolution digital camera, featuring a Sony 1/2.3" CMOS sensor with an active array of 4056H x 3040V pixels.

### Specifications and Features

- Back-illuminated, stacked CMOS sensor (Exmor RS)

- Digital Overlap High Dynamic Range (DOL-HDR) with raw output

- High signal-to-noise ratio (SNR)

- Full resolution @ 60 fps (Normal), 4K2K @ 60 fps, 1080p @ 240 fps

- Full resolution @ 40 fps (12-bit Normal), @ 30 fps (DOL-HDR, 2 frames)

- RAW12/10/8, COMP8 video formats

- Power Save Mode

- Pixel binning and V sub-sampling

- Independent flipping and mirroring

- Input clock frequency: 6–27 MHz

- CSI-2 serial data output (MIPI 2-lane/4-lane)

- Dual PLLs for clock generation

- Ambient Light Sensor (ALS)

- Dual sensor synchronization (multi-camera support)

- 7 Kbit OTP ROM for users

- Built-in temperature sensor

- 10/12-bit on-chip A/D conversion

### Camera Interface

The IMX477 camera module connects to the KRIA KV260 board via FPC flat cable. Camera pixel data travels over a dual-lane MIPI CSI-2 interface, using a 15-pin flexible cable.

The Camera Serial Interface (CSI) standard from the MIPI Alliance defines the communication protocol for mobile camera devices. CSI encompasses a physical layer (D-PHY) and a protocol layer (CSI-2).

### Configuration

The IMX477 camera is configured to output 1920x1080p frames in 30-bit RGB format at 60 fps. Both the camera and the MIPI CSI-2 subsystem are preconfigured. The subsystem captures images and outputs them as AXI4 stream video, while the Demosaic module converts raw data to RGB format.

## Ethernet UDP Video Streaming

Real-time UDP video streaming is enabled using the KRIA KV260 board. The design consists of transmitter and receiver platforms. The receiver collects video streams from the IMX477 camera and implements PL logic design, while the transmitter (PS side) handles the UDP/IP hardware protocol stack for high-speed LAN or point-to-point transmission.

UDP/IP streaming utilizes LWIP (LightWeight Internet Protocol), an open-source networking stack supporting both TCP and UDP. The KRIA KV260 board connects directly to the host PC, which must be manually configured with the IP address 192.168.0.42 and subnet mask 255.255.255.0. The FPGA board defaults to 192.168.0.10 if DHCP fails.

FFMPEG on the host PC connects to the transmitter application, and upon receiving video images, invokes ffplay.exe to decode the initial 54-byte video header. LWIP BSP settings impact ethernet performance.

## FPGA Firmware and Software Design

### FPGA Firmware Features

- MIPI Camera interfacing

- Modular and scalable architecture

- IP-Cores for Vivado Design Suite

- AXI4 / AXI-Stream compliance

- Support for Xilinx 7Series, Ultrascale, Ultrascale+, SoC, and MPSoC

### Software Features

- Control and application logic

- Modular and scalable

- Written in C

## Design Modules

### PL (Programmable Logic) Design IPs

### PS (Processing System) Design Modules

- Configuration and initialization of VTC, VDMA, Camera Sensors, and LWIP

## Running UDP Video Streaming on PC

- Run ffplay.exe with the following command to watch real-time UDP streaming:

- ffplay.exe -i -vf vflip -framerate 60 "udp://127.0.0.0:8080" -loglevel quiet