Overview & Purpose

- Describes a video color processing system implemented on a Kria KV260 FPGA board using Raspberry Pi camera modules.

- Captures RGB video via MIPI interface, processes it, and outputs to Ethernet or DisplayPort.

- Supports high-resolution video (up to 4056x3040 @ 15fps, 3840x2160 @ 30fps).

System Architecture

- Three main pipelines: Capture, Process, Output.

- VCP (Video Color Processing) module applies filters and color space conversions.

- Uses AXI4 streaming and AXI4-Lite for data/control.

- VDMA (Video Direct Memory Access) handles video streaming to/from memory.

Supported Cameras & Interfaces

- Sony IMX477 (12.5MP), AR1335 (13MP), IMX219 (8MP), OV5640/OV5647 (5MP), IMX519 (16MP), IMX682 (64MP).

- MIPI CSI-2 RX subsystem with configurable data lanes (2 or 4 lanes).

- Camera configuration via I2C.

Filters & Color Space Conversions

- Filters: Sharp, Blur, Emboss, Sobel Edge Detection, Contrast.

- Color Spaces: RGB, HSL, HSV, YCbCr, CMYK, YDbDr, CIEXYZ, CIEYUV, YIQ, YPbPr, LMS, ICtCp, HED, YC1C2.

- Color Correction Matrix and K-Means Color Clustering for advanced processing.

Technical Details

- Pixel clock and bandwidth calculations provided for various resolutions and formats.

- Modular design with expandable interfaces for video pipeline.

- Demosaic module converts Bayer pattern to RGB.

- Control registers and buffer management for frame processing.

Color Space Conversion Logic

- Detailed explanation of RGB to HSL/HSV conversion algorithms.

- Block diagrams and port descriptions for conversion modules.

- Equations and matrix transformations for each color space.

Image Enhancement

- Adjustable parameters: contrast, brightness, saturation, white/black balance, RGB gain.

- Color adjust matrix for display/light source sensitivity.

K-Means Clustering

- FPGA implementation for color quantization and image segmentation.

- Reduces color depth by clustering RGB values (examples: k=6, 9, 24, 51, 90).

- Euclidean distance used for pixel assignment to clusters.

Video Streaming

- Real-time UDP video streaming using LWIP stack.

- Host PC connects via Ethernet; FFMPEG used for decoding and playback.

- Network configuration details provided.

Testbench & Verification

- VHDL testbench for filter verification.

- UVM-based environment for simulation and validation.

- AXI4-Lite protocol for register access and control.

Sensor Specifications

- Detailed tables for camera sensor specs, supported resolutions, frame rates, and interface pinouts.

Additional Modules

- Local dynamic threshold segmentation for region-based pixel averaging.

- Histogram module for RGB channel analysis.