Overview

This technical document describes a video color processing system implemented on a Xilinx Kria KV260 FPGA board, using Raspberry Pi camera modules. The system captures RGB video data via MIPI CSI-2 interface, processes it through various filters and color space conversions, and outputs the processed video to Ethernet or DisplayPort.

Key Components & Architecture

- Capture Pipeline: Receives raw video frames from CMOS sensors (e.g., Sony IMX477, AR1335) in RAW10 format.

- Process Pipeline: Converts Bayer pattern to RGB, then applies filters and color space conversions via a custom Video Color Processing (VCP) module.

- Output Pipeline: Streams processed video data via AXI4 to memory, Ethernet, or display.

Supported Filters & Color Space Conversions

- Filters: Sharp, Blur, Emboss, Sobel Edge Detection, Contrast, Test Patterns

- Color Spaces: RGB, HSL, HSV, YCbCr, CMYK, YDbDr, CIEXYZ, CIEYUV, YIQ, YPbPr, LMS, ICtCp, HED, YC1C2

- Advanced: K-Means color clustering for color quantization

Technical Details

- Resolution & Frame Rate: Up to 3840x2160 @ 30fps, max 4056x3040 @ 15fps

- Bandwidth Calculations: Detailed formulas for pixel clock, video clock, bandwidth, and line rate for various resolutions.

- FPGA Implementation: Uses AXI4-Lite for register configuration, VDMA for memory access, and modular IP blocks for each filter/conversion.

- Camera Interfaces: Describes pinouts and configuration registers for Sony IMX477 and other sensors.

Color Space Conversion Logic

- RGB ↔ HSL/HSV: Algorithms for converting between RGB and HSL/HSV, including equations for hue, saturation, and lightness/value.

- Other Spaces: Matrix transformations for CMYK, YDbDr, CIEXYZ, CIEYUV, YIQ, YPbPr, LMS, ICtCp, HED, YC1C2.

Image Processing Filters

- Sharp/Blur/Emboss: Convolution-based filters for image enhancement.

- Sobel Edge Detection: 3x3 kernel for vertical/horizontal edge detection.

- Color Adjust Matrix: Customizable 3x3 matrix for color correction and manipulation.

K-Means Clustering

- Purpose: Reduces color depth by grouping similar colors, useful for segmentation and quantization.

- Implementation: FPGA module with AXI4 streaming interface, configurable number of clusters (k).

Verification & Testbench

- Testbench: UVM-based, with classes for driver, monitor, agent, scoreboard, and environment.

- Verification: Simulates filter modules, checks output images, and validates AXI4-Lite transactions.

Additional Features

- Ethernet UDP Streaming: Uses LWIP stack for real-time video transmission.

- Camera Raw Data Handling: Buffering and synchronization of pixel data.

- Histogram Module: Tracks pixel value distribution for each color channel.

Tables & Figures

The document includes extensive tables for:

- Supported video resolutions and clock rates

- Camera sensor specifications

- Color adjust matrix configurations

- K-means reference color schemes

Summary

This document is a comprehensive technical guide for implementing real-time video color processing and enhancement on FPGA hardware, covering everything from camera interfacing and color space conversions to advanced filtering, clustering, and verification.