The document provides a comprehensive overview of a verification testbench designed for filter modules, with a particular focus on the D5M camera interface and associated image processing filters. It details the components, structure, parameter initialization, communication protocols, and phases involved in verifying the design under test (DUT) using Universal Verification Methodology (UVM).

## Verification Testbench Overview

This document provides a detailed overview of a verification testbench developed for filter modules, with a focus on the D5M camera interface and its associated image processing filters. The testbench architecture is meticulously outlined, including its primary components and organizational structure.

### Testbench Components and Structure

The testbench is constructed to facilitate the verification of the design under test (DUT) by leveraging the Universal Verification Methodology (UVM). Key elements include dedicated modules for stimulus generation, response monitoring, and result validation. The environment is tailored to support various filter types and image processing scenarios associated with the D5M camera interface.

### Parameter Initialization and Communication Protocols

The initialization of parameters, communication protocols between components, and the configuration of the testbench are thoroughly addressed. The methodology ensures that all verification scenarios are correctly set up, enabling effective interaction between the stimulus generators and the DUT.

### Verification Phases

The document also details the sequential phases employed during verification, ensuring comprehensive coverage and systematic validation of the filter modules. Each phase is designed to verify specific aspects of the DUT, maintaining a structured approach throughout the verification process.

## Testbench Overview and Components

The verification testbench is designed to generate stimulation for the filter modules by utilizing a VHDL process. This process reads image files and applies RGB signals directly to the filter design under test (DUT). The testbench closely monitors the DUT's responses and produces validated BMP image outputs, which are intended for manual verification by the user.

To enable flexibility and comprehensive verification, the testbench is constructed using configuration techniques, a factory pattern, and a phase build process. These features work together to facilitate the generation and randomization of stimulus. Within the testbench, users can specify different image filters and to apply and the image size to usesulting output images. The responsibility for validating these output images ultimately lies with the user, ensuring that the verification process remains thorough.

## UVM Testbench Architecture for D5M Camera Interface

The Universal Verification Methodology (UVM) testbench is organized as a class-based environment designed to streamline verification activities for the D5M camera interface and its associated filter modules. At the core of this structure is the test component, which acts as the primary entry point for executing verification sequences.

Within the test, an environment is instantiated. This environment serves as the overarching container for all verification elements required to interact with the design under test (DUT). A crucial part of the environment is the inclusion of a D5M agent. The D5M agent is specifically configured to interface directly with the DUT, ensuring that the filter modules receive appropriate stimulus throughout the verification process.

The D5M agent's responsibilities include generating stimuli tailored to the operational requirements of the filter modules. By maintaining a direct connection to the DUT, the agent facilitates comprehensive verification coverage, enabling the system to assess the behavior and responses of the filter modules under various scenarios. This structured architecture enhances the efficiency and reliability of the verification process, supporting thorough validation of the D5M camera interface filters.

## Verification Scenario and Parameter Initialization

The verification process begins with the careful initialization of parameters, which are managed using enable variables defined within the frame enable package. These variables are set during the compilation stage at the unit scope, ensuring that each verification scenario is precisely configured for the intended test conditions.

### Image Size and Filter Definitions

To achieve comprehensive testing, a range of image sis is established. These sizes span from 64x64 pixels up to 1920x1080 pixels, allowing for evaluation across various resolutions. For each image size, macros are provided to facilitate the selection of different filter types. The available filters include sharp, blur, emboss, HSL, HSV, CGain, and Sobel filters, each designed to assess specific aspects of the filter module's behavior and performance within the verification scenarithe o.

## Image Size and Filter Definitions

The following table outlines the defined image sizes and available filter types for use in the frame enable package. Each image size is associated with a corresponding definition, while several filter and color space transformations are also listed for selection in test scenarios.

| Define | Size | Description |
| --- | --- | --- |
| rgb 0 | 64x64 | Defines a small square image resolution of 64 by 64 pixels. |
| rgb 1 | 128x125 | Specifies a medium image size with a width of 128 pixels and a height of 125 pixels. |
| rgb 2 | 255x255 | Represents a larger square image dimension of 255 by 255 pixels. |
| rgb 3 | 1920x1080 | Corresponds to a full HD image resolution of 1920 by 1080 pixels. |
| sharp |  | Enables the sharpening filter for image processing. |
| blur |  | Activates the blurring filter for test scenarios. |
| embross |  | Applies the emboss filter effect to images. |
| hsl |  | Utilizes the HSL (Hue, Saturation, Lightness) color space conversion. |
| hsv |  | Incorporates the HSV (Hue, Saturation, Value) color space conversion. |

## Frame Enable Package: Filter Type and Image Size Configuration

The configuration for filter types and image sizes in each test is detean address and data phase, whereas write transactions involve an address, data, and response phasecifically designed to facilitate the selection of various filters and image resolutions. By utilizing the macros, users can easily specify which filters to apply and the image size to use for individual test scenarios.

This approach ensures a consistent and systematic method for managing filter and size parameters, streamlining the testing process and allowing for flexible adjustments as needed.

## Filter Type Parameters

The filter type parameters are used to control which image filters are turned on or off for specific test scenarios. Each parameter corresponds to a particular filter or color space, and is set to either 1 (enabled) or 0 (disabled). By adjusting these values, users can customize the filter configuration for each test, ensuring that only the desired image processing techniques are applied.

| Parameter | Value | Description |
| --- | --- | --- |
| F_CGA | 1/0 | Enable Cgain color adjustment |
| F_SHP | 1/0 | Enable the Sharp filter |
| F_BLU | 1/0 | Enable Blur filter |
| F_HSL | 1/0 | Enable HSL color space |
| F_HSV | 1/0 | Enable HSV color space |
| F_RGB | 1/0 | Enable RGB color space |
| F_SOB | 1/0 | Enable the Sobel filter |
| F_EMB | 1/0 | Enable Emboss filter |

These parameters allow for flexible and systematic configuration of image filter settings, supporting a wide range of testing requirements.

These parameters configure the current test values and are managed via defined macros.

### AXI4-Lite Module Register Offsets

The document lists RTL register names with their corresponding base address offsets used in AXI4-Lite transactions, including registers such as initAddr, oRgbOsharp, filter_id, threshold, videoChannel, and several configuration and coefficient registers for filters.

| Register Name | Offset | Description |
| --- | --- | --- |
| initAddr | 8'h00 |  |
| oRgbOsharp | 8'h00 |  |
| oEdgeType | 8'h04 |  |
| filter_id | 8'h08 |  |
| aBusSelect | 8'h0C |  |
| threshold | 8'h10 |  |
| videoChannel | 8'h14 |  |
| dChannel | 8'h18 |  |
| cChannel | 8'h1C |  |
| klsk1 to klsk9 | 8'h20-40 |  |
| kls_config | 8'h44 |  |
| alsk1 to alsk9 | 8'h54-74 |  |
| als_config | 8'h78 |  |
| pReg_pointInterest | 8'h7C |  |
| pReg_deltaConfig | 8'h80 |  |
| pReg_cpuAckGoAgain | 8'h84 |  |
| pReg_cpuWgridLock | 8'h88 |  |
| pReg_cpuAckoffFrame | 8'h8C |  |
| pReg_fifoReadAddress | 8'h90 |  |
| pReg_clearFifoData | 8'h94 |  |
| rgbCoordrl to rgbCoordbh | 8'hC8-DC |  |
| oLumTh | 8'hE0 |  |
| oHsvPerCh | 8'hE4 |  |
| oYccPerCh | 8'hE8 |  |

## AXI4-Lite Module Register Map

The table below presents the register names and their corresponding base address offsets used for AXI4-Lite transactions with the Device Under Test (DUT). Each register in the table serves a specific function in configuring and operating the image filter system. This register map is instrumental in facilitating systematic control over the filter features and test parameters through AXI4-Lite protocol interactions.

| Register Name | Offset | Description |
| --- | --- | --- |
| initAddr | 8'h00 |  |
| oRgbOsharp | 8'h00 |  |
| oEdgeType | 8'h04 |  |
| filter_id | 8'h08 |  |
| aBusSelect | 8'h0C |  |
| threshold | 8'h10 |  |
| videoChannel | 8'h14 |  |
| dChannel | 8'h18 |  |
| cChannel | 8'h1C |  |
| klsk1 to klsk9 | 8'h20-40 |  |
| kls_config | 8'h44 |  |
| alsk1 to alsk9 | 8'h54-74 |  |
| als_config | 8'h78 |  |
| pReg_pointInterest | 8'h7C |  |

These offsets define the register map for AXI4-Lite transactions with the DUT  .

## AXI4-Lite Module Register Map

The following table outlines the register names and their corresponding base address offsets used in AXI4-Lite transactions with the Device Under Test (DUT). Each register serves a specific purpose within the configuration and operation of the image filter system, supporting various testing requirements and filter parameter adjustments. The register map facilitates systematic control over filter features and test parameters through AXI4-Lite protocol interactions.

| Register Name | Offset | Description |
| --- | --- | --- |
| initAddr | 8'h00 |  |
| oRgbOsharp | 8'h00 |  |
| oEdgeType | 8'h04 |  |
| filter_id | 8'h08 |  |
| aBusSelect | 8'h0C |  |
| threshold | 8'h10 |  |
| videoChannel | 8'h14 |  |
| dChannel | 8'h18 |  |
| cChannel | 8'h1C |  |
| klsk1 to klsk9 | 8'h20-40 |  |
| kls_config | 8'h44 |  |
| alsk1 to alsk9 | 8'h54-74 |  |
| als_config | 8'h78 |  |
| pReg_pointInterest | 8'h7C |  |

## UVM Driver and AXI4-Lite Communication

### UVM Driver Overview

The UVM driver is an extension of the base uvm_driver class. Its primary responsibility is to receive data items from the sequencer and drive these items to the Device Under Test (DUT). Throughout the run phase, the driver executes both read and write operations on the DUT using a virtual interface handle. This approach enables direct interaction with the DUT and ensures that data is transferred according to the required protocol.

### AXI4-Lite Protocol

The AXI4-Lite protocol facilitates communication between verification components and the DUT by operating across five parallel channels dedicated to read and write transactions. For read transactions, the protocol utilizes address and data phases, whereas write transactions involve address, data, and response phases. Signal naming conventions are consistently applied to distinguish between channels, such as using AR* for read address signals and AW* for write address signals. Flow control within the protocol is governed by the assertion of valid and ready signals, and data transfers are initiated only when both signals are asserted simultaneously.

### Driver Implementation

In terms of implementation, the UVM driver declares a virtual interface to facilitate communication with the DUT. During the build phase, the interface handle is retrieved using uvmconfigdb, allowing the driver to establish the necessary connections. Within the run phase, the driver continuously fetches sequence items and drives the corresponding DUT signals. To manage concurrent operations, such as resetting signals and driving frames, the driver employs fork and join constructs, enabling multiple tasks to execute in parallel and ensuring robust signal management throughout the verification process.

### Reset and Frame Tasks

- Reset Signals: Resets AXI4-Lite and D5M camera input signals upon system reset assertion.

- D5M Frame: Drives signals specified in sequences, fetching items continuously, and executing transfers.

### AXI4 Write and Read Operations

The driver implements tasks for AXI4 write address and data phases, waiting for valid responses with timeout detection and error reporting. Similarly, read address and data phases are handled with appropriate handshaking and response checks. Pixel data read/write tasks manage frame RGB pixels per transaction.

## Monitor and Transaction Collection

The monitor observes DUT activity, collecting events and data for analysis and coverage. It extends uvm_monitor and uses a virtual interface to sample DUT outputs every clock cycle, creating transaction objects with signals such as valid, RGB channels, coordinates, and frame indicators. Transactions are sent through analysis ports for further processing.

## D5M Agent and Scoreboard

The D5M agent, derived from uvm_agent, encapsulates the sequencer, driver, monitor, and coverage components, facilitating modular verification. During the connect phase, it links analysis ports and conditionally connects the driver and sequencer based on the configuration. The scoreboard collects both expected and actual DUT transactions to verify correct functionality by comparing predicted and observed outputs.

## D5M Camera Environment and Test Execution

The D5M camera verification environment is responsible for establishing the necessary components and connections to perform comprehensive testing. Within this environment, the agent and scoreboard are instantiated to facilitate stimulus generation and comparison of results. All relevant classes are registered with the UVM factory, ensuring the flexibility needed for dynamic object creation and management throughout the verification process.

A critical aspect of the environment setup is the connection of components, which enables seamless data flow between elements such as the sequencer, driver, monitor, and scoreboard. This connectivity is essential for accurate transaction tracking and functional coverage analysis.

The test class plays a central role in the execution phase. During the run phase, it initiates sequences on the sequencer, driving verification scenarios based on the specified test case. The test na's command-lined via the simulator command line arguments, allowing for easy selection and execution of different test scenarios.

At the top level, the test bench module orchestrates the overall test flow. It is responsible for building the UVM environment, triggering the sequence of UVM phases—namely build, connect, and run—and managing phase objections. Phase objections are used to coordinate stimulus application and checking activities, ensuring that all necessary operations are completed before the simulation concludes.

Upon successful completion of all verification activities, the simulation terminates gracefully. However, if any required configuration is missing, the test bench issues fatal messages to alert the user and halt further execution.

## D5M Transaction Class

The D5M transaction class encapsulates data items using structure objects and extends the UVM sequence item. It includes randomized data members representing RGB channels, coefficients, AXI4-Lite channels, patterns, and transaction types, capturing signals such as clock, valid, line/frame valid, RGB data, coordinates, and frame indicators.

This detailed summary captures the architecture, components, protocols, and procedures involved in verifying filter modules using a UVM-based testbench for the D5M camera interface, providing a thorough understanding of the testbench design and operation.

# Testbench for Filter Module Verification

# Overview

The verification of filter modules is performed using a VHDL test bench, which generates images and validates the design's functionality. The testbench operates by producing stimulus and monitoring the response within a single process. This process reads an image file, applies RGB stimulus to the filter design section, and waits until the end of the frame to produce a valid output image in BMP format.

## Testbench Components and Objects

The D5M verification testbench is constructed using configuration techniques, a factory, and a phase build process. These elements facilitate the generation of stimulus and randomization. The testbench outlines the image filters within the design, specifying the source of input stimuli or test patterns, and monitors output results in the form of generated images. Manual verification by the user is required to check the filtered output images and determine the pass or fail status of each test pattern.

## UVM Testbench Structure

The UVM testbench is built from classes. The structure consists of a test that contains an environment, which includes the D5M agent. The flow of the testbench involves connecting the Device Under Test (DUT) and the testbench, configuring the D5M agent, and generating stimulus for verification.

## Verification Scenario and Parameter Initialization

In the filter’s generic package, a typical verification scenario for VFP tests involves initializing a set of parameters by defining an enable variable within the frame allow package. This initialization can be performed at the terminal during compilation at the unit scope. The table below lists available defined variables along with their corresponding image sizes for testing purposes:

| Define | Size | Description |
| --- | --- | --- |
| rgb | 0 | 64x64 |
| rgb | 1 | 128x125 |
| rgb | 2 | 255x255 |
| rgb | 3 | 1920x1080 |
| sharp |  |  |
| blur |  |  |
| embross |  |  |
| hsl |  |  |
| hsv |  |  |

`define	size	description

rgb	0=64x64,

1=128x125,

2=255x255,

3=1920x1080

sharp

blur

embross

hsl

hsv

cgain

sobel

## Image Filter Types and Sizes

The following table outlines the available image filter types along with their corresponding size options and descriptions.

A set of parameters for the image filter type and its size is assigned for the current test values, as shown in Table 2, via a defined macro in the frame enable package, according to Table 1.

parameter	value	description

F_CGA	1/0	Enable ‘1’ Cgain color adjust for test.

F_SHP	1/0	Enable ‘1’ Sharp Filter for test.

F_BLU	1/0	Enable ‘1’ Blure Filter for test.

F_HSL	1/0	Enable ‘1’ HSL color space for test.

F_HSV	1/0	Enable ‘1’ HSV color space for test.

F_RGB	1/0	Enable ‘1’ RGB color space for test.

F_SOB	1/0	Enable ‘1’ Sobel filter for test.

F_EMB	1/0	Enable ‘1’ Embross Filter for test.

For AXI4-Lite module registers in RTL, the following parameters, including the offset for the base address, are set for AXI4-Lite transactions, as shown in the table below.

Register Name	Offet	Description

initAddr            	8'h00

oRgbOsharp          	8'h00

oEdgeType           	8'h04

filter_id           	8'h08

aBusSelect          	8'h0C

threshold           	8'h10

videoChannel        	8'h14

dChannel            	8'h18

cChannel            	8'h1C

kls_k1              	8'h20

kls_k2              	8'h24

kls_k3              	8'h28

kls_k4              	8'h2C

kls_k5              	8'h30

kls_k6              	8'h34

kls_k7              	8'h38

kls_k8              	8'h3C

kls_k9              	8'h40

kls_config          	8'h44

als_k1              	8'h54

als_k2              	8'h58

als_k3              	8'h5C

als_k4              	8'h60

als_k5              	8'h64

als_k6              	8'h68

als_k7              	8'h6C

als_k8              	8'h70

als_k9              	8'h74

als_config          	8'h78

pReg_pointInterest  	8'h7C

pReg_deltaConfig    	8'h80

pReg_cpuAckGoAgain  	8'h84

pReg_cpuWgridLock   	8'h88

pReg_cpuAckoffFrame 	8'h8C

pReg_fifoReadAddress	8'h90

pReg_clearFifoData  	8'h94

rgbCoord_rl         	8'hC8

rgbCoord_rh         	8'hCC

rgbCoord_gl         	8'hD0

rgbCoord_gh         	8'hD4

rgbCoord_bl         	8'hD8

rgbCoord_bh         	8'hDC

oLumTh              	8'hE0

oHsvPerCh           	8'hE4

oYccPerCh

## Register Address Overview

The following section lists register names along with their corresponding offset addresses. Each register serves a specific function within the module, aiding in data management and configuration for processing RGB and luminance thresholds, as well as HSV channel parameters.

- pReg_fifoReadAddress (Offset: 8'h90):

- Specifies the address used to read from the FIFO buffer.

- pReg_clearFifoData (Offset: 8'h94):

- Used to clear the data in the FIFO buffer.

- rgbCoord_rl (Offset: 8'hC8):

- Represents the lower byte of the Red coordinate value.

- rgbCoord_rh (Offset: 8'hCC):

- Represents the higher byte of the Red coordinate value.

- rgbCoord_gl (Offset: 8'hD0):

- Represents the lower byte of the Green coordinate value.

- rgbCoord_gh (Offset: 8'hD4):

- Represents the higher byte of the Green coordinate value.

- rgbCoord_bl (Offset: 8'hD8):

## AXI4-Lite Module Register Map

The following table provides a comprehensive overview of the AXI4-Lite module registers, including their names, offset addresses, and brief descriptions. This information is essential for understanding the configuration and operation of the filter modules during verification and testing scenarios.

| Register Name | Offset | Description |
| --- | --- | --- |
| initAddr | 8'h00 |  |
| oRgbOsharp | 8'h00 |  |
| oEdgeType | 8'h04 |  |
| filter_id | 8'h08 |  |
| aBusSelect | 8'h0C |  |
| threshold | 8'h10 |  |
| videoChannel | 8'h14 |  |
| dChannel | 8'h18 |  |
| cChannel | 8'h1C |  |
| kls_k1 | 8'h20 |  |
| kls_k2 | 8'h24 |  |
| kls_k3 | 8'h28 |  |
| kls_k4 | 8'h2C |  |
| kls_k5 | 8'h30 |  |
| kls_k6 | 8'h34 |  |
| kls_k7 | 8'h38 |  |
| kls_k8 | 8'h3C |  |
| kls_k9 | 8'h40 |  |
| kls_config | 8'h44 |  |
| als_k1 | 8'h54 |  |

## Verification Scenario and Parameter Initialization

A typical verification scenario for VFP tests in the filter’s generic package begins with the initialization of parameters, which is accomplished by defining an enable variable within the frame allow package. This initialization process can be executed at the terminal during compilation and is performed at the unit scope.

### Image Size Definitions for Test Patterns

The table below lists the available defined variables along with their corresponding image sizes used in testing. These definitions allow for the selection of various test image dimensions during verification.

| Define | Size | Description |
| --- | --- | --- |
| rgb | 0 | 64x64 |
| rgb | 1 | 128x125 |
| rgb | 2 | 255x255 |
| rgb | 3 | 1920x1080 |
| sharp |  |  |
| blur |  |  |
| embross |  |  |
| hsl |  |  |
| hsv |  |  |

## Variable Definitions and Parameter Assignment for Image Filters

The variables used for image filtering tests are defined through macros within the frame enable package. This approach enables flexible configuration tailored to the specific requirements of each test scenario. By using these macros, both the filter type and the image size can be selected appropriately for each case, ensuring that the tests are tailored to the intended evaluation parameters.

## Parameter Assignment for Filter Types

For each image filter type, a corresponding set of parameters is assigned utilizing the current test values. These parameters are systematically turned on or off for each test case using the defined macros. This method provides precise control over the filter configurations, ensuring that each test scenario can be adjusted as necessary to evaluate different filter behaviors and effects.

| Parameter | Value | Description |
| --- | --- | --- |
| F_CGA | 1 / 0 | Enable ‘1’ for Cgain color adjustment in the test. |
| F_SHP | 1 / 0 | Enable ‘1’ for the Sharp Filter in the test. |
| F_BLU | 1 / 0 | Enable ‘1’ for Blur Filter in the test. |
| F_HSL | 1 / 0 | Enable ‘1’ for HSL color space in the test. |
| F_HSV | 1 / 0 | Enable ‘1’ for HSV color space in the test. |
| F_RGB | 1 / 0 | Enable ‘1’ for RGB color space in the test. |
| F_SOB | 1 / 0 | Enable ‘1’ for the Sobel filter in the test. |
| F_EMB | 1 / 0 | Enable ‘1’ for Embross Filter in the test. |

### AXI4-Lite Module Register Parameters

For the AXI4-Lite module in RTL, the following register parameters are set for each base address offset, which are used during AXI4-Lite transactions. The table details the register names, their offsets, and their descriptions.

| Register Name | Offset | Description |
| --- | --- | --- |
| initAddr | 8'h00 |  |
| oRgbOsharp | 8'h00 |  |
| oEdgeType | 8'h04 |  |
| filter_id | 8'h08 |  |
| aBusSelect | 8'h0C |  |
| threshold | 8'h10 |  |
| videoChannel | 8'h14 |  |
| dChannel | 8'h18 |  |
| cChannel | 8'h1C |  |

## DUT Connections Overview

### Input Protocol Interface

The Design Under Test (DUT) features an input protocol interface that receives incoming signals and data according to the specified protocol. This interface plays a crucial role in facilitating accurate communication and ensuring that data is correctly received for further processing.

### Output Protocol Interface

The DUT also features an output protocol interface, which manages the transmission of data and signals from the DUT to other components in the simulation environment. This interface ensures that outgoing data adheres to protocol specifications, supporting reliable and efficient data transfer.

### Sequence Item

The sequence item represents a unit of data or transaction that is driven to the DUT. The sequencer typically constructs it and contains all the necessary information required to perform a specific operation on the DUT.

### Sequence

A sequence is a collection of related sequence items, organized to execute a series of operations or transactions. Sequences are used to simulate complex behaviors and scenarios by arranging sequence items in a particular order.

### Sequencer

The sequencer coordinates the generation and delivery of sequence items. It interacts with sequences to produce data items and manages their flow to the driver, which ultimately drives the transactions to the DUT.

## UVM Driver Overview

The UVM driver class serves as an extension of the base uvm_driver and plays a critical role in the simulation environment. Its primary responsibility is to receive data items generated by the sequencer and to drive these items to the Design Under Test (DUT). During the run phase, the driver leverages specific methods to execute read and write operations on the DUT, utilizing the DUT interface handle to facilitate these transactions.

## AXI4-Lite Communication Flow

The AXI4-Lite protocol operates across five parallel channels, enabling efficient communication between components. Read transactions are divided into two distinct phases: a request on the read address channel and a response on the read data channel. Conversely, write transactions are separated into three stages: address, data, and response, which also includes status information. Both read and write transactions can occur concurrently, allowing for flexible data flow and improved performance.

### Transaction Phases and Signal Naming

Within the AXI4-Lite protocol, the timing of write address transfers is flexible and may occur before, during, or after the write data phase. Signal naming conventions help distinguish the channels: signals for read address channels begin with AR*, while those for read data channels start with R*. Write address signals are prefixed with AW*, write data signals with W*, and write response signals with B*.

### Flow Control Mechanism

Flow control is implemented on each channel to manage the rate of information exchange, including address, data, and response signals. The source asserts a valid signal to indicate the presence of available information. The destination, in turn, asserts a ready signal to accept incoming information. Typically, the source asserts validity first and waits for the destination to become prepared before transferring the data. Alternatively, the destination may assert itself as ready first and wait for the source to provide the data. In all cases, the actual transfer only occurs when both valid and ready signals are high, ensuring synchronized and reliable communication.

## UVM Driver Implementation Overview

### Virtual Interface Declaration

The UVM driver class is derived from the generic UVM driver component, parameterized with the transaction type d5m_trans. Within the class, the virtual interface must be declared to allow the driver to interact with the DUT signals. The declaration typically takes the following form:

protected virtual d5m_camera_if d5m_camera_vif;

An integer data member, such as id, may also be declared for identification purposes:

Protected int id;

### Constructor

Every component in UVM must have a constructor, new(), which is responsible for initializing the component instance. The constructor must complete execution before the build_phase() or any other subsequent phase can proceed. A typical constructor for the driver class is:

function new (string name, uvm_component parent);

super.new(name, parent);

endfunction: new

### Obtaining the Interface Handle

The interface handle is retrieved using the UVM config DB mechanism. This is done within the build phase to ensure the driver has access to the DUT interface. The get method is used to read the required interface from the configuration database, allowing the driver to interact with the DUT.

### Driving Logic Implementation

In the run phase of the driver, the driver fetches sequence items using seq_item_port.get_next_item() and drives the relevant DUT signals accordingly. The driving logic is responsible for translating the abstract sequence items into pin-level activity on the DUT interface, ensuring the intended test operations are performed.

## Build Phase

The build phase in UVM is executed in a top-down manner, ensuring that parent components are constructed before their child components are built. During this phase, the handle to the Device Under Test (DUT) interface is configured. This is accomplished by retrieving the interface handle using the get method. The uvm_config_db parameterized class is used as a convenient interface for reading from the resource database, providing an abstraction over the uvm_resource_db class. It is important to note that uvm_config_db is derived from uvm_resource_db.

In the context of this driver, the value of the field named d5m_camera_vif is obtained, starting from the current component context, which is represented by this. The following code demonstrates this process:

## Build Phase Implementation

The build_phase method is responsible for configuring the UVM driver during the build stage. It begins by calling the parent class's build_phase method to ensure that base class initializations are performed:

super.build_phase(phase);

Next, the method attempts to retrieve the handle for the d5m_camera_vif virtual interface from the UVM configuration database. This is accomplished using the uvm_config_db#get method, which accesses the resource database to obtain the interface reference required for the driver to interact with the DUT:

if (!uvm_config_db#(virtual d5m_camera_if)::get

(this, "", "d5m_camera_vif", d5m_camera_vif))

If the interface handle cannot be obtained, the method triggers a fatal error using the `uvm_fatal macro. This ensures that simulation does not proceed without the necessary interface, as the driver would be unable to function correctly. The error message includes the full hierarchical name to aid in debugging:

`uvm_fatal("NOVIF", {"virtual interface must be set for:

",get_full_name(), ".d5m_camerSa_vif"});

This structured approach ensures that the UVM driver is properly configured with the required interface before simulation activities begin.

## Run Phase

In the run phase, the method utilizes fork and join constructs to separate threads, enabling the independent execution of tasks that drive each channel. This structure enables the driver to perform multiple operations concurrently. The code below illustrates this approach:

## Run Phase Implementation

The run_phase method initiates the execution of testbench activities within the UVM driver. It leverages SystemVerilog's fork and join constructs to enable parallel execution of tasks, allowing for independent handling of different functional components.

Within the run_phase, two primary tasks are executed concurrently:

- reset_signals(): This task is responsible for managing the reset of signals, ensuring that the necessary signals are properly initialized or cleared when a reset event occurs.

- d5m_frame(): This task handles the driving of frames, facilitating the simulation or transmission of data frames through the channel.

The use of the fork...join structure ensures that both tasks run independently but concurrently, allowing the driver to efficiently manage signal resets while also processing frame operations without waiting for one to complete before starting the other.

## Run Phase Implementation

The run_phase method is designed to execute key activities within the UVM driver, enabling the simulation of concurrent operations for improved efficiency. The implementation utilizes SystemVerilog's fork and join constructs to manage parallel execution of tasks.

Within the run_phase method, two primary tasks are initiated concurrently:

- reset_signals(): This task is responsible for managing the reset of signals. It ensures that the necessary signals are properly initialized or cleared whenever a reset event occurs.

- d5m_frame(): This task handles the driving of data frames, supporting the simulation or transmission of frames through the designated channel.

The fork...join structure ensures that both tasks operate independently but simultaneously. This enables the driver to manage signal resets and frame operations efficiently, without having to wait for one task to complete before starting the next.

## Concurrent Task Execution in run_phase

The run_phase method in the UVM driver is implemented to execute essential operations concurrently, enhancing simulation efficiency. This is achieved using SystemVerilog's fork-join construct, which allows multiple tasks to run independently in parallel.

### Implementation Details

Within the run_phase, two key tasks are launched to monitor the ARESETN signal continuously, to initiate the reset process for signals, ensuring that all necessary signals are properly initialized or cleared upon a reset event.

- d5m_frame(): Handles the driving of data frames, enabling the simulation or transmission of frames through the designated channel.

The use of fork...join ensures that both reset_signals() and d5m_frame() run simultaneously, allowing the driver to efficiently perform signal resets and frame operations without waiting for one task to finish before starting the other.

### Code Structure

The following SystemVerilog code demonstrates the concurrent execution of these tasks within the run_phase method:

## Concurrent Execution in run_phase

The run_phase method is implemented as a virtual task that allows for the concurrent execution of essential driver operations. This is achieved using the SystemVerilog fork...join construct, which enables multiple tasks to run independently and simultaneously. As a result, this approach enhances the simulation's efficiency by preventing delays caused by sequential task execution.

### Method Implementation

Within the run_phase task, two critical functions are launched in parallel:

- reset_signals(): Responsible for managing the reset process of relevant signals.

- d5m_frame(): Handles the driving of data frames through the designated channel.

The concurrent execution is structured as follows:

## Concurrent Execution in run_phase

The run_phase method is implemented as a virtual task, allowing for the concurrent execution of the driver's essential operations. Inside this method, the fork...join construct is used to launch two critical tasks in parallel: reset_signals() and d5m_frame(). This structure enables both processes to execute simultaneously, enhancing simulation efficiency by eliminating delays that would occur if the tasks were performed sequentially.

## Concurrent Execution in run_phase

The run_phase method is implemented as a virtual task, providing the capability to execute essential driver operations concurrently. This concurrency is achieved using SystemVerilog's fork...join construct, which enables multiple tasks to run independently and simultaneously within the simulation environment.

The structure of the run_phase method is as follows:

## Concurrent Execution in run_phase

The run_phase method is declared as a virtual task, enabling the concurrent execution of essential driver operations within the simulation environment. This concurrency is accomplished through the use of the SystemVerilog fork...join construct, which allows multiple tasks to be executed independently and simultaneously.

The structure of the run_phase task is as follows:

virtual task run_phase (uvm_phase phase);

fork

reset_signals();

d5m_frame();

join

In this implementation, the reset_signals() and d5m_frame() tasks are both launched in parallel. The reset_signals() task is responsible for managing the reset process of relevant signals, while d5m_frame() is tasked with driving data frames through the designated channel. By initiating both functions concurrently, the driver is able to handle signal resets and frame operations independently, thereby enhancing simulation efficiency by preventing delays that could result from sequential execution.

Within this method, the reset_signals() task is responsible for handling the reset process of relevant signals, while the d5m_frame() task manages the driving of data frames through the designated channel. By launching these tasks in parallel using fork...join, both reset and frame operations can occur simultaneously. This approach enhances simulation efficiency by preventing delays that would result from sequential task execution and ensures that the driver can manage signal resets and frame operations independently.

By structuring the code in this way, the driver can handle signal resets and frame operations independently, ensuring optimal performance during simulation.

By utilizing fork...join, both reset_signals() and d5m_frame() operate independently, allowing the driver to manage signal resets while simultaneously driving frame operations efficiently.

## Reset Signals Functionality

The reset_signals method is implemented as a virtual protected task, ensuring that the reset operation is managed securely within the simulation environment. This task is designed to reset both the DUT AXI4 Lite interface and the d5m_cam_mod input signals whenever the system reset is asserted, specifically when the ARESETN signal transitions from low to high.

The reset_signals task utilizes a loop that runs continuously to monitor the ARESETN signal. Upon detecting a positive edge of d5m_camera_vif.ARESETN, the following actions are performed:

- All AXI4 Lite interface signals—including AWADDR, AWPROT, AWVALID, WDATA, WSTRB, WVALID, BREADY, ARADDR, ARPROT, ARVALID, and RREADY—are reset to their default values (zeros).

- Input signals related to the d5m_cam_mod component, such as iImageTypeTest, iReadyToRead, fvalid, and lvalid, are also cleared and set to zero.

This structure ensures that, every time a system reset occurs, all relevant input signals are reliably returned to their initial states, thereby preparing the system for subsequent operations and maintaining simulation integrity.

## Reset Signals Task Implementation

The reset_signals method is defined as a virtual protected task, ensuring the secure management of the reset operation within the simulation environment. This task is responsible for resetting both the DUT AXI4 Lite interface and the d5m_cam_mod input signals whenever a system reset is asserted.

Within the reset_signals task, a forever loop is used to continuously monitor the ARESETN signal. Specifically, the loop waits for the positive edge of d5m_camera_vif.ARESETN. When this edge is detected, the following actions are performed to restore all relevant signals to their default values:

## Reset of AXI4 Lite Interface Signals

As part of the reset_signals task, all AXI4 Lite interface signals are reset to their default values when a system reset is detected. This action ensures that the interface is brought back to a known and stable state, which is critical for proper simulation behavior and subsequent operations.

- AWADDR: Set to 8'h0

- AWPROT: Set to 3'h0

- AWVALID: Set to 1'b0

- WDATA: Set to 32'h0

- WSTRB: Set to 4'h0

- WVALID: Set to 1'b0

- BREADY: Set to 1'b0

- ARADDR: Set to 8'h0

- ARPROT: Set to 3'h0

- ARVALID: Set to 1'b0

- RREADY: Set to 1'b0

By resetting each of these signals to zero, the simulation environment ensures that all transactions on the AXI4 Lite interface are properly terminated and that the interface is ready for new operations after the reset event.

d5m_cam_mod input signals are also cleared and set to zero:

iImageTypeTest: 1'b0

iReadyToRead: 1'b0

fvalid: 1'b0

valid: 1'b0

This approach ensures that, upon each system reset, all pertinent input signals are consistently restored to their initial state, thereby maintaining the integrity and reliability of the simulation.

virtual protected task reset_signals();

forever begin

@(posedge d5m_camera_vif.ARESETN);

d5m_camera_vif.axi4.AWADDR        <=  8'h0;

d5m_camera_vif.axi4.AWPROT        <=  3'h0;

d5m_camera_vif.axi4.AWVALID       <=  1'b0;

d5m_camera_vif.axi4.WDATA         <= 32'h0;

d5m_camera_vif.axi4.WSTRB         <=  4'h0;

d5m_camera_vif.axi4.WVALID        <=  1'b0;

d5m_camera_vif.axi4.BREADY        <=  1'b0;

d5m_camera_vif.axi4.ARADDR        <=  8'h0;

d5m_camera_vif.axi4.ARPROT        <=  3'h0;

d5m_camera_vif.axi4.ARVALID       <=  1'b0;

d5m_camera_vif.axi4.RREADY        <=  1'b0;

d5m_camera_vif.d5p.iImageTypeTest <=  1'b0;

d5m_camera_vif.d5p.iReadyToRead   <=  1'b0;

d5m_camera_vif.d5p.fvalid         <=  1'b0;

d5m_camera_vif.d5p.lvalid         <=  1'b0;

end

endtask: reset_signals

## Reset Signals Task Overview

The reset_signals task is defined as a virtual protected task, ensuring safe and controlled execution within the simulation environment. This task is responsible for initializing and resetting all relevant input signals to their default states when a system reset is detected.

### Continuous Monitoring of the Reset Signal

Within the reset_signals task, a forever loop is utilized to continuously monitor the reset signal. The task is triggered on each positive edge of d5m_camera_vif.ARESETN. This design guarantees that the reset process is consistently applied throughout the simulation, maintaining the integrity of the system.

### Resetting AXI4 Interface Signals

During each reset event, the following AXI4 interface signals are set to their initial values:

- AWADDR: 8'h0

- AWPROT: 3'h0

- AWVALID: 1'b0

- WDATA: 32'h0

- WSTRB: 4'h0

- WVALID: 1'b0

- BREADY: 1'b0

- ARADDR: 8'h0

- ARPROT: 3'h0

- ARVALID: 1'b0

- RREADY: 1'b0

### Resetting D5M Camera Module Input Signals

## D5M Frame Method

The d5m_frame method is responsible for driving the signals specified in a sequence using the UVM (Universal Verification Methodology) sequence. This process is managed within a virtual protected task, ensuring that operations are performed in a controlled simulation environment.

Within the d5m_frame task, an infinite loop is established using the forever construct. On each positive edge of the d5m_camera_vif.clkmm clock signal, the following sequence of operations occurs:

- The method retrieves the following item in the sequence by calling seq_item_port.get_next_item(req).

- The drive_transfer(req) function is then invoked to handle the data transfer for the current sequence item.

- Once the transfer is complete, seq_item_port.item_done() is called to indicate that the current item has been processed.

## D5M Frame Processing Procedure

The frame processing procedure is implemented as a virtual protected task named d5m_frame. This task is designed to operate continuously, efficiently managing the processing of sequence items as they become available within the simulation.

The procedure utilizes a forever loop to run indefinitely. At each positive edge of the d5m_camera_vif.clkmm clock signal, the following steps are performed:

- The following sequence item is retrieved by calling seq_item_port.get_next_item(req). This ensures that each item in the sequence is processed promptly as soon as it is available.

- The drive_transfer(req) function is invoked to handle the data transfer associated with the current sequence item. This function manages the actual communication and interaction with the device under test.

This loop continues without interruption, allowing for efficient and orderly processing of all sequence items and ensuring that data transfers are consistently managed throughout the simulation.

## D5M Frame Processing Task Implementation

The d5m_frame procedure is implemented as a virtual protected task, designed to manage the continuous processing of sequence items within the simulation environment. The task operates within an infinite loop, established using the forever construct, ensuring that it remains active throughout the simulation.

At each positive edge of the d5m_camera_vif.clkmm clock signal, the task performs the following sequence of operations:

- Retrieves the next sequence item by invoking seq_item_port.get_next_item(req), ensuring that each available item is processed in a timely manner.

- Initiates the data transfer for the current sequence item by calling the drive_transfer(req) method.

This loop continues indefinitely, efficiently handling all incoming sequence items as they become available. After the data transfer for an item is complete, the task signals that the current item has been processed by calling seq_item_port.item_done().

This structured approach guarantees orderly and consistent management of data transfers, supporting the reliable operation of the overall simulation.

DRIVE TRANSFER

This method, which is master of the AXI4lite interface, writes and reads data at a given address using a bus handshaking protocol. The first valid address is transmitted, and then a valid response is awaited within the given time of 61 clock cycles in the axi4_address method. A timeout is accorded after 62 clock cycles if no response is asserted high on the bvalid signal from the DUT, and the timeout is flagged using the uvm_error macro. If a valid response is claimed, then the axi4_data method writes/reads data depending on the case statement. Oh, then wait for a response from the driver regarding the event; then, the write/read operation can be initiated by calling the d5m_pixel method.

## Drive Transfer Method

The drive_transfer method is implemented as a virtual protected task and serves as the master routine for handling data transactions over the AXI4lite interface. The task takes a d5m_trans d5m_tx transaction object as input and executes a series of operations to facilitate data movement between the testbench and the device under test (DUT).

- Address Phase

- The first step invokes the axi4_address(d5m_tx) method. This operation is responsible for managing the address handshake with the DUT, ensuring that the appropriate address is transmitted in preparation for a read or write transaction.

- Data Phase

- Following the successful address phase, the axi4_data(d5m_tx) method is called. This phase handles the actual data transfer, either writing data to or reading data from the specified address, depending on the nature of the transaction.

- Pixel Data Operation

- Finally, the d5m_pixel(d5m_tx) method is executed. This step manages pixel-specific data operations associated with the transaction, completing the sequence of actions required for the transfer.

Each invocation of the drive_transfer task encompasses these three key phases, ensuring a structured and reliable approach to AXI4lite data communication within the simulation environment.

## AXI4 Address Channel

The axi4_address method is implemented as a virtual protected task and is responsible for managing transactions on the AXI4 address channel. This method determines whether a write or read operation should be performed based on the transaction type specified in the d5m_tx transaction object.

Within the axi4_address task, a case statement evaluates the d5m_tx.d5m_txn field to identify the nature of the transaction:

- AXI4_WRITE: In this scenario, the axi4_write_address(d5m_tx) method is invoked to initiate a write operation on the address channel.

- AXI4_READ: If a read transaction is specified, the axi4_wread_address(d5m_tx) method is called to facilitate a read operation on the address channel.

This structure ensures that the appropriate address handling routine is executed in accordance with the transaction type, thereby supporting streamlined communication with the device under test (DUT) via the AXI4lite protocol.

## AXI4 Address Channel Method Implementation

The axi4_address method is defined as a virtual protected task that manages address channel transactions in the AXI4lite protocol. This method accepts a transaction object, d5m_tx, which specifies the type of transaction to be performed.

Within this task, a case statement examines the d5m_tx.d5m_txn field to determine whether the transaction is a write or a read. For write transactions (AXI4_WRITE), the axi4_write_address(d5m_tx) method is called to initiate the write operation on the address channel. For read transactions (AXI4_READ), the axi4_wread_address(d5m_tx) method is invoked to perform the read operation on the address channel. This structure ensures that the appropriate routine is executed based on the transaction type.

The method concludes with the endtask: axi4_address statement, marking the end of the task definition.

## AXI4 Address Channel Method Implementation

The axi4_address method is defined as a virtual protected task that manages address channel transactions in the AXI4lite protocol. This method accepts a transaction object, d5m_tx, which specifies the type of transaction to be performed.

Within this task, a case statement examines the d5m_tx.d5m_txn field to determine the nature of the transaction. If the transaction is a write operation (AXI4_WRITE), the method axi4_write_address(d5m_tx) is called to initiate the write process on the address channel. Conversely, if the transaction is a read operation (AXI4_READ), the method axi4_wread_address(d5m_tx) is invoked to perform the read process on the address channel.

This structure ensures that the appropriate address handling routine is executed in accordance with the transaction type, supporting effective communication with the device under test (DUT) using the AXI4lite protocol.

## AXI4 Address Channel Method Implementation

The axi4_address method is implemented as a virtual protected task. Its purpose is to handle address channel transactions in the AXI4lite protocol. The method takes one argument, d5m_tx, which is a transaction object containing details about the current transaction.

Within the body of the task, a case statement is used to examine the d5m_tx.d5m_txn field. This field determines the type of transaction that should be performed. Based on its value, the method proceeds as follows:

- AXI4_WRITE: If the transaction is a write operation, the axi4_write_address(d5m_tx) method is called. This initiates the address phase for a write transaction on the AXI4 address channel.

- AXI4_READ: If the transaction is a read operation, the axi4_wread_address(d5m_tx) method is invoked. This initiates the address phase for a read transaction on the AXI4 address channel.

The structure of this method ensures that the correct address handling routine is executed according to the transaction type. This approach supports robust and efficient communication with the device under test (DUT) using the AXI4lite protocol.

## AXI4 Address Channel Method Implementation

The axi4_address method is implemented as a virtual protected task. Its primary role is to manage address channel transactions in the AXI4lite protocol by determining the type of transaction to be performed using the d5m_tx transaction object.

Within the body of the axi4_address task, a case statement evaluates the d5m_tx.d5m_txn field. Depending on its value, the method executes one of two address handling routines:

- AXI4_WRITE: If the transaction is a write operation, the method calls axi4_write_address(d5m_tx) to initiate the address phase for a write transaction on the AXI4 address channel.

- AXI4_READ: If the transaction is a read operation, the method invokes axi4_wread_address(d5m_tx) to initiate the address phase for a read transaction on the AXI4 address channel.

This structure ensures that the appropriate address handling routine is executed based on the transaction type defined in d5m_tx.d5m_txn.

virtual protected task axi4_address (d5m_trans d5m_tx);

case (d5m_tx.d5m_txn)

AXI4_WRITE : axi4_write_address(d5m_tx);

AXI4_READ  : axi4_wread_address(d5m_tx);

endcase

endtask: axi4_address

AXI4 DATA CHANNEL

In this method, write/read AXI4 data channel through AXI4_write data and AXI4_read_data methods on the selected case.

virtual protected task axi4_data (d5m_trans d5m_tx);

bit[31:0] rw_data;

bit err;

rw_data = d5m_tx.axi4_lite.data;

case (d5m_tx.d5m_txn)

AXI4_WRITE : axi4_write_data(d5m_tx);

AXI4_READ  : axi4_read_data(rw_data, err);

endcase

endtask: axi4_data

D5M PIXEL

In this method, d5m read/write frame rgb pixel per transaction.

virtual protected task d5m_pixel (d5m_trans d5m_tx);

case (d5m_tx.d5m_txn)

D5M_WRITE  : d5m_write_pixel_data(d5m_tx);

IMAGE_READ : d5m_read_pixel_data(d5m_tx);

endcase

endtask: d5m_pixel

D5M WRITE PIXEL DATA

In this method, write data to d5m camera mod from d5m_trans s equence.

virtual protected task d5m_write_pixel_data (d5m_trans d5m_tx);

d5m_camera_vif.d5p.iReadyToRead    <= 1'b0;

d5m_camera_vif.d5p.iImageTypeTest <= d5m_tx.d5p.iImageTypeTest;

d5m_camera_vif.d5p.rgb                       <= d5m_tx.d5p.rgb;

d5m_camera_vif.d5p.fvalid                   <= d5m_tx.d5p.fvalid;

d5m_camera_vif.d5p.lvalid                   <= d5m_tx.d5p.lvalid;

endtask: d5m_write_pixel_data

D5M READ PIXEL DATA

In this method, configure the test type during the read operation and wait for the end-of-frame pulse to occur.

virtual protected task d5m_read_pixel_data(d5m_trans d5m_tx);

@(posedge d5m_camera_vif.clkmm);

d5m_camera_vif.d5p.iImageTypeTest  <= 1'b0;

d5m_camera_vif.d5p.iReadyToRead    <= 1'b1;

forever begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.d5m.eof) break;

end

endtask: d5m_read_pixel_data

AXI4 WRITE ADDRESS

In this method, write the address and assert the write valid high and then wait for a response from the dut BVALID signal within 62 clock cycles.

virtual protected task axi4_write_address (d5m_trans d5m_tx);

int axi_lite_ctr;

d5m_camera_vif.axi4.AWADDR  <= {8'h0, d5m_tx.axi4_lite.addr};

d5m_camera_vif.axi4.AWPROT  <= 3'h0;

d5m_camera_vif.axi4.AWVALID <= 1'b1;

// wait for write response

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.BVALID) break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver", "AWVALID timeout");

end

endtask: axi4_write_address

AXI4 WRITE DATA

virtual protected task axi4_write_data (d5m_trans d5m_tx);

int axi_lite_ctr;

d5m_camera_vif.axi4.WDATA  <= d5m_tx.axi4_lite.data;

d5m_camera_vif.axi4.WSTRB  <= 4'hf;

d5m_camera_vif.axi4.WVALID <= 1'b1;

@(posedge d5m_camera_vif.clkmm);

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.WREADY)

d5m_camera_vif.axi4.AWADDR  <= 8'h0;

d5m_camera_vif.axi4.AWPROT  <= 3'h0;

d5m_camera_vif.axi4.AWVALID <= 1'b0;

break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver", "AWVALID timeout");

end

@(posedge d5m_camera_vif.clkmm);

d5m_camera_vif.axi4.WDATA  <= 32'h0;

d5m_camera_vif.axi4.WSTRB  <= 4'h0;

d5m_camera_vif.axi4.WVALID <= 1'b0;

// wait for write response

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.BVALID) break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver", "BVALID timeout");

end

else begin

if (d5m_camera_vif.axi4.BVALID == 1'b1 && d5m_camera_vif.axi4.BRESP != 2'h0)

`uvm_error("axi_lite_master_driver", "Received ERROR Write Response");

d5m_camera_vif.axi4.BREADY <= d5m_camera_vif.axi4.BVALID;

@(posedge d5m_camera_vif.clkmm);

end

endtask: axi4_write_data

AXI4 WREAD ADDRESS

virtual protected task axi4_wread_address (d5m_trans d5m_tx);

int axi_lite_ctr;

d5m_camera_vif.axi4.ARADDR  <= {8'h0, d5m_tx.axi4_lite.addr};

d5m_camera_vif.axi4.ARPROT  <= 3'h0;

d5m_camera_vif.axi4.ARVALID <= 1'b1;

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.ARREADY) break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver", "ARVALID timeout");

end

@(posedge d5m_camera_vif.clkmm);

d5m_camera_vif.axi4.ARADDR  <= 8'h0;

d5m_camera_vif.axi4.ARPROT  <= 3'h0;

d5m_camera_vif.axi4.ARVALID <= 1'b0;

endtask: axi4_wread_address

AXI4 READ DATA

In this method, the Axi4Lite reads data.

virtual protected task axi4_read_data (output bit [31:0] data, output bit error);

int axi_lite_ctr;

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.RVALID) break;

end

data = d5m_camera_vif.axi4.RDATA;

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver", "RVALID timeout");

end

else begin

if (d5m_camera_vif.axi4.RVALID == 1'b1 && d5m_camer a_vif.axi4.RRESP != 2'h0)

`uvm_error("axi_lite_master_driver","Received ERROR Read Response");

d5m_camera_vif.axi4.RREADY <= d5m_camera_vif.axi4.RVALID;

@(posedge d5m_camera_vif.clkmm);

end

endtask: axi4_read_data

## Monitor

The monitor is a critical component within the verification environment, responsible for obtaining and collecting events and data related to activity in the Design Under Test (DUT). The information gathered by the monitor, whether from the DUT or from stimulus, is used by checkers, the scoreboard, and coverage mechanisms to assess system behavior.

### Class Declaration

This class extends from uvm_monitor and is specifically implemented as d5m_mon_dut:

class d5m_mon_dut extends uvm_monitor;

### Data Members

- protected virtual d5m_camera_if d5m_camera_vif; – Virtual interface to the DUT.

- protected int id; – Identifier for the monitor instance.

- uvm_analysis_port #(d5m_trans) mon_d5m_dut; – Analysis port for sending transaction data.

### Constructor

For each component, the constructor must execute and complete to instantiate the component. The new() method runs before build() or any subsequent phases.

function new (string name, uvm_component parent);

super.new(name, parent);

endfunction: new

### Build Phase

During the build phase, the DUT interface handle is configured using the get method. The uvm_config_db parameterized class provides a convenient interface for reading from the resource database. It is derived from uvm_resource_db. The value of the field "d5m_camera_vif" is fetched using this as the starting search point.

function void build_phase (uvm_phase phase);

super.build_phase(phase);

if(!uvm_config_db#(virtual d5m_camera_if)::get(this, "", "d5m_camera_vif", d5m_camera_vif))

`uvm_fatal("NOVIF",{"virtual interface must be set for: ",get_full_name(), ".d5m_camera_vif"});

mon_d5m_dut = new("mon_d5m_dut", this);

endfunction: build_phase

### Run Phase

Within the run phase, the monitor calls the collection transaction method to gather activity and transaction data from the DUT for further analysis and coverage.

## Collecting Transactions in d5m_monitor

The collect_transactions method is a virtual, protected task within the d5m_monitor class. This task is responsible for capturing transaction data from the design under test (DUT) through the virtual interface. It continually samples the DUT outputs at every positive edge of the clock signal, d5m_camera_vif.clkmm. For each clock cycle, a new d5m_trans transaction object, rx_fdut, is created to store the current values of the DUT outputs. The task assigns the relevant fields of rx_fdut.d5m with values from the virtual interface, including signals such as valid, red, green, blue, RGB, LVALID, FVALID, X, Y, and EOF. Once the transaction is populated, it is written to the mon_d5m_dut analysis port for further processing and observation.

The d5m_monitor class extends from uvm_monitor. Its primary function is to observe and capture transactions from the DUT, utilizing the virtual interface obtained during the build phase through the configuration database. The monitor declares TLM analysis ports, specifically mon_d5m_dut for sending input items and d5m_mon_prd for predicted values. These ports are specialized for the transaction type and are constructed during the build phase. TLM connections between components are established as part of the monitor’s setup, ensuring that transaction data is correctly propagated within the verification environment. Notably, the TLM classes are instantiated directly using the new keyword, rather than being extended.

## D5M Agent

The D5M agent class is derived from the UVM agent and serves as a reusable verification component within the environment. It encapsulates several essential elements: a sequencer, a driver, a monitor, and coverage. By bundling these components together, the agent streamlines verification activities and promotes modularity.

During the connect phase, the agent establishes connections between its analysis ports and the corresponding ports of the monitor. This ensures that transaction data is efficiently communicated for observation and analysis. Additionally, the agent conditionally connects to the driver’s port and the sequencer export, depending on the agent’s configuration. When the agent is configured to be active, these connections are enabled, allowing the driver and sequencer to interact seamlessly as part of the verification process.

## Scoreboard

The scoreboard serves as a critical element within the verification environment, designed to determine whether the design under test (DUT) functions as intended. Its primary role is to collect data regarding the DUT’s operation and perform comparisons against expected values.

The D5M camera scoreboard class is derived from the uvm_component base class. It operates by receiving expected results transactions from a predictor and actual DUT output transactions from a monitor. By observing transactions at the input of the DUT, the scoreboard computes the expected effects of those transactions and stores a suitable representation for later verification. This mechanism enables the scoreboard to check the correspondence between predicted and actual outputs when the corresponding transaction is asserted from the DUT output.

## D5M Camera Environment

The D5M camera environment is responsible for establishing and configuring the verification environment used to stimulate the design under test (DUT). This environment begins by creating and configuring an agent, which is tasked with generating the necessary transactions to drive the DUT. In addition to the agent, the environment instantiates a scoreboard associated with the agent to facilitate thorough checking and comparison of the DUT outputs.

To enable reuse and flexibility, the environment registers its class in the UVM factory. This allows for dynamic creation and replacement of environment components as needed. Handles to key elements, such as the agent and the scoreboard, are declared and maintained within the environment to enable their interaction and management.

A crucial step in setting up the environment involves connecting the agent to the scoreboard. This connection ensures that transaction data generated by the agent and observed by its monitor can be passed to the scoreboard for checking against expected outcomes. By establishing these connections, the environment supports comprehensive verification activities and promotes the modularity and scalability of the testbench.

## Test Class Execution

The test class is responsible for initiating and executing sequences on the specified sequencer during the run phase. The process begins with the simulator command line, where the test name is determined using the +UVM_TESTNAME=test argument. This instructs the simulator to identify and run the desired test.

Upon receiving the test name, the UVM factory creates an instance of the specified test class. The factory mechanism ensures that the correct test component is constructed and registered within the UVM environment. Following instantiation, the run_test task is invoked from the static part of the test bench, typically located in the initial block. This task is responsible for starting the phase methods associated with the test, leading to the coordinated execution of the verification flow.

## Top-Level Test Bench Module

The top-level test bench module plays a crucial role as the main entry point for the verification environment. Its central function is to oversee and manage the entire test flow, beginning with the construction of the root component of the Universal Verification Methodology (UVM) environment.

Upon invocation of the run_test method, the top-level module is responsible for creating an instance of the selected test. This process initiates the UVM infrastructure and triggers all corresponding UVM phases, ensuring that each component within the environment is initialized correctly and coordinated for simulation.

During execution, the top-level test bench module begins by building the UVM environment from the top down, ensuring that all components, such as agents, monitors, drivers, and scoreboards, are properly initialized and connected. It obtains a reference to the uvm_root class instance from the UVM core services, which is essential for coordinating the UVM infrastructure and phase sequencing.

The UVM infrastructure begins with the build phase and continues through subsequent phases, including connect and run sub-phases, until all UVM phases are completed. Once all phases are finished, UVM calls $finish to conclude the simulation and returns control to the initial block of the top test bench module. In cases where no test or environment is created, the module issues a fatal message to notify the user of the missing configuration.

Tests can be executed within the UVM environment by specifying the test name as an argument to run_test or by providing the test name as a command line argument. After completing the build, connect, and end_of_elaboration phases, the start_of_simulation phase is initiated. This phase displays banners and provides information about the test bench topology and configuration.

Following the start_of_simulation phase, the run phase begins. This phase is dedicated to stimulus generation, checking activities, and the parallel execution of all uvm_component run phase tasks. The main phase is initiated within each run phase, during which the specified test case generates and applies stimulus to the DUT.

To manage the progression between phases, the UVM phase object provides methods such as raise_objection () and drop_objection(). These methods are commonly used in the user test's run phase to control the start and completion of stimulus sequences. Once all objections are dropped, the test bench transitions to the next non-time-consuming cleanup phase, and the test concludes.

The objection mechanism allows hierarchical status communication among components. When all raised objections are dropped for the run phase, the phase ends, signaling the completion of stimulus and checking activities.

When run_test method is called, it first creates the object of top test and then call all phases. It constructs the root component of the uvm environment in the top test which than trigger and initiate the uvm phasing component. Basically, calling run_test task causes the selected test to be constructed which first build uvm environment from top to downward and responsible for getting a reference to the uvm_root class instance from UVM core services. UVM infrastructure build phase start from selected test and continue to flow for next phasing until all uvm phases are completed which include connect and run sub phases. UVM calls $finish once all the phases are completed and return the control to top test bench module initial block.  If no test and environment is created than fatal message will issued. However, test can be run in a uvm environment by specifying the test name as argument to run_test or call test name in command line argument. Once build, connect and end_of_elaboration phases are completed start_of_simulation phase gets initiated before time consuming run phase. This phase display banners and test bench topology and configuration information.

After start_of_simulation phase run phase gets initiated which is used for the stimulus generation and checking activities of the test bench and execution of all uvm_component in parallel. Each uvm_component run phase task run in parallel where main phase gets initiated, and stimulus of specified test case is generated and applied to the dut.

Most commonly in the user test, the uvm_phase object is used to raise and drop objection to void moving on the next phase, raise_objection() and drop_objection () are the methods to that. Both methods are used in the user test in the run phase and in between raise and drop objection sequence get started. When drop objection condition is met, action taken is to move to next phase of non-time-consuming cleanup phase and finally test ends.

Raise and drop objection mechanism allow hierarchical status communication among components and once all raised objections are dropped for run phase than phase end.

There are various types of image filters and color space implemented in rtl which need to be verified. Therefore, for each filter and color space tests has been created to verify rtl code. Each test is capable to configure various image dimension size and vfp configuration registers.

## D5M Transaction Class Overview

The D5M Transaction class is designed to encapsulate D5M data items by utilizing structure objects. This approach provides a well-organized method for representing the various data elements necessary for D5M transactions within the testbench environment.

### Class Declaration

The D5M transaction class is derived from the uvm_sequence_item base class. This derivation ensures that the class is fully compatible with the UVM sequence and transaction infrastructure.

class d5m_trans extends uvm_sequence_item;

### Data Members

The d5m_trans class includes the following data members:

- rand rgb_channel vfp;

- rand rgb_channel d5m;

- rand cof_channel cof;

- rand axi4_lite_channel axi4_lite;

- rand pattern_channel d5p;

- vfp_axi4 axi4;

- rand d5m_txn_e d5m_txn;

### Details of Data Members

The data members listed above represent specific D5M data items. Both the vfp and d5m variables are of type rgb_channel. These variables include members such as clock, valid, line valid, frame valid, end of frame, start of frame, RGB channel data, as well as x and y coordinate information.