# Video Color Processing

Draft Version: 2.0Sakinder Ali

# Overview

This section covers the fundamental aspects of video color processing, outlining the concepts and approaches included in the draft version 2.0 authored by Sakinder Ali.

## Draft Version Information

Current Version: 2.0

Author: Sakinder Ali

## Scope

This part of the document presents an introduction to video color processing, which will later be elaborated with details on architecture, bandwidth calculations, color space conversions, and related technical aspects as outlined in the following sections.

# Video Color Processing

Draft Version: 2.0 Sakinder Ali

# Overview

This section introduces fundamental concepts of video color processing, summarizing the topics addressed in draft version 2.0 by Sakinder Ali.

Draft Version Information

Current Version: 2.0

Author: Sakinder Ali

Scope

This segment provides an introduction to video color processing. Subsequent sections will present detailed information regarding architecture, bandwidth calculations, color space conversions, and other relevant technical aspects.

Building upon the introductory overview, the subsequent chapters delve into the practical and theoretical aspects that underpin video color processing. Readers will move from foundational concepts toward a comprehensive exploration of Vivado block diagram IP customization, examining the architecture that supports efficient video data handling. The document then advances to critical calculations for bandwidth and data rate, ensuring accurate management of video streams. A focused section on CMOS pixel capture highlights the importance of sensor technology in acquiring high-quality color information. Further, an in-depth analysis of color spaces—such as HSL and the conversion methodologies from RGB to HSV—provides insight into the manipulation and interpretation of color data, setting the stage for advanced topics within the broader scope of video processing.

Contents

VIDEO COLOR PROCESSING	1

INTRODUCTION	9

VIVADO BLOCK DIAGRAM IP CUSTOMIZATION	11

ARCHITECTURE	15

BANDWIDTH AND DATA RATE CALCULATION	17

CMOS PIXEL CAPTURE	18

VCP	20

HSL COLOR SPACE	21

RGB TO HSV	22

HSV TO RGB	26

HSV COLOR SPACE	28

HSL COLOR RANGE SPACE	31

CMYK COLOR SPACE	35

YDBDR COLOR SPACE	37

CIEXYZ COLOR SPACE	38

CIEYUV COLOR SPACE	39

YIQ COLOR SPACE	40

YPBPR COLOR SPACE	41

I1I2I3 OHTA COLOR INTENSITIES	42

LMS COLOR SPACE	43

ICTCP COLOR SPACE	44

HED COLOR SPACE	45

YC1C2 COLOR SPACE	46

SHARP FILTER	47

BLUR FILTER	48

EMBOSS FILTER	50

SOBEL FILTER	51

YCBCR COLOR SPACE	53

COLOR ADJUST MATRIX	55

AN FPGA IMPLEMENTATION OF K-MEANS CLUSTERING	75

VIDEO CONTROLLER INTERFACE	85

LOCAL DYNAMIC THRESHOLD SEGMENTATION	86

IMAGE CONTRAST AND BRIGHTNESS	87

CAMERA RAW DATA	88

THREE TAPS DATA	89

FOUR TAPS DATA	89

PIXEL COORDINATES	89

CAMERA RAW DATA	91

HISTOGRAM	94

ETHERNET UDP VIDEO STREAMING	96

VIDEO STREAM DATA AND FORMAT	97

SONY IMX477/IMX682/IMX519/IMX219 AND ONSEMI AR1335 CAMERAS VIDEO STREAMING	98

LWIP ETHERNET INTERFACE	98

5MP OV5640 SENSOR INTERFACE	98

5MP OV5647 SENSOR INTERFACE	98

8MP IMX219 SENSOR INTERFACE	98

SONY 12.5MP IMX477 SENSOR	100

SPECIFICATIONS	100

DRIVE MODE	100

FEATURES	100

INTERFACE	101

16MP IMX519 SENSOR INTERFACE	104

64MP IMX682 SENSOR INTERFACE	104

TESTBENCH	105

TESTBENCH COMPONENTS/OBJECTS	106

DUT CONNECTIONS	110

SEQUENCE ITEM	111

SEQUENCE	112

SEQUENCER	113

DRIVER	114

MONITOR	122

AGENT

## Sensor Interfaces and Testbench Architecture

### Sensor Interfaces

The design incorporates two primary sensor interfaces: the 16MP IMX519 sensor interface and the 64MP IMX682 sensor interface. Both interfaces are integral to the overall system, allowing for the connection and data acquisition from high-resolution image sensors.

### Testbench Overview

The testbench is a critical component used to validate the design. It is composed of multiple objects and components that work together to simulate real-world conditions and ensure proper functionality of the sensor interfaces.

### Testbench Components

- Testbench Components/Objects: These are the fundamental building blocks of the testbench, each serving a specific role in simulation and verification.

- DUT Connections: The Device Under Test (DUT) is connected to the testbench through defined interfaces, enabling controlled interactions during testing.

- Sequence Item: Represents individual transactions or operations that are executed as part of the test sequence.

- Sequence: A collection of sequence items that defines the order and manner in which operations are performed.

- Sequencer: Manages the dispatch and execution of sequences, coordinating the flow of transactions to the DUT.

- Driver: Drives the signals and data to the DUT, simulating the behavior of external interfaces.

- Monitor: Observes the DUT's responses and records the outcomes for analysis and verification.

124

SCOREBOARD	124

ENVIRONMENT/ENV	125

TEST	125

D5M TRANSECTION	126

TESTBENCH_TOP	127

Figure 1 The overall structure of implemented design, which shows video RX and TX video data flow.	10

Figure 2	15

Figure 3	20

Figure 4	22

Figure 5	22

Figure 6: Hue Numerator Logic	23

Figure 7: Hue Denominator Logic	23

Figure 8: Hue Degree Logic	24

Figure 9 : HSL Filter Wave Diagram	24

Figure 10	24

Figure 11	24

Figure 12: RGB image	24

Figure 13: HSL converted image	24

Figure 14: RGB image	24

Figure 15: HSL converted image	24

Figure 16: HSL Image	25

Figure 17: Hue channel	25

Figure 18: Saturate channel	25

Figure 19: Luminosity channel	25

Figure 20: HSL Image	25

Figure 21: Hue channel	25

Figure 22: Saturate channel	25

Figure 23: Luminosity channel	25

Figure 24	25

Figure 25	26

Figure 26	26

Figure 27	26

Figure 28	26

Figure 29	26

Figure 30	27

Figure 31	27

Figure 32	28

Figure 33	28

Figure 34	28

Figure 35	28

Figure 36	29

Figure 37	29

Figure 38	29

Figure 39	29

Figure 40	29

Figure 41	29

Figure 42	29

Figure 43	29

Figure 44	29

Figure 45	29

Figure 46	29

Figure 47	29

Figure 48	30

Figure 49	30

Figure 50	30

Figure 51	30

Figure 52	31

Figure 53	31

Figure 54	31

Figure 55	31

Figure 56	31

Figure 57	31

Figure 58	31

Figure 59	31

Figure 60	32

Figure 61	32

Figure 62	32

Figure 63	32

Figure 64	32

Figure 65	32

Figure 66	32

Figure 67	32

Figure 68	33

Figure 69	33

Figure 70	33

Figure 71	33

Figure 72	33

Figure 73	33

Figure 74	33

Figure 75	33

Figure 76	33

Figure 77	33

Figure 78	33

Figure 79	33

Figure 80	34

Figure 81	34

Figure 82	34

Figure 83	34

Figure 84	34

Figure 85	34

Figure 86	34

Figure 87	34

Figure 88	35

Figure 89	35

Figure 90	35

Figure 91	35

Figure 92	36

Figure 93	36

Figure 94 RGB Image	37

Figure 95 YDBDR Image	37

Figure 96 RGB Image	37

Figure 97 YDBDR Image	37

Figure 98	38

Figure 99	38

Figure 100	38

Figure 101	38

Figure 102	39

Figure 103	39

Figure 104	39

Figure 105	39

Figure 106	40

Figure 107	40

Figure 108	40

Figure 109	40

Figure 110	41

Figure 111	41

Figure 112	41

Figure 113	41

Figure 114	42

Figure 115	42

Figure 116	42

Figure 117	42

Figure 118	42

Figure 119	43

Figure 120	43

Figure 121	43

Figure 122	43

Figure 123	43

Figure 124	44

Figure 125	44

Figure 126	44

Figure 127	44

Figure 128	44

Figure 129	45

Figure 130	45

Figure 131	45

Figure 132	45

Figure 133	45

Figure 134	46

Figure 135	46

Figure 136	46

Figure 137	46

Figure 138	46

Figure 139	47

Figure 140	48

Figure 141	52

Figure 142	53

Figure 143	54

Figure 144	55

Figure 145	75

Figure 146	76

Figure 147	76

Figure 148 : camera raw data module	88

Figure 149 : General view of camera raw data flow	88

Figure 150	89

Figure 151	89

Figure 152	90

Figure 153	91

Figure 154	91

Figure 155	94

Figure 156	94

Figure 157	95

Figure 158	109

Figure 159	110

Figure 160	115

INTRODUCTION

Video color processing based upon implementation of raspberry pi camera link running on a Kria KV260 board.  This design capture rgb video data via MIPI interface and process video data into axi4 stream and output to ethernet /display port.

This design takes 10-bit RGB format video stream and apply various filters and color space conversions. At high level, the design comprises of three pipelines: Capture, Process and Output pipelines. During process pipeline each rgb pixel either filtered or converted into color space which is implemented video color processing (VCP) module.

VCP module contains a collection of control registers and maintain a local small buffers to store video frame lines. A VCP module takes input video pixel stream, performs computation, or adds new contents to the stream through filters, and then outputs the processed pixel stream. VCP module can be either a pixel transformation or a pixel generation.

VCP have following filters and color space conversion which can enabled before build:

Color Correction Matrix

K-Mean Color Clustering

Test-Pattern

Filters

SHARP

BLUR

EMBOSS

SOBEL EDGE DETECTION

CONTRAST

Color Space Conversions

RGB TO HSL

HSL TO RGB

RGB TO YCBCR

YCBCR TO RGB

RGB TO CMYK

RGB TO YDBDR

RGB TO CIEXYZ

RGB TO CIEYUV

RGB TO YIQ

RGB TO YPBPR

RGB TO LMS

RGB TO ICTCP

RGB TO HED

RGB TO YC1C2

RGB TO YCBCR

Image frame resolution is set to 3840x2160 at 30 frames per second and maximum full resolution of 4056x3040 is also supported but limited to 15 frames per second.

The simplified block diagram for implemented end-to-end pipeline design is shown in Figure 1. The design is appliable to process video from both connected cameras AR1335 and IMX477 and transmit video stream to output components display and ethernet ports.

Figure 1 The overall structure of implemented design, which shows video RX and TX video data flow.

The Sony IMX477 12.5MP sensor is a 1/2.3-inch CMOS digital image sensor with an active imaging pixel array of 4056H x 3040V 60fps@max resolution. This sensor is configured by I2C interface and has MIPI output interface that is connected to the MIPI CSI-2 RX subsystem inside.

The AR 1335 13.0MP sensor is a 1/3.2-inch CMOS digital image sensor with an active imaging pixel array of 4208H x 3120V 30fps@max resolution. This sensor is configured by I2C interface and has MIPI output interface that is connected to the MIPI CSI-2 RX subsystem inside.

The MIPI CSI-2 receiver subsystem includes a MIPI D-PHY core and that connects 2 data lanes for imx477 camera device and 4 lanes for AR1335 camera device. The subsystem captures video stream frames from the AR1335 camera and IMX477 camera in RAW10 format.

The Demosaic module converts bayer pattern input frame to RGB color frame.

The VCP custom module apply various filters, color space conversion and K-Mean color clusters. It also applies image enhancement control to improve image quality which include contrast, brightness, saturation, white/black balance and rgb gain through axi4 lite configuration registers.

VDMA (Video Direct Memory Access) transfer video streaming to and from external memory and operate under software control.

The DP (display port) in PS source (live input) video data from PL.

VIVADO BLOCK DIAGRAM IP CUSTOMIZATION

ZYNQ ULTRASCALE+ MPSoC	SETTINGS

I/O CONFIGURATION

DPAUX  	MIO 27 TO MIO 30

DisplayPort Lane Selection  	Dual Lower

CLOCK CONFIGURATION

PL Fabric Output Clock PL0	IOP 100 MHz

DisplayPort Reference Frequency	27

PS-PL CONFIGURATION

PS-PL Interface: AXI HPM0 FPD	Select

PS-PL Interface: AXI HPM0 FPD Data Width	128

PS-PL Interface: AXI HPM1 FPD	Select

PS-PL Interface: AXI HPM1 FPD Data Width	128

PS-PL Interface: Slave Interface AXI HPC0 FPD	Select

PS-PL Interface: Slave Interface AXI HPC0 FPD Data Width	128

MIPI CSI2 RX SUBSYSTEM 1	SETTINGS

Pixel Format	RAW10

Serial Data Lanes	2

Line Rate	2500 Mbps

CSI2 Controller Register Interface	Select

Filter User Defines Data Types	Select

Line Buffer Defined Data Types	4096

Allowed VC	All

Pixel Per Clock	1

TUSER Width	1

Enable CRC	Select

MIPI CSI2 RX SUBSYSTEM 2	SETTINGS

Pixel Format	RAW10

Serial Data Lanes	4

Line Rate	2000 Mbps

CSI2 Controller Register Interface	Select

Filter User Defines Data Types	Select

Line Buffer Defined Data Types	4096

Allowed VC	All

Pixel Per Clock	1

TUSER Width	1

Enable CRC	Select

RESOLUTIONS	V_CLK

WIDTH	HEIGHT	MODE	HTOTAL	HBLANK	VTOTAL	VBLANK	HFREQ	VFREQ	CLOCK

640	480	PROGRESSIVE	800	160	525	45.0	31.469	59.940	25.175

720	480	PROGRESSIVE	858	138	525	45.0	31.469	59.940	27.000

720	480	PROGRESSIVE	858	138	525	45.0	31.469	59.940	27.000

1280	720	PROGRESSIVE	1650	370	750	30.0	45.000	60.000	74.250

1920	1080	INTERLACED	2200	280	1125	22.5	33.750	60.000	72.250

1440	480	INTERLACED	1716	276	525	22.5	15.734	59.940	27.000

1440	480	INTERLACED	1716	276	525	22.5	15.734	59.940	27.000

1440	240	PROGRESSIVE	1716	276	262	22.0	15.734	60.054	27.000

1440	240	PROGRESSIVE	1716	276	262	22.0	15.734	59.826	27.000

2880	480	INTERLACED	3432	552	525	22.5	15.734	59.940	54.000

2880	480	INTERLACED	3432	552	525	22.5	15.734	59.940	54.000

2880	240	PROGRESSIVE	3432	552	262	22.0	15.734	60.054	54.000

2880	240	PROGRESSIVE	3432	552	262	22.0	15.734	59.826	54.000

1440	480	PROGRESSIVE	1716	276	525	45.0	31.469	59.940	54.000

1440	480	PROGRESSIVE	1716	276	525	45.0	31.469	59.940	54.000

1920	1080	PROGRESSIVE	2200	280	1125	45.0	67.500	60.000	148.500

720	576	PROGRESSIVE	864	144	625	49.0	31.250	50.000	27.000

720	576	PROGRESSIVE	864	144	625	49.0	31.250	50.000	27.000

1280	720	PROGRESSIVE	1980	700	750	30.0	37.500	50.000	74.250

1920	1080	INTERLACED	2640	720	1125	22.5	28.125	50.000	74.250

1440	576	INTERLACED	1728	288	625	24.5	15.625	50.000	27.000

1440	576	INTERLACED	1728	288	625	24.5	15.625	50.000	27.000

1440	288	PROGRESSIVE	1728	288	312	24.0	15.625	50.080	27.000

1440	288	PROGRESSIVE	1728	288	313	25.0	15.625	49.920	27.000

2880	576	INTERLACED	3456	576	625	24.5	15.625	50.000	54.000

2880	576	INTERLACED	3456	576	625	24.5	15.625	50.000	54.000

2880	288	PROGRESSIVE	3456	576	312	24.0	15.625	50.080	54.000

2880	288	PROGRESSIVE	3456	576	313	25.0	15.625	49.920	54.000

1440	576	PROGRESSIVE	1728	288	625	49.0	31.250	50.000	54.000

1440	576	PROGRESSIVE	1728	288	625	49.0	31.250	50.000	54.000

1920	1080	PROGRESSIVE	2640	720	1125	45.0	56.250	50.000	148.500

1920	1080	PROGRESSIVE	2750	830	1125	45.0	27.000	24.000	74.250

1920	1080	PROGRESSIVE	2640	720	1125	45.0	28.125	25.000	74.250

1920	1080	PROGRESSIVE	2200	280	1125	45.0	33.750	30.000	74.250

2880	480	PROGRESSIVE	3432	552	525	45.0	31.469	59.940	108.500

2880	480	PROGRESSIVE	3432	552	525	45.0	31.469	59.940	108.500

2880	576	PROGRESSIVE	3456	576	625	49.0	31.250	50.000	108.000

2880	576	PROGRESSIVE	3456	576	625	49.0	31.250	50.000	108.000

1920	1080	INTERLACED	2304	384	1250	85.0	31.250	50.000	72.000

1920	1080	INTERLACED	2640	720	1125	22.5	56.250	100.000	148.500

1280	720	PROGRESSIVE	1980	700	750	30.0	75.000	100.000	148.500

720	576	PROGRESSIVE	864	144	625	49.0	62.500	100.000	54.000

720	576	PROGRESSIVE	864	144	625	49.0	62.500	100.000	54.000

1440	576	INTERLACED	1728	288	625	24.5	31.250	100.000	54.000

1440	576	INTERLACED	1728	288	625	24.5	31.250	100.000	54.000

1920	1080	INTERLACED	2200	280	1125	22.5	67.500	120.000	148.500

1280	720	PROGRESSIVE	1650	370	750	30.0	90.000	120.000	148.500

720	480	PROGRESSIVE	858	138	525	45.0	62.937	119.880	54.000

720	480	PROGRESSIVE	858	138	525	45.0	62.937	119.880	54.000

1440	480	INTERLACED	1716	276	525	22.5	31.469	119.880	54.000

1440	480	INTERLACED	1716	276	525	22.5	31.469	119.880	54.000

720	576	PROGRESSIVE	864	144	625	49.0	125.000	200.000	108.000

720	576	PROGRESSIVE	864	144	625	49.0	125.000	200.000	108.000

1440	576	INTERLACED	1728	288	625	24.5	62.500	200.000	108.000

1440	576	INTERLACED	1728	288	625	24.5	62.500	200.000	108.000

720	480	PROGRESSIVE	858	138	525	45.0	125.874	239.760	108.000

720	480	PROGRESSIVE	858	138	525	45.0	125.874	239.760	108.000

1440	480	INTERLACED	1716	276	525	22.5	62.937	239.760	108.000

1440	480	INTERLACED	1716	276	525	22.5	62.937	239.760	108.000

1280	720	PROGRESSIVE	3300	2020	750	30	18	24.0003	59.4

1280	720	PROGRESSIVE	3960	2680	750	30	18.75	25	74.25

1280	720	PROGRESSIVE	3300	2020	750	30	22.5	30.0003	74.25

1920	1080	PROGRESSIVE	2200	280	1125	45	135	120.003	297

1920	1080	PROGRESSIVE	2640	720	1125	45	112.5	100	297

1280	720	PROGRESSIVE	3300	2020	750	30	18	24.0003	40-pin connection provides input/output communication using the pixel clock, pixel data (12 bits), power (3.3V), Serial clock, serial data(I2C), frame valid,

1280	720	PROGRESSIVE	3960	2680	750	30	18.75	25	74.25

1280	720	PROGRESSIVE	3300	2020	750	30	22.5	30.0003	74.25

1280	720	PROGRESSIVE	1980	700	750	30	37.5	50	74.25

1280	720	PROGRESSIVE	1650	370	750	30	45	60.0003	74.25

1280	720	PROGRESSIVE	1980	700	750	30	75	100	148.5

1280	720	PROGRESSIVE	1650	370	750	30	90	120.003	148.5

1920	1080	PROGRESSIVE	2750	830	1125	45	27	24.0003	74.25

1920	1080	PROGRESSIVE	2640	720	1125	45	28.125	25	74.25

1920	1080	PROGRESSIVE	2200	280	1125	45	33.75	30.0003	74.25

1920	1080	PROGRESSIVE	2640	720	1125	45	56.25	50	148.5

1920	1080	PROGRESSIVE	2200	280	1125	45	67.5	60.0003	148.5

1920	1080	PROGRESSIVE	2640	720	1125	45	112.5	100	297

1920	1080	PROGRESSIVE	2200	280	1125	45	135	120.003	297

1680	720	PROGRESSIVE	3300	1620	750	30	18	24.0003	59.4

1680	720	PROGRESSIVE	3168	1488	750	30	18.75	25	59.4

1680	720	PROGRESSIVE	2640	960	750	30	22.5	30.0003	59.4

1680	720	PROGRESSIVE	2200	520	750	30	37.5	50	82.5

1680	720	PROGRESSIVE	2200	520	750	30	45	60.0003	99

1680	720	PROGRESSIVE	2000	320	825	105	82.5	100	165

1680	720	PROGRESSIVE	2000	320	825	105	99	120.003	198

2560	1080	PROGRESSIVE	3750	1190	1100	20	26.4	24.0003	99

2560	1080	PROGRESSIVE	3200	640	1125	45	28.125	25	90

2560	1080	PROGRESSIVE	3520	960	1125	45	33.75	30.0003	118.8

2560	1080	PROGRESSIVE	3300	740	1125	45	56.25	50	185.625

2560	1080	PROGRESSIVE	3000	440	1100	20	66	60.0003	198

2560	1080	PROGRESSIVE	2970	410	1250	170	125	100	371.25

2560	1080	PROGRESSIVE	3300	740	1250	170	150	120.003	495

3840	2160	PROGRESSIVE	5500	1660	2250	90	54	24.0003	297

3840	2160	PROGRESSIVE	5280	1440	2250	90	56.25	25	297

3840	2160	PROGRESSIVE	4400	560	2250	90	67.5	30.0003	297

3840	2160	PROGRESSIVE	5280	1440	2250	90	112.5	50	594

3840	2160	PROGRESSIVE	4400	560	2250	90	135	60.0003	594

4096	2160	PROGRESSIVE	5500	1404	2250	90	54	24.0003	297

4096	2160	PROGRESSIVE	5280	1184	2250	90	56.25	25	297

4096	2160	PROGRESSIVE	4400	304	2250	90	67.5	30.0003	297

4096	2160	PROGRESSIVE	5280	1184	2250	90	112.5	50	594

4096	2160	PROGRESSIVE	4400	304	2250	90	135	60.0003	594

3840	2160	PROGRESSIVE	5500	1660	2250	90	54	24.0003	297

3840	2160	PROGRESSIVE	5280	1440	2250	90	56.25	25	297

3840	2160	PROGRESSIVE	4400	560	2250	90	67.5	30.0003	297

3840	2160	PROGRESSIVE	5280	1440	2250	90	112.5	50	594

3840	2160	PROGRESSIVE	4400	560	2250	90	135	60.0003	594

1280	720	PROGRESSIVE	2500	1220	750	30	36	48.0003	90

1280	720	PROGRESSIVE	2500	1220	750	30	36	48.0003	90

1680	720	PROGRESSIVE	2750	1070	750	30	36	48.0003	99

1920	1080	PROGRESSIVE	2750	830	1125	45	54	48.0003	148.5

1920	1080	PROGRESSIVE	2750	830	1125	45	54	48.0003	148.5

2560	1080	PROGRESSIVE	3750	1190	1100	20	52.8	48.0003	198

3840	2160	PROGRESSIVE	5500	1660	2250	90	108	48.0003	594

4096	2160	PROGRESSIVE	5500	1404	2250	90	108	48.0003	594

3840	2160	PROGRESSIVE	5500	1660	2250	90	108	48.0003	594

3840	2160	PROGRESSIVE	5280	1440	2250	90	225	100	1188

3840	2160	PROGRESSIVE	4400	560	2250	90	270	120.003	1188

3840	2160	PROGRESSIVE	5280	1440	2250	90	225	100	1188

3840	2160	PROGRESSIVE	4400	560	2250	90	270	120.003	1188

5120	2160	PROGRESSIVE	7500	2380	2200	40	52.8	24.0003	396

5120	2160	PROGRESSIVE	7200	2080	2200	40	55	25	396

5120	2160	PROGRESSIVE	6000	880	2200	40	66	30.0003	396

5120	2160	PROGRESSIVE	6250	1130	2475	315	118.8	48.0003	742.5

5120	2160	PROGRESSIVE	6600	1480	2250	90	112.5	50	742.5

5120	2160	PROGRESSIVE	5500	380	2250	90	135	60.0003	742.5

5120	2160	PROGRESSIVE	6600	1480	2250	90	225	100	1485

5120	2160	PROGRESSIVE	5500	380	2250	90	270	120.003	1485

7680	4320	PROGRESSIVE	11000	3320	4500	180	108	24.0003	1188

7680	4320	PROGRESSIVE	10800	3120	4400	80	110	25	1188

7680	4320	PROGRESSIVE	9000	1320	4400	80	132	30.0003	1188

7680	4320	PROGRESSIVE	11000	3320	4500	180	216	48.0003	2376

7680	4320	PROGRESSIVE	10800	3120	4400	80	220	50	2376

7680	4320	PROGRESSIVE	9000	1320	4400	80	264	60.0003	2376

7680	4320	PROGRESSIVE	10560	2880	4500	180	450	100	4752

7680	4320	PROGRESSIVE	8800	1120	4500	180	540	120.003	4752

7680	4320	PROGRESSIVE	11000	3320	4500	180	108	24.0003	1188

7680	4320	PROGRESSIVE	10800	3120	4400	80	110	25	1188

7680	4320	PROGRESSIVE	9000	1320	4400	80	132	30.0003	1188

7680	4320	PROGRESSIVE	11000	3320	4500	180	216	48.0003	2376

7680	4320	PROGRESSIVE	10800	3120	4400	80	220	50	2376

7680	4320	PROGRESSIVE	9000	1320	4400	80	264	60.0003	2376

7680	4320	PROGRESSIVE	10560	2880	4500	180	450	100	4752

7680	4320	PROGRESSIVE	8800	1120	4500	180	540	120.003	4752

10240	4320	PROGRESSIVE	12500	2260	4950	630	118.8	24.0003	1485

10240	4320	PROGRESSIVE	13500	3260	4400	80	110	25	1485

10240	4320	PROGRESSIVE	11000	760	4500	180	135	30.0003	1485

10240	4320	PROGRESSIVE	12500	2260	4950	630	237.6	48.0003	2970

10240	4320	PROGRESSIVE	13500	3260	4400	80	220	50	2970

10240	4320	PROGRESSIVE	11000	760	4500	180	270	60.0003	2970

10240	4320	PROGRESSIVE	13200	2960	4500	180	450	100	5940

10240	4320	PROGRESSIVE	11000	760	4500	180	540	120.003	5940

4096	2160	PROGRESSIVE	5280	1184	2250	90	225	100	1188

4096	2160	PROGRESSIVE	4400	304	2250	90	270	120.003	1188

ARCHITECTURE

The Video Color Processing module provides a modular expandable interface for video pipeline.

General architecture consists of camera interface module “camera_raw_to_rgb” which convert bayer format data into rgb format” rgb_set”. Video stream module which filters rgb data into various filters and axis external module which stream the filtered data to axi4-stream.

Figure 2

The idea behind using the D5M camera for the FPGA was to have an easy connection of 40 pins between these two devices. This 40 pins connection provide input/output communication using the pixel clock, pixel data (12 bits), power (3.3V), Serial clock, serial data(I2C), frame valid and line valid. A typical camera system is shown in figure 1. At the heart of every digital camera is an image sensor either CCD image senor or a CMOS sensor. Both types of sensors capture the light through the lens and convert into the electrical signal which is further processed by ADC. Nowadays, majority of sensors consist of high-performance ADC converters which are employed to produce the digital output. Most CMOS image sensors have ADC resolutions of 10 to 12 bits. In this project, D5M camera has 12 bits ADC resolution. The D5M pixel array consists of a 2752-coloumn by 2004-row matrix. The pixel array is addressed by column and row. The array consists of a 2592-column by 1944-row active region and a border region as shown below. The output images are divided into frames, which are further divided into lines. The output signals frame valid and lines valid are used to indicate the boundaries between the lines and frames [3]. Pixel clock is used as a clock to latch the data. For each pixel clock cycle, one 12-bit pixel data outputs on the data out pins [3]. When both frame valid and line valid are asserted, the pixel is valid. Pixel clock cycles that occur when frame valid is negated are called vertical blanking. Pixel clock cycles that occur when only line valid is negated are called horizontal blanking. The camera has an array of 255 register, a lot of them are configurable, and it is possible to set up the operation of the camera by writing to these registers. This communication is performed using a generic two wire serial interface commonly referred to as I2C[3]. There are two different communication modes, control / configuration mode which included read/write values to the register in the D5M card, and pixel data read out which consists of reading the pixel data from the camera card [3].

BANDWIDTH AND DATA RATE CALCULATION

The design is divided into three clock domains. The frequency of the output clock from ZYNQ UltraScale+ MPSoC is set to 100MHz, and it than fed to the clock generator that create all other clocks in the PL design. A 100Mhz input clock is fed to generator block which than generate three clock outputs 300MHz and 297MHz and 200MHz. Pixel clock is nominally the pixel clock rate set according to resolution and refresh rate.

Following are the clocks used in this design:

Clock 300MHz for video Axi4 video configuration and process.

Clock 200MHz for MIPI DPHY video input.

Clock 297MHz for video output.

PIXEL CLOCK FREQUENCY

Pixel Clock Frequency = Total Horizontal Samples * Total Vertical Lines * Refresh Rate

VIDEO CLOCK

The following equation is used to calculate the minimum required Video Clock:

video_aclk (MHz)=(Line Rate(Mb/s)*Data Lanes)/(Pixels per Clock* Number of Bits Per Pixel)

TOTAL DATA RATE

The following equation is used to calculate the minimum required Total Data Rate:

Total Data Rate (Bandwidth)= Pixel Clock Frequency * Pixel Size (in bits)

BANDWIDTH

The bandwidth of a given video format is a product of the Pixel Clock Frequency and Pixel Size in bits.

Bandwidth = Pixel Clock Frequency * Pixel Size

LINE RATE

The Line Rate is the total data rate (bandwidth) divided by the number of lanes.

Line Rate = Total Data Rate (Bandwidth)/Number of Data Lane.

1920x1080p@60Hz, RAW10, 2-lane

Total Horizontal Samples = 2200,

Total Vertical Lines = 1125

Refresh Rate = 60

Number of Data Lanes = 2

Pixel per clock = 1

Video Clock = 2200 x 1125 x 60 = 148.5 MHz

Bandwidth (Total Data Rate) = 148.5 MHz * 10-bit = 1485 Mbps

Line Rate (Data Rate per Lane) = 1485 Mbps /2-lane = 742.5 Mbps

Video Clock = 742.5 Mbps * 2 / 1*10 = 148.5 MHz

2560x1080p@30Hz, RAW10, 2-lane

Total Horizontal Samples = 3520

Total Vertical Lines = 1125

Pixel Clock Frequency = 3520 x 1125 x 30 = 118.8 MHz

Bandwidth (Total Data Rate) = 118.8 MHz * 10-bit = 1188 Mbps

Line Rate (Data Rate per Lane) = 1188 Mbps /2-lane = 594.0 Mbps

2560x1080p@60Hz, RAW10, 2-lane

Total Horizontal Samples = 3520

Total Vertical Lines = 1125

Pixel Clock Frequency = 3520 x 1125 x 60 = 237.6 MHz

Bandwidth (Total Data Rate) = 237.6MHz * 10-bit = 2376 Mbps

Line Rate (Data Rate per Lane) = 2376 Mbps /2-lane = 1188.0 Mbps

3840x2160@30Hz, RAW8, 4-lane

Total Horizontal Samples = 4400,

Total Vertical Lines = 2250

Pixel Clock Frequency = 4400 x 2250 * 30 = 297 MHz

Bandwidth (Total Data Rate) = 297 MHz * 8-bit = 2376 Mbps

Line Rate (Data Rate per Lane) = 2376 Mbps /4-lane = 594 Mbps

In above calculation, resolution “p” signifies “progressive scan” which display both odd and even lines and where as “i” denotes the use of interlaced scanning, with quality being lower which displays odd and even scan lines as individual fields.

CMOS PIXEL CAPTURE

Pixel capture module is the first module inside the FPGA which communicate with camera. It receives the data of 12 bits per pixel at each clock cycle from the cmos camera when the frame valid and line valid are asserted high. Pixel clock (50MHz) is used to latch the data on the rising edge of the clock. In the case of when clear is set, then it captures on the falling edge of the pixel clock. Vertical blank occurs when frame valid is high. However, horizontal blank only occurs when both frames line valid and frame valid are high. To pause, restart, snapshot and change the exposure level of the video, certain registers need to be assigned the hex values. These hex values are given in the datasheet of this camera. Setting these values will enable the features and functionality of the camera. These features were enabled by using the I2C bus which has two wires SDA and SCL. SDA is the data line. SCL is the clock line which is used to synchronize all data transfers over the I2C bus. The SCL & SDA lines are connected to the FPGA and the camera on the I2C bus. Once the registers were assigned the values and valid signal are asserted, the camera output the 12 bits data at the maximum data rate of 96Mp/s. Input clock for the camera is 96MHz which gives the maximum data rate, but this data rate should be latched at the 50MHz clock. Active pixels: 2,592H x 1,944V Pixel size: 2.2μm x 2.2μm Color filter array RGB Bayer pattern Full resolution Programmable up to 15 fps Frame rate VGA (640 x 480) Programmable up to 70 fps ADC resolution: 12-bit Pixel dynamic range: 70.1dB SNRMAX: 38.1dB Supply Power Voltage: 3.3V I/O Voltage: 1.7V～3.1V

VCP

Prior to the overview of the various filters, it necessary to overview top module and io connections to internal modules. Generic defined in VCP give user option to select various filter, config axi4-lite address/data bus width and set revision number. The TDATA_WIDTH generic specifies the number of bits in a word and the ADDR_WIDTH specifies the number of address bits, which implies that there is 2^ADDR_WITH words. BMP_WIDTH AND BMP_HEIGHT specifies image frame width and height.

Figure 3

To configure video stream register, axi4-lite protocol is implemented which present a memory map register interface to the processor.

HSL COLOR SPACE

HSV, HSL and HSI: Hue is a color range degree from 0 degree to 360 degree which describe a pure color. Saturate is shade of gray to full color. Amount of saturation of color is known as chroma. Higher value of chroma is clear and bight. The strongest magnitude is value and its range correspond to brightness and balanced magnitude corresponds to the intensity. Hue color describe the pure color whereas saturate values determine pure color range strength and luminosity describe pure color range. A color with 100% saturation yields purest color and 0% yields grayscale. A color with 0% luminosity is black and 100% luminosity yields bright color.

Value of angles below are hue values on the color wheel.

0 degree     – Red

60 degree   – Yellow

120 degree – Green

180 degree – Cyan

240 degree – Blue

300 degree – Magenta

360 degree – Red-Magenta

From RGB triplet saturation equation shown below where max is calculated of between red, green, and blue channel. It represents the strength of the color and the radius of the cone.

saturate=((max⁡(rgb)-min⁡(rgb)))/(max⁡(rgb))

Figure 4

RGB TO HSV

The implemented rgb to hsv color space in video color processing module uses hsv algorithm for FPGA devices, and it has been designed with a standard Xilinx AXI4 streaming interface, so that it can be inserted as module ip within any image processing pipeline.

This module converts rgb color space to hsl color space. First logic calculates maximum and minimum value of rgb values. Hue is calculated by determining the hue fraction from greatest rgb channel value. If current max channel is red than Hue numerator will be set to be green subtract blue only if green is greater than blue else blue is subtracted from green and Hue degree would be zero.  If current max channel is green than Hue numerator will be set to be blue subtract red only if blue is greater than red else red is subtracted from blue and Hue degree would be 129. Similarly, if current channel is blue than Hue numerator will be set to be red subtract green only if red is greater than green else green subtracted from red and Hue degree would be 212. Hue denominator would be rgb delta. Once Hue fraction values are calculated than fraction values would be added to hue degree which would give final hue value as done logic. Saturate value is calculated from difference between rgb max and min over rgb max whereas Lightness value rgb max value.

Figure 5

Rgb to hsl module convert input rgb to hsl color space. The module has clock and reset ports. Port iRGB consist of red, green, and blue rgb channels with valid signal, which is used to convert to rgb pixels to hsl pixels.

Ports	Description

clk	Reference clock for input and output data stream.

reset	Specifies a module with an asynchronous active low reset.

iRgb.red	8-bit input data. Red value.

iRgb.green	8-bit input data. Green value.

iRgb.blue	8-bit input data. Blue value.

iRgb.valid	Input data is valid. Indicates whether the next data point is available for processing.

oHsl.red	8-bit output data. Hue value.

oHsl.green	8-bit output data. Saturate value.

oHsl.blue	8-bit output data. Luminosity value.

oHsl.valid	Output data is valid. Control signal to indicate the validity of each pixel.

The Functional block diagram of the implemented RGB to HSL color space conversion is shown in Figure 9-12.

The maximum and minimum RGB values are calculated according to the logic implementation shown in the figure above.

Figure 6: Hue Numerator Logic

Figure 7: Hue Denominator Logic

Figure 8: Hue Degree Logic

Figure 9 : HSL Filter Wave Diagram

The simulation results of RGB channel conversion to the HSL color space are presented in the wave diagram in Figure 8.

Figure 10

Figure 11

The figures below show the RGB color space conversion implementation into six regions of the hexagon images. The representation of the RGB color space is shown in the 1st and 3rd figures.

Figure 12: RGB image

Figure 13: HSL converted image

Figure 14: RGB image

Figure 15: HSL converted image

1st and 3rd figures are RGB images, whereas 2nd and 4th are HSL simulated results.

Figure 16: HSL Image

Figure 17: Hue channel

Figure 18: Saturate channel

Figure 19: Luminosity channel

Figure 20: HSL Image

Figure 21: Hue channel

Figure 22: Saturate channel

Figure 23: Luminosity channel

1st image is hsl image, 2nd represent hue channel, 3rd saturate channel and 4th luminosity channel.

1ST figure shows an RGB image. 2nd figure shows the red channel. The 3rd figure shows the green channel, and the 4th figure shows the blue channel.

Figure 24

Figure 25

Figure 26

Figure 27

Figure 28

Figure 29

1st image is hsl image, 2nd represent hue channel, 3rd saturate channel and 4th luminosity channel.

HSL is a cylindrical geometry with hue, saturation, and lightness, where colors transition from red to orange, yellow, green, cyan, blue, and magenta.

HSV TO RGB

The implemented HSV RGB color space in the video color processing module uses the HSV to RGB conversion algorithm for FPGA devices, and it has been designed with a standard Xilinx AXI4 streaming interface, so that it can be inserted as a module IP within any image processing pipeline.

Figure 30

The HSV to RGB module converts input HSV values to the RGB color space. The module has clock and reset ports. Port iRGB consists of red, green, and blue RGB channels with a valid signal, which is used to convert to RGB pixels to HSL pixels.

Ports	Description

clk	Reference clock for input and output data stream.

reset	Specifies a module asynchronous active low reset.

iRgb.red	8-bit input data. Red value.

iRgb.green	8-bit input data. Green value.

iRgb.blue	8-bit input data. Blue value.

iRgb.valid	Input data is valid. Indicates whether the next data point is available for processing.

oHsl.red	8-bit output data. Hue value.

oHsl.green	8-bit output data. Saturate value.

oHsl.blue	8-bit output data. Luminosity value.

oHsl.valid	Output data is valid. Control signal to indicate the validity of each pixel.

Figure 31

HSV COLOR SPACE

This module converts the RGB color space to the HSL color space. First, logic calculates the maximum and minimum values of RGB values. Hue is calculated first by determining the hue fraction from the most excellent RGB channel value. If the current max channel is red, then the Hue numerator will be set to be green, subtracting blue only if green is greater than blue; else, blue is subtracted from green, and the Hue degree will be zero.  If current max channel is green than Hue numerator will be set to be blue subtract red only if blue is greater than red else red is subtracted from blue and Hue degree would be 129. Similarly, if current channel is blue than Hue numerator will be set to be red subtract green only if red is greater than green else green subtracted from red and Hue degree would be 212. Hue denominator would be rgb delta. Once Hue fraction values are calculated than fraction values would be added to hue degree which would give final hue value as done logic. Saturate value is calculated from difference between rgb max and min over rgb max whereas Intensity value rgb max value.

Figure 32

Figure 33

Figure 34

Figure 35

Figure 36

Figure 37

Figure 38

Figure 39

1st and 2nd figure are rgb image and whereas 3rd and 4th are hsl simulated results.

Figure 40

Figure 41

Figure 42

Figure 43

1st and 2nd figure are rgb image and whereas 3rd and 4th are hsl simulated results.

Figure 44

Figure 45

Figure 46

Figure 47

1st and 2nd figure are rgb image and whereas 3rd and 4th are hsl simulated results.

Figure 48

Figure 49

Figure 50

Figure 51

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSL COLOR RANGE SPACE

Figure 52

Figure 53

Figure 54

Figure 55

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

COLORHSL

Figure 56

Figure 57

Figure 58

Figure 59

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSL_1RANGE

Figure 60

Figure 61

Figure 62

Figure 63

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSL_2RANGE

Figure 64

Figure 65

Figure 66

Figure 67

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSL_3RANGE

Figure 68

Figure 69

Figure 70

Figure 71

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSVL_1RANGE

Figure 72

Figure 73

Figure 74

Figure 75

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSVL_2RANGE

Figure 76

Figure 77

Figure 78

Figure 79

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSVL_3RANGE

Figure 80

Figure 81

Figure 82

Figure 83

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

HSVL_4RANGE

Figure 84

Figure 85

Figure 86

Figure 87

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

CMYK COLOR SPACE

The implemented cmyk to rgb color space in video color processing module uses cmyk to rgb conversion algorithm for FPGA devices, and it has been designed with a standard Xilinx AXI4 streaming interface, so that it can be inserted as module ip within any image processing pipeline.

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam cmyk color space in output oRgb channel.

CMYK (Cyan, Magenta, Yellow, Key/Black) is the color space for printed materials.

Figure 88

Figure 89

Figure 90

Figure 91

Figure 92

Figure 93

From these values, the value of colors can be calculated from the following equations:

K=1-MAX(R,G,B)

C =(1-R-K)(1-K)

M=(1-G-K)(1-K)

Y =(1-B-K)(1-K)

CMYK color space has four dimensions, and it is used for printing color images. The color printer makes use of subtractive(secondary) color scheme. The minimum requirement for color printer is equipped with three primary colors: cyan(C), magenta (M), and yellow (Y). The secondary colors Yellow, Cyan and Magenta are generated by adding primary colors Red, Green and Blue. Above figures shows, a 500 x 500 color image in RGB and CYM formats. The pinkish color in RGB transformed into greenish, as the red and blue component intensities are high compared to green. The black color is generated by combining all the colors whereas white is set with no colors.

YDBDR COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam YDRDB color space in output oRgb channel.

Figure 94 RGB Image

Figure 95 YDBDR Image

Figure 96 RGB Image

Figure 97 YDBDR Image

In this color space, Figure 95 image represent by three components, namely, Y, DB, and DR.

The R, G, B tristimulus values converted to Y, DB, DR tristimulus values as follows:

[■(Y@D_R@D_B )]=[■(0.299&0.587&0.114@-0.450&-0.883&1.333@-1.333&-1.160&0.217)][■(R@G@B)]

Above figures illustrates the result of convert color image space in RGB domain to image with YDRDB components.

CIEXYZ COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam CIE-XYZ color space in output oRgb channel.

Figure 98

Figure 99

Figure 100

Figure 101

Above figures illustrates the result of convert color image space in RGB domain to image with XYZ components.

The R, G, B tristimulus values converted to X, Y, Z tristimulus values. The relationship between RGB and the XYZ values expressed by a 3x3 matrix as shown in equation below.

[■(X@Y@Z)]=[■(0.412&0.357&0.180@0.212&0.715&0.072@0.019&0.119&0.950)][■(R@G@B)]

The coefficients values in 3x3 matrix are multiplied to input RGB components and the sum of the result values as per XYZ channel.

CIEYUV COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam CIE-YUV color space in output oRgb channel.

Figure 102

Figure 103

Figure 104

Figure 105

The R, G, B tristimulus values converted to Y, U, V tristimulus values as follows:

[■(Y@U@V)]=[■(0.299&0.587&0.114@-0.147&-0.289&0.436@0.615&-0.515&-0.100)][■(R@G@B)]

YIQ COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam YIQ color space in output oRgb channel.

Figure 106

Figure 107

Figure 108

Figure 109

In this color space, Figure 107 image represent by three components, namely, Y, I, and Q. The Y-channel (this corresponds roughly with intensity) is the luminance component, and I(In-Phase)/Q(Quadrature) represent chrominance (color information: chroma, jointly describe the hue and saturation). This color space separate intensity information from color details.

The R, G, B tristimulus values converted to Y, I, Q tristimulus values as follows:

[■(Y@I@Q)]=[■(0.299&0.587&0.114@0.595&-0.274&-0.321@0.211&-0.522&-0.311)][■(R@G@B)]

Figure 106 shows RGB converted to YIQ color space Figure 106 using the transformation of above equation. Note the elements of 1st row in the conversion matrix sum to 1 and 2nd and 3rd rows sum to zero. IQ components are transmitted with slower amplitude variations with limited image details whereas Y component contain more gray level image details of information.

YPBPR COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam YPBPR color space in output oRgb channel.

Figure 110

Figure 111

Figure 112

Figure 113

In this color space, Figure 111 image represent by three components, namely, Y, PB, and PR. The Y-channel corresponds to luminance component, PB channel corresponds to difference between blue and luma and whereas PR channel corresponds to difference between red and luma.

The R, G, B tristimulus values converted to Y, PR, PB tristimulus values as follows:

[■(Y@P_R@P_B )]=[■(0.213&0.715&0.072@-0.115&-0.385&0.500@0.500&-0.454&-0.046)][■(R@G@B)]

I1I2I3 OHTA COLOR INTENSITIES

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam I1I2I3 color space in output oRgb channel.

Figure 114

Figure 115

Figure 116

Figure 117

Figure 118

〖RED〗_INTENSITY=I_1=(RED+GREEN+BLUE)/3

〖GREEN〗_INTENSITY=I_2=(RED-BLUE)/2

〖BLUE〗_INTENSITY=I_3=(2xGREEN-RED-BLUE)/4

LMS COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam LMS color space in output oRgb channel.

Figure 119

Figure 120

Figure 121

Figure 122

Figure 123

The R, G, B tristimulus values converted to L, M, S tristimulus values as follows:

[■(L@M@S)]=[■(0.400&0.707&-0.080@-0.228&1.150&0.061@0.000&0.000&0.918)][■(R@G@B)]

ICTCP COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam ICTCP color space in output oRgb channel.

Figure 124

Figure 125

Figure 126

Figure 127

Figure 128

The R, G, B tristimulus values converted to I, CT, CP tristimulus values as follows:

[■(I@C_T@C_P )]=[■(0.400&0.400&0.200@4.455&-4.851&3.960@8.056&3.572&-1.162)][■(R@G@B)]

HED COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam HED color space in output oRgb channel.

Figure 129

Figure 130

Figure 131

Figure 132

Figure 133

The R, G, B tristimulus values converted to H, E, D tristimulus values as follows:

[■(H@E@D)]=[■(1.800&-0.070&-0.600@-1.020&-1.130&-0.480@-0.550&-0.130&1.570)][■(R@G@B)]

YC1C2 COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam YC1C2 color space in output oRgb channel.

Figure 134

Figure 135

Figure 136

Figure 137

Figure 138

The R, G, B tristimulus values converted to Y, C1, C2 tristimulus values as follows:

[■(Y@P_R@P_B )]=[■(0.213&0.715&0.072@-0.115&-0.385&0.500@0.500&-0.454&-0.046)][■(R@G@B)]

SHARP FILTER

Figure 139

The sharp filter has been implemented and tested for an image size of 1024 × 1024 as shown figure below.

BLUR FILTER

Figure 140

Gray Level Colors

0.400	0.350	0.200	0.950

0.200	0.700	0.100	1.00

0.05	0.100	0.900	1.05

0.605	1.15	1.20	∑

EMBOSS FILTER

An emboss filter will take video frame and convert into an embossed image.

SOBEL FILTER

For Sobel edge detection, monochrome RGB pixel are used of 8 bits which are stored in row-by-row memory of successive pixels. Two Sobel filters are used for edge detection. There are two distinct filters, one which computes vertical edge left and one for horizontal edge right. The filters are convolved with the entire input 3-line buffer which holds the pixel information. The vertical edge component is calculated with vertical filter Ky into input the first tap1 of 8 bits and then tap2 and tap3 sum to 3 Ky taps (3 elements of 1st rows of filter). The Same calculation is applied to horizontal filter 3 columns Kx gives the output of sum 3 Kx taps(3 elements of 1st column of filter). The resulted sum 3 Ky taps gives us the final sum of Vertical Kvy1,2,3 and 3 sum for horizontal Kvx1,2,3. Final output of vertical and horizontal is given by Rxy which is the square root of sum Kvy and Kvx. √ . The Figure below illustrates the concept of Sobel Filter for Video Edge detection.

Three lines video buffer have used for a filter with 3x3 kernel for both vertical and horizontal edge detection. Then the video buffer enters the filter in a parallel form. Each 3 t-taps module takes the data taps from the line buffer with a rate faster than system clock rate by 3 times and multiply them by the 3 filter coefficients. The output from each multiplications process is accumulated in the accumulator. Then finally sum all three 3-Tap filter from the accumulator fed into parallel adder which form one processed pixel. Once the Rxy is calculated the value is compared with threshold which is the user input from the switches (12 to 1). If the pixel output from Rxy is greater than the threshold, the pixel is detected as the edge otherwise no output to the RGB channel.

A Sobel filter will take video frame and convert into an Sobel image.

Figure 141

YCBCR COLOR SPACE

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs iRgb.red, iRgb.red, iRgb.red, iRgb.valid, iRgb.eol, iRgb.eof and iRgb.sof. The result of this module steam CIE-XYZ color space in output oRgb channel.

Figure 142

Cb refers to the blue-difference chroma component, and Cr refers to the red-difference chroma component.

Figure 143

COLOR ADJUST MATRIX

Figure 144

The objective of the color adjust matrix is to transform each pixel red, green and blue channel into improve suitable agreement in display and light sources sensitivity. The color adjust matrix can eb expressed as a 3x3 matrix as in equation below. The ‘CAM’ is used to denote the color adjust matrix rgb values.

[■(R_CAM@G_CAM@B_CAM )]=[■(K1&K2&K3@K4&K5&K6@K7&K8&K9)][■(R@G@B)]

The color transformations defined in this section are for each color in pixel changed into unique color as multicolor colors range changed into single color.

COLOR MANIPULATION INDEX # 01

Color Adjust Matrix

CAM-1	5.000	-3. 313	-1. 687	0.00

-1. 687	5.000	-3. 313	0.00

-3. 313	-1. 687	5.000	0.00

0.00	0.00	0.00

CAM-2	1.200	-0.435	-0.165	0.600

-0.165	1.200	-0.435	0.600

-0.435	-0.165	1.200	0.600

0.600	0.600	0.600

CAM-3	0.900	0.300	0.350	1.650

0.350	0.900	0.300	1.650

0.300	0.350	0.900	1.650

1.650	1.650	1.650

Color Manipulation Index # 02

Color Adjust Matrix

CAM-1	1.340	-0.850	-0.400	0.09

-0.400	1.340	-0.850	0.09

-0.850	-0.400	1.340	0.09

0.09	0.09	0.09

CAM-2	2.000	-0.200	-0.100	1.70

-0.100	2.000	-0.200	1.70

-0.200	-0.100	2.000	1.70

1.70	1.70	1.70

CAM-3	1.5	0.900	0.650	3.05

0.650	1.5	0.900	3.05

0.900	0.650	1.5	3.05

3.05	3.05	3.05

Color Manipulation Index # 03

Color Adjust Matrix

CAM-1	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-2	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-3	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

Color Manipulation Index # 04

Color Adjust Matrix

CAM-1	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-2	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-3	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

Color Manipulation Index # 05

Color Adjust Matrix

CAM-1	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-2	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-3	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

Color Manipulation Index # 06

Color Adjust Matrix

CAM-1	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-2	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-3	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

Color Manipulation Index # 07

Color Adjust Matrix

CAM-1	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-2	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

CAM-3	1.5	-0.250	-0.250	1.00

-0.250	1.5	-0.250	1.00

-0.250	-0.250	1.5	1.00

1.00	1.00	1.00

Color Manipulation Index # 08

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 09

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 10

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 11

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 12

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 13

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 14

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 15

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

Color Manipulation Index # 16

Color Adjust Matrix

CAM-1	1.250	-0.850	-0.400	0.00

-0.400	1.250	-0.850	0.00

-0.850	-0.400	1.250	0.00

0.00	0.00	0.00

CAM-2	1.00	-0.600	-0.400	0.00

-0.400	1.00	-0.600	0.00

-0.600	-0.400	1.00	0.00

0.00	0.00	0.00

CAM-3	1.5	1.200	0.500	3.20

0.500	1.5	1.200	3.20

1.200	0.500	1.5	3.20

3.20	3.20	3.20

AN FPGA IMPLEMENTATION OF K-MEANS CLUSTERING

This section of the document describes computing the k-mean rgb clustering of rgb video stream.

The implemented image segment uses K-Mean clustering algorithm for FPGA Devices, and it has been designed with a standard Xilinx AXI4 streaming interface, so that it can be inserted as module ip within any image processing pipeline.

Figure 145

This module takes a stream of camera data in pixel pipeline format. This stream must be presented to the inputs data (iRgb.red, iRgb.green, iRgb.blue) and control signals (iRgb.valid, iRgb.eol, iRgb.eof, iRgb.sof). The result of this module steam K-Mean rgb cluster color space in output oRgb channel.

Figure 146

Figure 147

Implementation and Verification

The module was synthesized and implemented using Vivado 2022.1 specifically for the KRIA KV260 board. Following the hardware design and synthesis process, verification was conducted using the ModelSim 2020 edition simulator to ensure the correctness and reliability of the implementation.

The Functional block diagram of the implemented rgb data path to n clustered conversion is shown in Figure below.

In this module, K mean-based color quantization algorithm is applied to rgb input stream pixels.

Select rgb k color references for k points to decide number of clusters.

Find the distance of each input pixels with selected rgb k points.

Assign each new pixel value to closest selected rgb n values using Euclidean distance.

Pixels which are nearest to the selected rgb values allocated to a cluster.

Euclidean distance is calculated between original image pixel Red1, Green1 and Blue1 and reference pixel color schemes.

D=√(〖〖(RED〗_2 〖-RED〗_1)〗^2+〖〖(GREEN〗_2 〖-GREEN〗_1)〗^2+〖〖(BLUE〗_2 〖-BLUE〗_1)〗^2 )

Rgb Image of video frame consist of 0 to 255 values per rgb channel which gives 256*256*256 colors, and the goal is to use color k mean cluster to set number of clusters. Minimum distance is the final candidate for being closest to the source rgb color. If an RGB image color depth of 24 bits which is 16 million of colors, after K-mean clustering with value n, then image is converted to a version of n colors.

The codebook created for K-Means is called the color palette or reference color scheme.

K-mean cluster module convert 16 million of rgb colors into n color version. The module has clock and reset ports. Port iRGB and oRGB consist of red, green, and blue rgb channels with valid signal.

Ports	Description

clk	Reference clock for input and output data stream.

reset	Specifies module asynchronous active low reset.

iRgb.red	8-bit input data. Red value.

iRgb.green	8-bit input data. Green value.

iRgb.blue	8-bit input data. Blue value.

iRgb.valid	Input data valid. Specifies whether the next data point has arrived for processing.

oRgb.red	8-bit output data. N clustered red value.

oRgb.green	8-bit output data. N clustered green value.

oRgb.blue	8-bit output data. N clustered blue value.

oRgb.valid	Output data valid. Control signal to indicate the validity of each pixel.

K EQUALTO 6 REFERENCE COLOR SCHEMES

Generated image below is the result of k-mean clustering using 6 colors references of palette schemes.

#	Red	Green	Blue

1	230	170	120

2	70	40	35

3	150	200	130

4	20	25	10

5	75	150	180

6	15	30	60

K EQUALTO 9 REFERENCE COLOR SCHEMES

In this reference, k parameter set to k = 9, where k is the number of clusters. K-mean color quantization quantizes input image to number of colors into 9 clusters from 9 refences of color schemes.

K EQUALTO 24 REFERENCE COLOR SCHEMES

In this reference, k parameter set to k = 24, where k is the number of clusters. K-mean color quantization quantizes input image to number of colors into 24 clusters from 24 refences of color schemes.

K EQUALTO 51 REFERENCE COLOR SCHEMES

In this reference, k parameter set to k = 51, where k is the number of clusters. K-mean color quantization quantizes input image to number of colors into 51 clusters from 51 refences of color schemes.

Image below is generated using 51 reference color schemes.

#	RED	GREEN	BLUE

1	255	250	240

2	230	200	150

3	220	190	140

4	240	180	120

5	230	170	120

6	220	160	150

7	210	160	130

8	200	160	100

9	180	120	80

10	160	100	70

#	RED	GREEN	BLUE

11	150	80	60

12	140	80	60

13	120	80	55

14	100	80	50

15	80	60	40

16	70	40	35

17	60	40	30

18	50	30	20

19	40	20	15

20	25	15	10

#	RED	GREEN	BLUE

21	255	255	200

22	200	255	230

23	200	224	190

24	190	224	200

25	150	200	130

26	130	200	150

27	120	150	100

28	100	150	120

29	80	100	40

30	40	100	80

#	RED	GREEN	BLUE

31	50	75	30

32	30	75	50

33	40	50	20

34	20	50	40

35	10	25	20

36	20	25	10

37	15	10	0

38	0	10	5

39	220	240	55

40	150	220	40

#	RED	GREEN	BLUE

41	100	200	20

42	90	180	0

43	75	150	80

44	50	100	60

45	50	80	40

46	50	75	20

47	25	50	0

48	20	40	80

49	15	30	60

50	5	10	20

51	0	5	10

IMX477 CAMERA: K EQUALTO 90 REFERENCE COLOR SCHEMES

In this reference, k parameter set to k = 90, where k is the number of clusters. K-mean color quantization quantizes input image to number of colors into 90 clusters from 90 refences of color schemes.

Figure 29. (Left) Original image; (Right) results obtained using the K-Mean Clustering.

90 REFERENCE PROGRAMABLE COLOR SCHEMES

VIDEO CONTROLLER INTERFACE

LOCAL DYNAMIC THRESHOLD SEGMENTATION

This module takes the pixel as the center and check its neighboring pixels and compared with one by one. If the difference level is within the threshold limits than average, it and the new pixel value will be compared with each neighborhood pixels. The produced new region is uniform and difference in region would be small.

The aim is to average pixel by thresholding. Typically, in the design for this module, threshold of value 10 is selected with the pixel values in a 0-255 range. Pixel with values less than the threshold will map to average, whilst those with values greater than or equal to the threshold will map to original pixel values. Average pixel values are close to current and next values are averaged within the threshold; this mapping gives local clustering of image pixel.

Not implemented Yet: For given selected region of area below the define threshold such that there would exist a maximum value before averaging. Such those max value does not need to be part of average values. This would better result for averaging the pixels within the threshold. If all values are same than there is no max, in other words all value are constant in that region. Such implementation would be valid for at-least 3 pixels or greater.

The module rgb pixel data is available on when valid data is asserted high by the module.

IMAGE CONTRAST AND BRIGHTNESS

CAMERA RAW DATA

This module read 12-bit RGB data from Demosaic Xilinx IP. External Pixel clock is from camera used to sample 1 pixel which equal to 1 pll generated master external clock. Host camera data is than stored into buffer line by line which is ready to be fetch at system clock rate. Input line and frame valid signals are used to start reading stored buffer data. Valid read is enabled when both frame and line valid signal are asserted high. Buffer size set to be the size of frame width. The read controller side reads whole frame width from the buffer. Values written to buffer run at pixel clock rate whereas buffer read side run at faster rate than pixel clock.

Buffersize is auto size supported which is controlled by input line valid from host camera and maximum is set to default value of 3071. A base configuration consisting of a single 24-bits pipeline, capable of processing 1080p HD video at 30 frames per second.

Input data from camera is color filtered which is arranged in a bayer patten. Input data is read with line-by-line transfer rate of pixel clock which later synchronize into system clock.

Figure 148 : camera raw data module

Figure 149 : General view of camera raw data flow

Host camera interface uses 12 bits parallel input data with line and frame valid control signals.

THREE TAPS DATA

Figure 150

FOUR TAPS DATA

Figure 151

PIXEL COORDINATES

Figure 152

Figure 153

Figure 154

CAMERA RAW DATA

HISTOGRAM

Figure 155

This module assigns memory location as rgb red channel input integer between 0 to 255 which equally 8 bits to express 256 levels address. Every input value would accumulate to its location to show how many hits per level.

Figure 156

Histogram Equation

1ST figure shows rgb image. 2nd figure shows red channel. 3rd figure shows green channel and 4th figure shows blue channel.

Figure 157

ETHERNET UDP VIDEO STREAMING

To successfully run the real-time network UDP (User Datagram Protocol) video streaming, this reference design uses the KRIA KV260 development board developed by Xilinx.

The design is composed of two platforms: the transmitter platform and the receiver platform. The receiver platform which receives input video stream from imx477 camera implements PL logic design of device. The transmitter platform which is PS Side of device implements a UDP/IP hardware protocol stack that enables high speed communication over a LAN or a point-to-point connection.

UDP/IP video streaming is implemented by using LWIP (LightWeight Internet Protocol) which is open-source networking stacked design for embedded system. The main purpose usage of lwip in this design is portability, as well as small code and data size, making it especially suited for network video transmission.

LWIP supports both TCP and UDP at transport layer and application-level protocols. To get better performance, UDP protocol is used as a transport layer protocol. UDP is a connectionless protocol and faster than TCP since there is no error checking for packets. UDP protocol is used as transport layer in this design since it is suitable for video transmission that require fast transportation.

CONFIGURATION

KRIA KV260 board connect directly to the host PC and not running a DHCP server and host PC need to configure to an IP address of 192.168.0.42 and subnet mask of 255.255.255.0. Default IP address for FPGA development board is configured to “192.168.0.10”.

NETWORK

At startup LWIP fetch an IP address from DHCP server. If there no DHCP server than DHCP server request will timeout and the board will default to preset default to an IP address of 192.168.0.10.

The FFMPEG application on the host PC will connect with the video transmitter application on board and video data transmission will commence. When the video images are received, the FFMPEG invoke ffplay.exe which decode first 54 bytes source video type. Below figure shows 54 bytes of video header format.

The BSP required to run the lwip application and it includes many settings that impact ethernet performance. Below figure shows lwip BSP settings.

VIDEO STREAM DATA AND FORMAT

The GUI design on the remote computer provide the following features:

Send the commands to the remote FPGA development board IP address “192.168.0.10”.

Receive the image data from the development board.

FFMPEG “ffplay.exe” decode the received bmp images to video stream at specified frame rate.

SONY IMX477/IMX682/IMX519/IMX219 AND ONSEMI AR1335 CAMERAS VIDEO STREAMING

Sony Cameras are used on Raspberry Pi interface the Kria KV260 Development Board.

LWIP ETHERNET INTERFACE

The KRIA KV260 development board has 1 Gigabit Ethernet which is connected through the RGMII interface.

5MP OV5640 SENSOR INTERFACE

The image sensor in the PCAM 5C Module is a CMOS type digital sensor model OV5640 which is 5 Megapixel (MP) color image sensor.

Image pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to KRIA KV260 development via 15-pin flat flexible cable.

5MP OV5647 SENSOR INTERFACE

The image sensor in the PCAM 5C Module is a CMOS type digital sensor model OV5640 which is 5 Megapixel (MP) color image sensor.

Image pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to KRIA KV260 development via 15-pin flat flexible cable.

8MP IMX219 SENSOR INTERFACE

The image sensor in the SONY IMX219 Module is a CMOS type digital sensor which is 8 Megapixel (MP) color image sensor. It’s support 1080p30,720p60 and 640x480p90 video resolutions and still image of max resolution of 3280x2464 pixels.

Image pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to KRIA KV260 development via 15-pin flat flexible cable.

SPECIFICATIONS

Sensor	Sony IMX219

Optical Format	Type 1/4(Diagonal 4.60mm)

Pixel	8MP

Pixel Size	1.12μm×1.12μm

Active Pixels	3280 (H) × 2464 (V)

CSI-2 Data Output	2-lane mode

Data Format	Raw Bayer 10bit

Frame Rates	1080p30, 720p60 and VGA90 video modes

Shutter Type	Rolling Shutter

FOV (Field of View)	75°(H)

Camera Board Size	24mm x 25mm

SONY 12.5MP IMX477 SENSOR

IMX477-MIPI-CS is a high-resolution digital camera. It incorporates a Sony 1/2.3" CMOS digital image sensor with an active imaging pixel array of 4056H x3040V.

SPECIFICATIONS

Sensor	Sony IMX477

Optical Format	Type 1/4(Diagonal 4.60mm)

Pixel	12.5MP

Pixel Size	1.55μm×1.55μm

Active Pixels	4056 (H) × 3040 (V) approx. 12.33 M pixels

CSI-2 Data Output	2/4-lane mode

Data Format	Raw Bayer 10/12 bits

Frame Rates	4K2Kp60, 1080p240 and VGA90 video modes

Shutter Type	Rolling Shutter

Camera Board Size	24mm x 25mm

DRIVE MODE

Drive Mode	Number of active pixels	Maximum frame rate [frames/s]	Output Interface	ADC [bit]

Full (4:3) [Normal]	4056(H) × 3040(V) 12.3MP	60	CSI-2	10

Full (4:3) [Normal]	4056(H) × 3040(V) 12.3MP	40	CSI-2	12

Full (4:3) [DOL-HDR]	4056(H) × 3040(V) 12.3MP	30/15 (DOL 2/DOL 3)	CSI-2	10

Full (16:9) 4K2K [Normal]	4056(H) × 2288(V) 9.3MP	79	CSI-2	10

Full (16:9) 4K2K [DOL-HDR]	4056(H) × 2288(V) 9.3MP	39/19 (DOL 2/DOL 3)	CSI-2	10

Full (4:3) Binning [Normal]	2028(H) × 1520(V) 3.1MP	179	CSI-2	10

Full (16:9) Binning 1080P [Normal]	2028(H) × 1128(V) 2.3MP	240	CSI-2	10

Full (16:9) Binning 720P [Normal]	1348(H) × 750(V) 1MP	240	CSI-2	10

Full (16:9) Scaling 1080P [Normal]	2024(H) × 1142(V) 2.3MP	79	CSI-2	10

Full (16:9) Scaling 720P [Normal]	1348(H) × 762(V) 1MP	79	CSI-2	10

FEATURES

Back-illuminated and stacked CMOS image sensor Exmor RS

Digital Overlap High Dynamic Range (DOL-HDR) mode with raw data output.

High signal to noise ratio (SNR).

Full resolution @60 frame/s (Normal), 4K2K @60 frame/s (Normal), 1080p @240 frame/s

Full resolution @40 frame/s (12 bit Normal), Full resolution @30 frame/s (DOL-HDR, 2 frame)

Output video format of RAW12/10/8, COMP8.

Power Save Mode

Pixel binning readout and V sub-sampling function.

Independent flipping and mirroring.

Input clock frequency 6 to 27 MHz

CSI-2 serial data output (MIPI 2lane/4lane, Max. 2.1 Gbps/lane, D-PHY spec. ver. 1.2 compliant)

2-wire serial communication.

Two PLLs for independent clock generation for pixel control and data output interface.

Ambient Light Sensor (ALS)

Fast mode transition.

Dual sensor synchronization operation (Multi camera compatible)

7 K bit of OTP ROM for users.

Built-in temperature sensor

10-bit/12-bit A/D conversion on chip

92-pin high-precision ceramic package

INTERFACE

IMX477 Camera module and the KRIA KV260 development board are connected through the FPC flexible flat cable. Camera pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to development board via 15-pin flat flexible cable. Raspberry Pi camera module interface only support 2 MIPI lanes which directly connected directly to the Zynq UltraScale+ MPSoC HPA bank.

The interface is defined as follows:

PIN	SIGNAL NAME	DESCRIPTION

PIN 1	Ground	Ground

PIN 2	MIPI_LAN0_N	Camera CMOS LANE0 Data Negative Output

PIN 3	MIPI_LAN0_P	Camera CMOS LANE0 Data Positive Output

PIN 4	Ground	Ground

PIN 5	MIPI_LAN1_N	Camera CMOS LANE1 Data Negative Output

PIN 6	MIPI_LAN1_P	Camera CMOS LANE1 Data Positive Output

PIN 7	Ground	Ground

PIN 8	MIPI_CLK_N	Camera CMOS Clock Negative Output

PIN 9	MIPI_CLK_P	Camera CMOS Clock Data Positive Output

PIN 10	Ground	Ground

PIN 11	CAM_PWUP	Camera CMOS Power-on Control Signal

PIN 12	CAM_CLK	Camera CMOS Input Clock Signal

PIN 13	CAM_SCL	Camera CMOS I2C Clock Signal

PIN 14	CAM_SDA	Camera CMOS I2C Data Signal

PIN 15	+3.3V	Power Supply 3.3V

The Camera Serial Interface (CSI), a division from the MIPI Alliance which have formulated set of interface standards to standardize the interface of mobile devices such as camera and display. MIPI's full name is "Mobile Industry Processor Interface", MIPI DSI correspond to video display and MIPI CSI correspond to video input standards respectively. CSI interface is divided into physical layer (D-PHY) and protocol layer (CSI-2).

CONFIGURATION

For this design application, the IMX477 camera device is configured to output 1920x1080p video frames with RGB 30 bits pixel format, at 60 frames rate. Both the camera device and MIPI CSI-2 subsystem module is configured in advance.

VIDEO INPUT SUBSYSTEMS

The MIPI CSI-2 subsystem module is used to captures images from IMX477 camera sensor and outputs to axi4 stream video stream. Demosaic module is used to convert raw axi4 stream video data to rgb format video stream.

VDMA IP is used convert Demosaic video stream data to AXI4 memory map format and DDR fetching decoding and execution via AXI_HP interface.

VDMA (Video Direct Memory Access) is a general DMA, specially used for video and image applications. It has an AXI4 Memory Map interface for reading (writing channel) and writing (read channel) video to memory.

The display port controller in the PS is coupled to the STDP4320 demultiplexer on the carrier which consists of dual mode output ports configured as DP/HDMI.

IMX477 CONFIGURATION REGISTERS

ADDRESS	REGISTER NAME	VALUE

0x0342	LINE_LEN_MSB	0X31

0x0343	LINE_LEN_LSB	0XC4

0x0344	X_ADD_STA_MSB	0X00

0x0345	X_ADD_STA_LSB	0X00

0x0348	X_ADD_END_MSB	0X0F

0x0349	X_ADD_END_LSB	0XD7

0x0346	Y_ADD_STA_MSB	0X01

0x0347	Y_ADD_STA_LSB	0XB8

0x034A	Y_ADD_END_MSB	0X0A

0x034B	Y_ADD_END_LSB	0X27

0x040C	DIG_CROP_WIDTH_MSB	0X0F

0x040D	DIG_CROP_WIDTH_LSB	0XD7

0x040E	DIG_CROP_HEIGHT_MSB	0X04

0x040F	DIG_CROP_HEIGHT_LSB	0XB0

0x034C	X_OUT_SIZE_MSB	0X08

0x034D	X_OUT_SIZE_LSB	0X00

0x034E	Y_OUT_SIZE_MSB	0X04

0x034F	Y_OUT_SIZE_LSB	0XB0

0x0408	DIG_CROP_X_OFFSET_MSB	0X00

0x0409	DIG_CROP_X_OFFSET_LSB	0X00

0x040A	DIG_CROP_Y_OFFSET_MSB	0X00

0x040B	DIG_CROP_Y_OFFSET_LSB	0X00

0x0381	X_ENV_INC_CONST	0X01

0x0383	X_ODD_INC_CONST	0X01

0x0385	Y_ENV_INC_CONST	0X01

0x0387	Y_ODD_INC	0X01

0x0900	BINNING_MODE	0X01

0x0901	BINNING_HV	0X12

0x0902	BINNING_WEIGHTING	0X02

0x3F0D	ADC_BIT_SETTING	0X0A

0x0401	SCALE_MODE	0X01

0x0404	SCALE_M_MSB	0X00

0x0405	SCALE_M_LSB	0X20

0x0301	IVTPXCK_DIV	0X05

0x0303	IVTSYCK_DIV	0X02

0x0305	IVT_PREPLLCK_DIV	0X04

0x0306	PLL_IVT_MPY_MSB	0X01

0x0307	PLL_IVT_MPY_LSB	0X5E

0x0309	IOPPXCK_DIV	0X0C

0x030B	IOPSYCK_DIV	0X02

0x030D	IOP_PREPLLCK_DIV	0X02

0x030E	IOP_MPY_MSB	0X00

0x030F	IOP_MPY_LSB	0X96

0x0310	PLL_MULTI_DRV	0X01

0x0820	REQ_LINK_BIT_RATE_MSB	0X07

0x0821	REQ_LINK_BIT_RATE_LMSB	0X08

0x0822	REQ_LINK_BIT_RATE_MLSB	0X00

0x0823	REQ_LINK_BIT_RATE_LSB	0X00

0x080A	TCLK_POST_EX_MSB	0X00

0x080B	TCLK_POST_EX_LSB	0X7F

0x080C	THS_PRE_EX_MSB	0X00

0x080D	THS_PRE_EX_LSB	0X4F

0x080E	THS_ZERO_MIN_MSB	0X00

0x080F	THS_ZERO_MIN_LSB	0X77

0x0810	THS_TRAIL_EX_MSB	0X00

0x0811	THS_TRAIL_EX_LSB	0X5F

0x0812	TCLK_TRAIL_MIN_MSB	0X00

0x0813	TCLK_TRAIL_MIN_LSB	0X57

0x0814	TCLK_PREP_EX_MSB	0X00

0x0815	TCLK_PREP_EX_LSB	0X4F

0x0816	TCLK_ZERO_EX_MSB	0X01

0x0817	TCLK_ZERO_EX_LSB	0X27

0x0818	TLPX_EX_MSB	0X00

0x0819	TLPX_EX_LSB	0X3F

0x3E37	PDAF_CTRL1_0	0X00

0x3F50	POWER_SAVE_ENABLE	0X00

0x3F56	LINE_LEN_INCLK_MSB	0X01

0x3F57	LINE_LEN_INCLK_LSB	0X6C

0x0204	ANA_GLOBAL_GAIN_U	0X03

0x0205	ANA_GLOBAL_GAIN_L	0X7F

0x020E	DIG_GAIN_GR_U	0X02

0x020F	DIG_GAIN_GR_L	0X00

0x0210	DIG_GAIN_R_U	0X03

0x0211	DIG_GAIN_R_L	0X00

0x0212	DIG_GAIN_B_U	0X02

0x0213	DIG_GAIN_B_L	0X00

0x0214	DIG_GAIN_GB_U	0X02

0x0215	DIG_GAIN_GB_L	0X00

0x0202	COARSE_INTEGRATION_TIME_MSB	0X02

0x0203	COARSE_INTEGRATION_TIME_LSB	0X00

0x0100	MODE_SEL	0X01

16MP IMX519 SENSOR INTERFACE

The image sensor in the PCAM 5C Module is a CMOS type digital sensor model OV5640 which is 5 Megapixel (MP) color image sensor.

Image pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to KRIA KV260 development via 15-pin flat flexible cable.

64MP IMX682 SENSOR INTERFACE

The image sensor in the PCAM 5C Module is a CMOS type digital sensor model OV5640 which is 5 Megapixel (MP) color image sensor.

Image pixels data is transferred over dual -lane MIPI CSI-2 interface which is connected to KRIA KV260 development via 15-pin flat flexible cable.

VITIS PROGRAM DEVELOPMENT

The program design is mainly divided into five parts, video initialization, configuration, control, and transmission. Below figure present the overall software structure of implemented application, which shows the program and data flow.

In the main function, the initialization of the 1 frame buffered is performed for VDMA1. The VIDEO1_MAX_FRAME macro definition is set to 1, the initial initialization of pFrame0 pointer, pFrame0 points to the cache of camera 1.

TESTBENCH

For filter modules verification vhdl testbench is being used to generate image. Approach used to generate stimulus and checking response by using single process testbench. Testbench include a process which read image file to apply rgb stimulus to design filter section and then wait until end of frame to generate valid output image bmp file.

TESTBENCH COMPONENTS/OBJECTS

D5M verification testbench is created through configuration, the factory and the phase build process which allows stimulus and randomization generation. The testbench describes image filters of design, where source of input stimuli, or test patterns are applied and monitor of output results inform of output generated images. Testbench requires the user to manually check filtered output image and check pass/fail test pattern testing.

UVM testbench build from classes. Test has environment, the environment has d5m agent.

Flow of testbench is to connect DUT and testbench, configure d5m agent, generate stimulus.

In the filter’s generic package, a common verification scenario for vfp tests is to initialize set of parameters by setting to define enable variable in frame enable package. This type of initialization can be done at terminal during compilation unit scope. In table shows available define variable with available size for image testing.

`define	size	description

rgb	0=64x64,

1=128x125,

2=255x255,

3=1920x1080

sharp

blur

embross

hsl

hsv

cgain

sobel

Set of parameters for image filter type and their size are assigned for current test values as shown in table 2 via defined macro in frame enable package according to table 1.

parameter	value	description

F_CGA	1/0	Enable ‘1’ Cgain color adjust for test.

F_SHP	1/0	Enable ‘1’ Sharp Filter for test.

F_BLU	1/0	Enable ‘1’ Blure Filter for test.

F_HSL	1/0	Enable ‘1’ HSL color space for test.

F_HSV	1/0	Enable ‘1’ HSV color space for test.

F_RGB	1/0	Enable ‘1’ RGB color space for test.

F_SOB	1/0	Enable ‘1’ Sobel filter for test.

F_EMB	1/0	Enable ‘1’ Embross Filter for test.

For axi4-lite module registers in rtl, following parameters offset for base address are set for axi4 lite transections as shown table below.

Register Name	Offet	Description

initAddr            	8'h00

oRgbOsharp          	8'h00

oEdgeType           	8'h04

filter_id           	8'h08

aBusSelect          	8'h0C

threshold           	8'h10

videoChannel        	8'h14

dChannel            	8'h18

cChannel            	8'h1C

kls_k1              	8'h20

kls_k2              	8'h24

kls_k3              	8'h28

kls_k4              	8'h2C

kls_k5              	8'h30

kls_k6              	8'h34

kls_k7              	8'h38

kls_k8              	8'h3C

kls_k9              	8'h40

kls_config          	8'h44

als_k1              	8'h54

als_k2              	8'h58

als_k3              	8'h5C

als_k4              	8'h60

als_k5              	8'h64

als_k6              	8'h68

als_k7              	8'h6C

als_k8              	8'h70

als_k9              	8'h74

als_config          	8'h78

pReg_pointInterest  	8'h7C

pReg_deltaConfig    	8'h80

pReg_cpuAckGoAgain  	8'h84

pReg_cpuWgridLock   	8'h88

pReg_cpuAckoffFrame 	8'h8C

pReg_fifoReadAddress	8'h90

pReg_clearFifoData  	8'h94

rgbCoord_rl         	8'hC8

rgbCoord_rh         	8'hCC

rgbCoord_gl         	8'hD0

rgbCoord_gh         	8'hD4

rgbCoord_bl         	8'hD8

rgbCoord_bh         	8'hDC

oLumTh              	8'hE0

oHsvPerCh           	8'hE4

oYccPerCh           	8'hE8

Figure 158

Figure 159

DUT CONNECTIONS

Input protocol interface

Output protocol interface

SEQUENCE ITEM

SEQUENCE

SEQUENCER

DRIVER

This class which extended from uvm driver pull data items generated by a sequencer and drive it to the DUT. In run phase, methods are used for reading and writing operation to dut through dut interface handle.

Axi4lite communication flow across 5 parallel channels. Read transection phases split into 2 phases. Request on read address channel and response on read data channel. Write transection split into 3 phases: address, data, and response with the status. Multiple read and write transaction are concurrent. Write address transfer can come before, during, or after the write data. Signal for read address channels start with AR* and read data signals start with R*. Write address signal start with AW*, write data signal name start with W* and write response start with B*. Axi4lite has flow control on each channel to control the rate the information is exchange such as address, data, and response. The source generates valid to indicate information is available. Destination generates ready signal to accept the information. The source assert valid first and wait for destination to be ready and then transfer the info. Or destination could go first, asserting ready to receive and wait for the source. When source is ready, it asserts valid and transfer the info. The transfer only occurs when both valid and ready are high.

Basic overview:

Declare the virtual interface.

Get the interface handle using get config_db.

Add the get config_db in the build_phase.

Add driving logic. get the seq_item and drive to dut signals.

Figure 160

DECLARATION:

This uvm driver class is derived from uvm component.

class d5m_camera_driver extends uvm_driver #(d5m_trans);

DATA MEMEBERS

protected virtual d5m_camera_if d5m_camera_vif;

protected int     id;

NEW CONSTRUCT

For Each component, the constructor must execute and complete to bring the component into existence. Therefore, new () must run before build() or any other subsequent phase can execute.

function new (string name, uvm_component parent);

super.new(name, parent);

endfunction: new

BUILD_PHASE

Build method run top-down and the rest of the phases run bottom-up. In this phase, config the dut interface handle through get method. The uvm_config_db parameterized class provides a convenience interface on top of uvm_resource_db is used to for reading resource database.  The uvm_config_db is derived from uvm_resource_db class. Get value of field_name “d5m_camera_vif”, using component cntxt ”this” as starting search point.

function void build_phase (uvm_phase phase);

super.build_phase(phase);

if (!uvm_config_db#(virtual d5m_camera_if)::get

(this, "", "d5m_camera_vif", d5m_camera_vif))

`uvm_fatal("NOVIF", {"virtual interface must be set for:

",get_full_name(), ".d5m_camera_vif"});

endfunction: build_phase

RUN_PHASE

In this method, fork join constructs are used to separate threads that drive each of the channels.

virtual task run_phase (uvm_phase phase);

fork

reset signals();

d5m_frame();

join

endtask: run_phase

RESET SIGNALS

Reset the dut axi4 lite and d5m_cam_mod input signals when system reset is asserted from low to high.

virtual protected task reset_signals();

forever begin

@(posedge d5m_camera_vif.ARESETN);

d5m_camera_vif.axi4.AWADDR        <=  8'h0;

d5m_camera_vif.axi4.AWPROT        <=  3'h0;

d5m_camera_vif.axi4.AWVALID       <=  1'b0;

d5m_camera_vif.axi4.WDATA         <= 32'h0;

d5m_camera_vif.axi4.WSTRB         <=  4'h0;

d5m_camera_vif.axi4.WVALID        <=  1'b0;

d5m_camera_vif.axi4.BREADY        <=  1'b0;

d5m_camera_vif.axi4.ARADDR        <=  8'h0;

d5m_camera_vif.axi4.ARPROT        <=  3'h0;

d5m_camera_vif.axi4.ARVALID       <=  1'b0;

d5m_camera_vif.axi4.RREADY        <=  1'b0;

d5m_camera_vif.d5p.iImageTypeTest <=  1'b0;

d5m_camera_vif.d5p.iReadyToRead   <=  1'b0;

d5m_camera_vif.d5p.fvalid         <=  1'b0;

d5m_camera_vif.d5p.lvalid         <=  1'b0;

end

endtask: reset_signals

D5M FRAME

In this method, drive the signals from defined seq in uvm_sequence.

virtual protected task d5m_frame();

forever begin

@(posedge d5m_camera_vif.clkmm);

seq_item_port.get_next_item(req);

drive_transfer(req);

seq_item_port.item_done();

end

endtask: d5m_frame

DRIVE TRANSFER

This method which is master to dut axi4lite interface write/read data at given address using bus handshaking protocol. First valid address is transmitted and then wait for valid response in given time of 61 clock cycles in axi4_address method. Timeout accord on 62 clock cycle, if no response is asserted high on bvalid signal from dut and timeout is flagged using uvm_error macro. If valid response is asserted then axi4_data method write/read data depending on case statement. Once axi4 bus config the video process module in dut then write/read operation can be initiated by calling the d5m_pixel method.

virtual protected task drive_transfer (d5m_trans d5m_tx);

axi4_address(d5m_tx);

axi4_data(d5m_tx);

d5m_pixel(d5m_tx);

endtask: drive_transfer

AXI4 ADDRESS CHANNEL

In this method, write/read axi4 address channel.

virtual protected task axi4_address (d5m_trans d5m_tx);

case (d5m_tx.d5m_txn)

AXI4_WRITE : axi4_write_address(d5m_tx);

AXI4_READ  : axi4_wread_address(d5m_tx)

endcase

endtask: axi4_address

AXI4 DATA CHANNEL

In this method, write/read axi4 data channel through axi4_write data and axi4_read_data methods on selected case.

virtual protected task axi4_data (d5m_trans d5m_tx);

bit[31:0] rw_data;

bit err;

rw_data = d5m_tx.axi4_lite.data;

case (d5m_tx.d5m_txn)

AXI4_WRITE : axi4_write_data(d5m_tx);

AXI4_READ  : axi4_read_data(rw_data, err);

endcase

endtask: axi4_data

D5M PIXEL

In this method, d5m read/write frame rgb pixel per transaction.

virtual protected task d5m_pixel (d5m_trans d5m_tx);

case (d5m_tx.d5m_txn)

D5M_WRITE  : d5m_write_pixel_data(d5m_tx);

IMAGE_READ : d5m_read_pixel_data(d5m_tx);

endcase

endtask: d5m_pixel

D5M WRITE PIXEL DATA

In this method, write data to d5m camera mod from d5m_trans sequence.

virtual protected task d5m_write_pixel_data (d5m_trans d5m_tx);

d5m_camera_vif.d5p.iReadyToRead    <= 1'b0;

d5m_camera_vif.d5p.iImageTypeTest <= d5m_tx.d5p.iImageTypeTest;

d5m_camera_vif.d5p.rgb                       <= d5m_tx.d5p.rgb;

d5m_camera_vif.d5p.fvalid                   <= d5m_tx.d5p.fvalid;

d5m_camera_vif.d5p.lvalid                   <= d5m_tx.d5p.lvalid;

endtask: d5m_write_pixel_data

D5M READ PIXEL DATA

In this method, config test type during read operation and wait for end of frame pulse.

virtual protected task d5m_read_pixel_data(d5m_trans d5m_tx);

@(posedge d5m_camera_vif.clkmm);

d5m_camera_vif.d5p.iImageTypeTest  <= 1'b0;

d5m_camera_vif.d5p.iReadyToRead    <= 1'b1;

forever begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.d5m.eof) break;

end

endtask: d5m_read_pixel_data

AXI4 WRITE ADDRESS

In this method, write address and assert write valid high and then wait for response from dut BVALID signal within 62 clock cycles.

virtual protected task axi4_write_address (d5m_trans d5m_tx);

int axi_lite_ctr;

d5m_camera_vif.axi4.AWADDR  <= {8'h0, d5m_tx.axi4_lite.addr};

d5m_camera_vif.axi4.AWPROT  <= 3'h0;

d5m_camera_vif.axi4.AWVALID <= 1'b1;

// wait for write response

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.BVALID) break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver","AWVALID timeout");

end

endtask: axi4_write_address

AXI4 WRITE DATA

virtual protected task axi4_write_data (d5m_trans d5m_tx);

int axi_lite_ctr;

d5m_camera_vif.axi4.WDATA  <= d5m_tx.axi4_lite.data;

d5m_camera_vif.axi4.WSTRB  <= 4'hf;

d5m_camera_vif.axi4.WVALID <= 1'b1;

@(posedge d5m_camera_vif.clkmm);

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.WREADY)

d5m_camera_vif.axi4.AWADDR  <= 8'h0;

d5m_camera_vif.axi4.AWPROT  <= 3'h0;

d5m_camera_vif.axi4.AWVALID <= 1'b0;

break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver","AWVALID timeout");

end

@(posedge d5m_camera_vif.clkmm);

d5m_camera_vif.axi4.WDATA  <= 32'h0;

d5m_camera_vif.axi4.WSTRB  <= 4'h0;

d5m_camera_vif.axi4.WVALID <= 1'b0;

// wait for write response

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.BVALID) break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver","BVALID timeout");

end

else begin

if (d5m_camera_vif.axi4.BVALID == 1'b1 && d5m_camera_vif.axi4.BRESP != 2'h0)

`uvm_error("axi_lite_master_driver","Received ERROR Write Response");

d5m_camera_vif.axi4.BREADY <= d5m_camera_vif.axi4.BVALID;

@(posedge d5m_camera_vif.clkmm);

end

endtask: axi4_write_data

AXI4 WREAD ADDRESS

virtual protected task axi4_wread_address (d5m_trans d5m_tx);

int axi_lite_ctr;

d5m_camera_vif.axi4.ARADDR  <= {8'h0, d5m_tx.axi4_lite.addr};

d5m_camera_vif.axi4.ARPROT  <= 3'h0;

d5m_camera_vif.axi4.ARVALID <= 1'b1;

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.ARREADY) break;

end

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver","ARVALID timeout");

end

@(posedge d5m_camera_vif.clkmm);

d5m_camera_vif.axi4.ARADDR  <= 8'h0;

d5m_camera_vif.axi4.ARPROT  <= 3'h0;

d5m_camera_vif.axi4.ARVALID <= 1'b0;

endtask: axi4_wread_address

AXI4 READ DATA

In this method, axi4lite read data.

virtual protected task axi4_read_data (output bit [31:0] data, output bit error);

int axi_lite_ctr;

for(axi_lite_ctr = 0; axi_lite_ctr <= 62; axi_lite_ctr ++) begin

@(posedge d5m_camera_vif.clkmm);

if (d5m_camera_vif.axi4.RVALID) break;

end

data = d5m_camera_vif.axi4.RDATA;

if (axi_lite_ctr == 62) begin

`uvm_error("axi_lite_master_driver","RVALID timeout");

end

else begin

if (d5m_camera_vif.axi4.RVALID == 1'b1 && d5m_camera_vif.axi4.RRESP != 2'h0)

`uvm_error("axi_lite_master_driver","Received ERROR Read Response");

d5m_camera_vif.axi4.RREADY <= d5m_camera_vif.axi4.RVALID;

@(posedge d5m_camera_vif.clkmm);

end

endtask: axi4_read_data

MONITOR

The monitor is a system is critical component in the verification environment. It obtains and collect events and data related activity in the DUT. The information collected by monitor from dut, or stimulus is for checkers, scoreboard, and coverage.

This class which extended from uvm_monitor.

Basic overview:

•	Declare the virtual interface.

•	Get the interface handle using get config_db.

DECLARATION:

This uvm_monitor class is derived from uvm component.

class d5m_mon_dut extends uvm_monitor;

DATA MEMEBERS

protected virtual d5m_camera_if d5m_camera_vif;

protected int     id;

uvm_analysis_port #(d5m_trans) mon_d5m_dut;

NEW CONSTRUCT

For Each component, the constructor must execute and complete to bring the component into existence. Therefore, new () must run before build() or any other subsequent phase can execute.

function new (string name, uvm_component parent);

super.new(name, parent);

endfunction: new

BUILD_PHASE

In this phase, config the dut interface handle through get method. The uvm_config_db parameterized class provides a convenience interface on top of uvm_resource_db is used to for reading resource database.  The uvm_config_db is derived from uvm_resource_db class. Get value of field_name “d5m_camera_vif”, using component cntxt ”this” as starting search point.

function void build_phase (uvm_phase phase);

super.build_phase(phase);

if(!uvm_config_db#(virtual d5m_camera_if)::get(this, "", "d5m_camera_vif", d5m_camera_vif))

`uvm_fatal("NOVIF",{"virtual interface must be set for: ",get_full_name(), ".d5m_camera_vif"});

mon_d5m_dut = new("mon_d5m_dut", this);

endfunction: build_phase

RUN_PHASE

In this method, call collection transection method.

COLLECT_TRANSACTIONS

virtual protected task collect_transactions();

d5m_trans rx_fdut;

rx_fdut            = d5m_trans::type_id::create("rx_fdut");

forever begin

@(posedge d5m_camera_vif.clkmm)

rx_fdut.d5m.valid   = d5m_camera_vif.d5m.valid;

rx_fdut.d5m.red      = d5m_camera_vif.d5m.red;

rx_fdut.d5m.green  = d5m_camera_vif.d5m.green;

rx_fdut.d5m.blue    = d5m_camera_vif.d5m.blue;

rx_fdut.d5m.rgb      = d5m_camera_vif.d5m.rgb;

rx_fdut.d5m.lvalid  = d5m_camera_vif.d5m.lvalid;

rx_fdut.d5m.fvalid  = d5m_camera_vif.d5m.fvalid;

rx_fdut.d5m.x         = d5m_camera_vif.d5m.x;

rx_fdut.d5m.y         = d5m_camera_vif.d5m.y;

rx_fdut.d5m.eof      = d5m_camera_vif.d5m.eof;

mon_d5m_dut.write(rx_fdut);

end

endtask: collect_transactions

d5m_monitor extended from uvm monitor. Rx monitor watches transection from dut.  It receives transection from dut through virtual interface. During build phase, the monitor gets the virtual interface from the configuration database. Tlm anaysis port is declared, first is mon_d5m_dut for sending input items in d5m_monitor_dut monitor and then d5m_mon_prd port for predicted values in d5m_monitor_predict monitor. Both ports are specialized are transection type. TLM connections are components and must be constructed in build phase. The TLM classes are never extended, just call new directly.

AGENT

D5M agent class is derived from uvm_agent which bundles together a sequencer, driver, monitor and coverage as a reusable verification component. In connect phase, agent connect analysis ports to the monitor’s port. Connection to the driver and sequencer is conditional connect to driver’s port and sequencer export of the agent which is configured active.

SCOREBOARD

The term scoreboard whose function is to answer does it work. The main purpose of scoreboard is to collect data about the operation of dut and compares it with expected values. D5M camera scoreboard class is derived from uvm component. It receives expected results transections and from a predictor and actual dut output transections from a monitor. It observes transactions to input of dut and computes the expected effects of those transaction and stores a representation in suitable format for later checking when corresponding transaction asserted from dut output.

ENVIRONMENT/ENV

D5M camera environment creates and configures agent to stimulate the dut and create scoreboard for the agent. The environment first registers the class in the factory. Declare handles to components. Connect the agent to the scoreboard.

TEST

Test class initiate and execute sequence on the specified sequencer in the run phase. First, simulator command line specifies the name of the test +UVM_TESTNAME=test to run. Then UVM factory creates a component of the test and starts its phase methods through run_test task which is called from the static part of test bench which is in initial block of the top-level test bench module.

When run_test method is called, it first creates the object of top test and then call all phases. It constructs the root component of the uvm environment in the top test which than trigger and initiate the uvm phasing component. Basically, calling run_test task causes the selected test to be constructed which first build uvm environment from top to downward and responsible for getting a reference to the uvm_root class instance from UVM core services. UVM infrastructure build phase start from selected test and continue to flow for next phasing until all uvm phases are completed which include connect and run sub phases. UVM calls $finish once all the phases are completed and return the control to top test bench module initial block.  If no test and environment is created than fatal message will issued. However, test can be run in a uvm environment by specifying the test name as argument to run_test or call test name in command line argument. Once build, connect and end_of_elaboration phases are completed start_of_simulation phase gets initiated before time consuming run phase. This phase display banners and test bench topology and configuration information.

After start_of_simulation phase run phase gets initiated which is used for the stimulus generation and checking activities of the test bench and execution of all uvm_component in parallel. Each uvm_component run phase task run in parallel where main phase gets initiated, and stimulus of specified test case is generated and applied to the dut.

Most commonly in the user test, the uvm_phase object is used to raise and drop objection to void moving on the next phase, raise_objection() and drop_objection () are the methods to that. Both methods are used in the user test in the run phase and in between raise and drop objection sequence get started. When drop objection condition is met, action taken is to move to next phase of non-time-consuming cleanup phase and finally test ends.

Raise and drop objection mechanism allow hierarchical status communication among components and once all raised objections are dropped for run phase than phase end.

There are various types of image filters and color space implemented in rtl which need to be verified. Therefore, for each filter and color space tests has been created to verify rtl code. Each test is capable to configure various image dimension size and vfp configuration registers.

D5M TRANSECTION

This class which consists of d5m data items which are implemented as struct objects.

DECLARATION

This class is inherited from "uvm_sequence_item".

class d5m_trans extends uvm_sequence_item;

DATA MEMEBERS

rand    rgb_channel         vfp;

rand    rgb_channel         d5m;

rand    cof_channel         cof;

rand    axi4_lite_channel   axi4_lite;

rand    pattern_channel     d5p;

vfp_axi4                    axi4;

rand d5m_txn_e              d5m_txn;

In data member section d5m data items are listed. Data items vfp and d5m are rgb_channel type which consist of clock, valid, line valid, frame valid end of frame, start of frame, rgb channels x/y coordinates data members.

TESTBENCH_TOP

References: